3GPP TSG-RAN WG2 #128					      		 R2-2410570

Orlando, USA – November 18 - 22, 2024

Agenda item:	8.13.2

Source: 	Kyocera

Title: 	Discovery and (re)selection under multihop relay

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues to address for relay discovery and (re)selection under U2N multihop relay.

# Discussion

During RAN2#127, the following agreements were reached on discovery and relay (re)selection:

Agreements:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms.

RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

From RAN2 perspective, models A and B can both be supported.

Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying.

Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying.

Reuse existing resource pools defined for discovery message transmission and reception

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18.

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

FFS discovery conditions for the intermediate relay UEs.

And based on the outcome of the post email discussion [1], several issues remain unclear that should be further discussed and addressed below.

## 2.1.	Discovery

Based on SA2’s TR [2], for Model A discovery the U2N relay UE transmits a discovery message containing the User Info ID of itself, maximum number of hops and the associated RSC. Subsequently, the intermediate relay UEs are expected to propagate the discovery message from the U2N relay UE while updating the multi-hop counter until the maximum number of hops is reached. It is also expected that the remote UE, when receiving multiple discovery messages from the intermediate relay UEs will choose the intermediate relay UE with the smallest multihop counter.  The following relay UE behavior for discovery was concluded in the previous meeting as follows:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

Based on the above FFS, it is not yet clear if a candidate relay UEs (U2N relay UEs or intermediate relay UEs) is allowed to act as a relay UE before it is RRC CONNECTED.  In our understanding, only a UE acting as a relay UE is allowed to perform discovery transmissions, so it should be further discussed if any candidate relay UE is required to be in RRC CONNECTED.

In contrast to the single-hop relay case, the multihop relaying scenario will contain many PC5 links which results in many RRC connections towards the gNB, esp. if we consider the possibility for more than two relay UEs in the path which results in a condition that is no longer scalable. Since RAN2 has agreed that “RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms”, for increasing the likelihood of successful connection establishment with the gNB, it should be assumed that, at the minimum the PC5 connection between relay UEs are already completed. If the remote UE selects the intermediate relay UE and the intermediate UE fails to connect to the U2N relay UE, it would result in failure in the remote UE’s relay (re)selection which should be prevented.

- Proposal 1	Candidate relay UEs in all the hops need to be PC5 connected prior to discovery transmissions.

If Proposal 1 is adopted, it would still allow the last relay UE to act as a relay UE regardless of its RRC state.  However, for further stability of a selected path, it would be beneficial for candidate intermediate relay UEs to be RRC CONNECTED prior to serving as a relay UE.  Furthermore, according to the SA2 LS [2], it is stated that “the intermediate U2N relay had already been authenticated and established a connection to the network before serving the remote UE (for discovery or connection)”.

- Proposal 2	Candidate relay UEs need to be RRC CONNECTED prior to discovery transmissions.

## 2.2	Relay (re)selection

For relay selection and reselection, during the email discussion [2], many companies think some of the legacy rules for the remote UE to trigger selection and reselection as below:

[Relay (re)selection triggering conditions for Remote UE]

- If the Remote UE has no serving cell;

- If the Remote UE does not have a selected intermediate Relay UE;

- If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold;

- If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold;

- If the upper layer indicates not to use the currently selected intermediate Relay UE;

- If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection;

- If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE;

- If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following:

h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED)

h-3) if intermediate Relay UE performs cell reselection

h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE

h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

In particular, a), b), c), d), f) seem to be agreeable by most companies, while most companies also think the above conditions are not used by intermediate relay UEs to trigger relay reselection. So, there remains the issue of under what condition(s) will it be appropriate for the intermediate relay UE to perform relay reselection. It is also necessary to consider if and when the intermediate relay UE should send notification message to the remote UE as described in h) above. In our view, the intermediate relay UE’s trigger conditions for relay reselection should be discussed together with the notification message delivered to the remote UE.

The following options for trigger conditions and notification delivery should be considered:

- The intermediate relay UE initiates relay reselection using the same triggering conditions adopted for the remote UE.  The intermediate relay UE does not send notification message to the remote UE if a suitable candidate relay UE (e.g., last relay UE) is selected.

- The intermediate relay UE initiates relay reselection using the same triggering conditions adopted for the remote UE.  The intermediate relay UE sends the notification message (w/ failure condition) to the remote UE, including any candidate relay UE(s) along with measured quantities (e.g., SL-RSRP/SD-RSRP, number of hops).

- The intermediate relay UE sends a notification message to the remote UE and it’s up to the remote UE to perform relay reselection. The intermediate relay UE is allowed to initiate discovery transmissions after PC5 connection to the remote UE is released.

- The intermediate relay UE does not initiate relay reselection upon detecting a failure condition, but it sends a notification message to the remote UE.  If needed, the remote UE should be allowed to request the intermediate relay UE to perform relay reselection (e.g., if the remote UE cannot find candidate relay UEs on its own).

With Option 1, the intermediate relay UE may act as a remote UE in terms of relay reselection and if a candidate relay UE from the same cell meets the SL-RSRP/SD-RSRP threshold condition(s) and the upper layer criteria is satisfied, the path re-routing can be done seamlessly without remote UE involvement during the relay reselection process. The intermediate relay UE will send a notification message to the remote UE only if a suitable candidate relay UE cannot be selected.

With Option 2, the intermediate relay UE reports to the remote UE, via the notification message, candidate relay UE(s) that it found. Upon receiving the notification message, the remote UE is allowed to trigger relay reselection on its own and decide whether to select a new intermediate relay UE or to stay with the existing intermediate relay UE.

With Option 3, the intermediate relay UE sends a notification message to the remote UE (w/ failure condition) which triggers the remote UE to perform relay reselection.  It is assumed that the intermediate relay UE is allowed to find a parent relay UE for discovery transmission once the PC5 connection with the remote UE is released.  As in the legacy procedure, the remote UE is allowed to select any of the available Intermediate relay UE that satisfies its criteria for relay reselection.

Option 4 is similar to Option 3, with the exception that the remote UE is allowed to request the intermediate relay UE to perform relay reselection if the remote UE cannot find a candidate relay UE meeting the necessary criteria.

Among the 4 options, Options 1, 2 and 4 have the benefit that intermediate relay UE is allowed to perform relay reselection, albeit under different triggering conditions.

- Proposal 3	Intermediate relay UEs should be allowed to perform relay reselection upon failure detection and whether it should send notification message to the remote UE.  It is FFS what triggers the intermediate relay UE to perform relay reselection.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay discovery and relay (re)selection.  RAN2 is kindly asked to take into account the proposals below:

- Proposal 1	Candidate relay UEs in all the hops need to be PC5 connected prior to discovery transmissions.

- Proposal 2	Candidate relay UEs need to be RRC CONNECTED prior to discovery transmissions.

- Proposal 3	Intermediate relay UEs should be allowed to perform relay reselection upon failure detection and whether it should send notification message to the remote UE.  It is FFS what triggers the intermediate relay UE to perform relay reselection.

# References

- R2-24xxxxx, “Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection” LGE

- S2-2407207, “LS on security considerations for 5G ProSe multi-hop relays”, Qualcomm