3GPP TSG-RAN WG2 #128	R2-2410578

Orlando, FL, USA, 18th – 22nd November, 2024

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on FFSs and further aspects on the following LCM topics for UE-sided models:

- Triggering of data collection for training and resource configuration for training

- Mobility

- Applicability reporting

- AI/ML functionality activation/deactivation

- Performance monitoring.

# 2	Discussion

## 2.1	Training

RAN2#127bis reached the following agreement regarding data collection for training at the UE side:

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

According to the FFS, one of the remaining aspects to discuss is how the NW determines whether data collection for UE-side training should be initiated. Our understanding of the two example approaches that are captured in the agreement is:

- the UE sends a training request directly to the gNB

- the UE server sends a training request to the CN and then the CN sends a request to the gNB.

Regarding the first approach (i.e. the gNB receives the request from the UE), we believe it should be supported, however the gNB must be able to allow/disallow the UE to send the request, as shown in Figure 1. For example, the network can use RRCReconfiguration (e.g. OtherConfig if the UE request is agreed to be transmitted via UAI), or even SIB signalling to indicate whether the UE is allowed to send this training request. Then the UE could send the training request, for example, in the UAI. The training request could also be signalled as part of the applicability functionality reporting, e.g. the UE could report that the AI/ML functionality is not applicable for a given configuration and that training is needed. Then the gNB can decide whether to configure radio resources for training.

Figure 1 Signalling between gNB and UE, for UE-side training initiated by the UE directly.

The gNB indicates to the UE whether the UE is allowed to request resources for data collection for UE-side model training (e.g. via RRCReconfiguration or SIB signalling).

If allowed, the UE can request training resources (e.g. via UAI or via the applicable functionality report) to start the data collection for UE-side model training, or to stop it.

Regarding the second approach (the gNB receives the request from the CN), it is outside of RAN2’s scope whether/how the CN can trigger a training request, upon receiving a request from a server. Thus, the second approach is more complex, including an impact on SA WGs, so the training request from the UE to the gNB (the first approach) is better, due to simplicity reasons.

It is out of RAN2 scope how the NW determines whether data collection should be initiated upon a request from a server.

Another important aspect is that the radio resources for training need a configuration, which should coexist with the legacy radio measurement configuration (used for conventional “real time” measurement reports). For example, the gNB may need to provide a specific measurement configuration (CSI-RS/SSBs) for the UE to perform data collection for training and for the related reporting, including the Set A/B, and the associated NW-side additional conditions. For example, it should be possible to configure the UE to not report measurement results associated to resources in which the UE is performing data collection for UE-side model training, given that such measurements are not intended to be used by the gNB for conventional “real time” decisions, such as scheduling, beam/mobility management, etc.

RAN2 to discuss how to configure radio measurement resources (including Set A/B, NW-side additional conditions, etc.) and radio measurement reports for data collection for UE-side training.

Another aspect that seems to be common to both solutions 1) and 2) outlined above is that the gNB can indicate to the UE to stop performing the UE-side data collection by just deconfiguring the radio measurements resources previously configured for such purpose.

When gNB deconfigures the radio measurement resources previously configured for the purpose of UE-side data collection, the UE stops the UE-side data collection.

## 2.2	Mobility

In this section we discuss our view on mobility aspects, regarding the consistency of the associated IDs across different cells and ensuring a fast activation of AI/ML beam management functionality at handover.

So far RAN1 has agreed that the associated IDs are cell specific. This is captured in RAN1#117 and confirmed in RAN1#118 agreements:

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We believe that this is sufficient and PLMN/area-specific associated IDs are not needed due to the following reasons.

- New PLMN/area-specific associated IDs will require extensive coordination across vendors, to ensure the uniqueness of the IDs. This diminishes the practical feasibility of this feature. Instead, we should aim at a solution simple to implement.

- There are already legacy ways for the UE to identify the area where it is located. The UE already knows the CGIs (which uniquely identify a cell) from SIB, so based on implementation it can store the cell-specific associated IDs together with their corresponding CGIs and thus obtain the equivalent of a unique PLMN/area-specific associated IDs.

For the above reasons, unless further agreements are reached in RAN1, we suggest RAN2 just taking the RAN1 working assumption for the normative work.

RAN2 to confirm the RAN1 working assumption that the associated IDs are consistent within a cell.

Regarding the fast activation of AI/ML beam management functionality at handover, we believe that for this first release of AIML, RAN2 should aim at specifying simple baseline support for mobility. The focus of RAN2 should be rather on specifying solid procedures within a cell (also considering the BM use case at hand), whereas complex optimizations for handovers, especially those involving RAN3, should be avoided.

The simplest support at handover for signalling related to applicable functionality reporting is shown in Figure 2. With this solution, the target gNB can send the RRC configuration with NW-side additional conditions (associated IDs) and/or inference configuration via the Handover Request Acknowledgement. The RRC configuration is the same as within a cell, so no additional RAN2/3 specification impact is foreseen. The source gNB forwards the configuration to the UE and then the UE can signal its response (in RRCReconfigurationComplete and/or UAI with applicable functionality reporting) to the target gNB upon completing the handover, as will be specified for the case within a cell.

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Figure 2 Baseline signalling for applicable functionality reporting in case of handover.

Other mobility optimizations come at the cost of complexity and scalability issues. Let us consider the example where the UE reports to the source some applicable functionality information about neighbouring cells, before the handover. Since the UE does not know at this point which neighbouring cells will become the target in the future, this solution would introduce overhead by extending the UE report in UAI, to include potentially useless information about some neighbouring cells that the UE will never be handed over to. Additionally, the source would need to store all this information in view of future handovers. Another optimization example with similar issues is when the neighbouring cells (potential targets) send applicable functionality information to the source, before the handover. This means again that potentially a large amount of information that may not be needed is sent to the source and the source needs to store it and decide at subsequent handovers what is actually useful. Additionally, this latter example has impact also in RAN3.

Related to mobility optimizations for applicable functionality reporting, RAN2 to down prioritize solutions requiring large signalling overhead, coordination between nodes, and impacts in other WGs.

## 2.3	Applicability reporting

In RAN2#127bis the following agreements were reached regarding functionality applicability reporting in the case of beam management with UE-sided model. In this section we provide our view on further issues to be discussed.

Agreements for BM

- 1. UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- 2. FFS if inference configuration can be signalled in step3.

- 3. UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

RAN2 has been using Figure 3 to illustrate the procedure of applicable functionality reporting. However, this diagram does not contain the RRCReconfigurationComplete messages that are sent by the UE as responses to Step 3 and Step 5. According to the first agreement above, the RRCReconfigurationComplete messages can also be used for applicable functionality reporting, so it is relevant to add them to the diagram, to ease further discussion on the applicability reporting content for stage-2. We thus propose to add Step 3b and Step 5b to the diagram, to explicitly mark these messages, as illustrated in Figure 4.

Figure 3: Signalling procedure for functionality applicability reporting.

Figure 4 Proposed updates for signalling procedure diagram.

Add Step 3b and Step 5b to the signalling procedure for functionality applicability reporting, to mark the RRCReconfigurationComplete messages for completeness, as shown in Figure 4.

Regarding other FFS points above, overall, in our opinion there could be two approaches for ensuring the consistency between training and inference, which determine the content of Step 3 and the content and signalling type of the applicable functionality reporting.

- Approach 1: Step 3 contains NW-side additional conditions

- Additionally, Step 3 contains one or more inference configuration(s) (including the NW-side additional conditions) for one or more functionalities, and the UE is requested to confirm the applicability of the received configuration(s) for those functionalities. This is aligned with the working assumption from RAN1#118 that “The associated ID at least can be configured within CSI framework”. Then, in Step 4 the UE indicates the one or more of the inference configuration(s) that is/are applicable. If there is at least an inference configuration signalled in Step 3 that is applicable, then the UE can acknowledge that it supports the inference according to such inference configuration and it can apply such inference configuration straight away; otherwise the UE operates according to a fallback configuration. In this case, Step 5 is not needed.

- Approach 2: Step 3 does not contain NW-side additional conditions

- The UE can signal in Step 4 the recommended/preferred associated ID(s) to make the AI/ML functionality applicable, e.g. based on the training data set used for generating the AI/ML functionality. Then in Step 5, the gNB can provide an inference configuration matching the recommendation/preference of the UE in Step 4.
Step 3 can still contain one or more inference configurations, and the UE could do a similar evaluation as in approach 1, without the need to receive an inference configuration in Step 5.

As discussed for Approach 2, Step 3 does not always need to contain NW-side additional conditions. Therefore, the indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is an optional feature and can be done via RRC signalling.

The indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is optional.

As discussed for Approach 1 and Approach 2, it should be possible for the network to provide an inference configuration already in Step 3. For example, the gNB can provide the UE with an AI/ML-related resource configuration (e.g. CSI measurement configuration with Set A and/or Set B) for model inference purposes. This is possible because the network may have previously stored applicability information from this UE, or it may have collected applicability information from other similar UEs. In such cases, providing an inference configuration in Step 3 has the benefit of signalling overhead reduction, since Step 5 may be skipped, and it allows a quick configuration/activation of the AI/ML functionality, without the need to go through two loops of RRC reconfigurations (i.e. one RRC Reconfiguration to configure the UE to report the applicability reporting, and one to configure the UE for the AI/ML inference), which is very desirable, e.g. during the mobility. Additionally, the network may be interested in providing the UE only with certain specific inference configurations to optimize the reference signal transmission overhead (e.g. using a common Set B across multiple UEs to minimize the number of Set B reference signal transmissions). 
Given the examples provided above, we believe that there exist valid use cases for which relying exclusively on the step-5 for the provisioning of the inference configuration, is not only inefficient from a signalling point of view and UE performances, but also completely unnecessary.

There exist valid use cases (e.g. network already knowing the applicability information of the UE, network interested only in certain inference configuration(s)) for which relying exclusively on Step 5 for the provisioning of the inference configuration, is not only inefficient from a signalling point of view and UE performances, but also completely unnecessary.

Furthermore, Step 3 could provide a performance monitoring configuration.

The network can provide inference and/or performance monitoring configuration(s) in Step 3.

Given these possible cases for Step 3, the content of Step 3 can be similar to that of Step 5, regarding the configuration for inference/performance monitoring. Regardless of when the inference configuration is sent from the NW to the UE (in Step 3 or Step 5), the UE should behave in the same way. That means the UE should in any case check the functionality applicability when receiving an inference configuration and report to the NW if it is not applicable, irrespective of whether the inference configuration is received in Step 3 or Step 5. This is because it is still possible that an inference configuration sent in Step 5 is not applicable, e.g. due to changing UE conditions between transmission of Step 4 (transmission of applicability report), and reception of Step 5 (reception of inference configuration). Thus, it is important that the UE always handles the inference configuration the same way, no matter if that is received in Step 3 or Step 5.

It should be assumed that the UE always evaluates the applicability of the AIML functionalities upon receiving an inference configuration, regardless of whether the inference configuration is sent in Step 3 or Step 5.

As discussed above, the content of Step 4 depends on the content of Step 3. For instance, for Approach 2, if Step 3 does not contain an inference configuration, it should be possible that the UE replies with more detailed information about the functionality applicability and its preferred configuration. It is thus meaningful to signal such information in the UAI, which already provides a framework to signal detailed UE preferences.

However, for Approach 1 (Step 3 contains NW-side additional conditions and also one or more inference configurations), it is sufficient that the UE replies acknowledging whether the inference configuration is applicable or which one of them is applicable (in case more inference configurations are provided). It is thus suitable for this latter approach to carry this acknowledgement in the RRCReconfigurationComplete message (Step 3b and Step 5b).

After the network sends one or more inference configuration(s) in RRCReconfiguration, the UE reports applicable functionality via RRCReconfigurationComplete, by indicating whether the inference configuration is applicable/non-applicable, or which one of them is applicable/non-applicable.

Let us now discuss the FFS on how the UE can report to the network when an applicable AI functionality becomes non-applicable or vice versa. In our opinion, this case refers to a situation after Step 5, when an AI functionality that was previously reported by the UE as applicable now becomes non-applicable or vice versa. This means that by this time the network has already sent to the UE the related configurations or functionality applicability report requests for some specific functionalities that are of interest to the network, in Step 3 or Step 5. Thus, when there is a change at the UE in a functionality status (either from applicable to non-applicable or vice versa), the UE should report to the network only information about the functionalities that have been previously configured/requested by the network. The applicable/non-applicable status of other functionalities that the UE supports as capabilities, but that are not requested by the network, should not be included. This avoids unnecessarily long reports with information that is not needed at the network.

Then, upon applicability changes and out of the configured/requested functionalities, the UE should include in the report only functionalities whose applicable/non-applicable status has changed. This is consistent with the information typically reported by the UE in legacy procedures, upon changes. Further, it is beneficial if the UE reports which functionalities became non-applicable, since this way the UE could also include the reason for each individual change to non-applicable. Additionally, it is beneficial if the UE also reports which functionalities became applicable. This is to avoid ambiguities at the network side. For example, let’s consider the case in Figure 5, where Config_1 becomes applicable and Config_2 becomes non-applicable around the same time. If the UE reports only changes from applicable to non-applicable (Config_2 in this example), then the network does not know what happened to Config_1. Namely, it is ambiguous whether Config_1 became applicable, or whether Config_1 was not stored at the UE at all, since the UE may have deleted Config_1 when it initially reported it as non-applicable. To avoid this ambiguity, the UE should include in the report also functionalities that became applicable. Moreover, such a UE report including both applicable and non-applicable functionalities can be used not only upon applicability changes, but also when the network sends for the first time AI configurations or requests for applicable functionality reports. Hence, the UE can reuse the same report type, which makes the specification changes and implementation easier.

Figure 5 Example of two applicability changes occurring around the same time.

Among those functionalities previously configured/requested by the network in Step 3 or Step 5 and whose applicability status has changed, the UE includes in the report for each of such functionalities whether the functionality is applicable or non-applicable.

Regarding the different scenarios with active or inactive functionalities, in our opinion the UE report should not preclude either of them. Whether an active/inactive functionality is relevant to be reported as applicable/non-applicable depends on the activation type and RAN1 definition of “active” for each configuration type. For instance, for AI-based periodic beam prediction reporting, if the configuration is active, then the UE would report the updated applicable/non-applicable status for an active configuration. In another case, if the network has configured this periodic reporting with the response of non-applicable functionality from the UE but this functionality becomes applicable later, then the UE would report the updated applicable status for this inactive configuration. For the examples of AI-based semi-persistent or aperiodic beam prediction reporting, the configurations may not be active (not triggered by MAC CE or DCI), but they are still configured and are thus of interested to the network. Therefore, the UE should report the applicable/non-applicable status of such inactive but configured functionalities.

When a change in applicability of an AI functionality occurs, the UE report includes both active and inactive functionalities, according to RAN1 definition of “active”.

## 2.4	Functionality activation/deactivation

RAN2#127 has agreed the following:

Agreements on procedures - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality.

Regarding the FFSs on activation/deactivation of inference functionality, RAN2 has already sent a question about this to RAN1 in the LS in [1]. Thus, RAN2 should wait for RAN1’s reply before discussing further related signaling details. In the meantime, RAN2 can assume that when there is no applicable AI/ML functionality for a UE, the UE can apply a fallback non-AI/ML configuration. The fallback non-AI/ML configuration can be a legacy CSI reporting configuration that the NW can configure and activate based on existing procedures.

For fallback purposes, the network can provide a non-AI/ML CSI reporting configuration according to legacy procedures.

## 2.5	Performance monitoring

For the performance monitoring phase, the UE may report performance results of functionality monitoring to the NW, e.g., accuracy, or other performance metrics. In our view, which specific layer (Layer 1/2/3) should be adopted to report this information may depend on the type and amount of information to be reported, and the time when this information should be reported. Since all these issues seem to be more in the RAN1 domain and the impact in RAN2 protocols might be limited, we suggest letting RAN1 drive this topic.

RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is out of RAN2 scope how the NW determines whether data collection should be initiated upon a request from a server.

Observation 2	There exist valid use cases (e.g. network already knowing the applicability information of the UE, network interested only in certain inference configuration(s)) for which relying exclusively on Step 5 for the provisioning of the inference configuration, is not only inefficient from a signalling point of view and UE performances, but also completely unnecessary.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The gNB indicates to the UE whether the UE is allowed to request resources for data collection for UE-side model training (e.g. via RRCReconfiguration or SIB signalling).

Proposal 2	If allowed, the UE can request training resources (e.g. via UAI or via the applicable functionality report) to start the data collection for UE-side model training, or to stop it.

Proposal 3	RAN2 to discuss how to configure radio measurement resources (including Set A/B, NW-side additional conditions, etc.) and radio measurement reports for data collection for UE-side training.

Proposal 4	When gNB deconfigures the radio measurement resources previously configured for the purpose of UE-side data collection, the UE stops the UE-side data collection.

Proposal 5	RAN2 to confirm the RAN1 working assumption that the associated IDs are consistent within a cell.

Proposal 6	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Proposal 7	Related to mobility optimizations for applicable functionality reporting, RAN2 to down prioritize solutions requiring large signalling overhead, coordination between nodes, and impacts in other WGs.

Proposal 8	Add Step 3b and Step 5b to the signalling procedure for functionality applicability reporting, to mark the RRCReconfigurationComplete messages for completeness, as shown in Figure 4.

Proposal 9	The indication of NW-side additional condition (i.e. associated ID) from the network to the UE in Step 3 is optional.

Proposal 10	The network can provide inference and/or performance monitoring configuration(s) in Step 3.

Proposal 11	It should be assumed that the UE always evaluates the applicability of the AIML functionalities upon receiving an inference configuration, regardless of whether the inference configuration is sent in Step 3 or Step 5.

Proposal 12	After the network sends one or more inference configuration(s) in RRCReconfiguration, the UE reports applicable functionality via RRCReconfigurationComplete, by indicating whether the inference configuration is applicable/non-applicable, or which one of them is applicable/non-applicable.

Proposal 13	Among those functionalities previously configured/requested by the network in Step 3 or Step 5 and whose applicability status has changed, the UE includes in the report for each of such functionalities whether the functionality is applicable or non-applicable.

Proposal 14	When a change in applicability of an AI functionality occurs, the UE report includes both active and inactive functionalities, according to RAN1 definition of “active”.

Proposal 15	For fallback purposes, the network can provide a non-AI/ML CSI reporting configuration according to legacy procedures.

Proposal 16	RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

# References

R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.