3GPP TSG-RAN WG2 Meeting #128							R2-2410587

Orlando, USA, 18th – 22nd November, 2024

Title: 	Relay discovery and (re)selection for multi-hop Relay

Source: 	Huawei, HiSilicon

Agenda item:	8.13.2

Document for:	Discussion/Decision

# Introduction

At RAN #104 meeting, the new work item on NR sidelink multi-hop relay [1] was approved and in RAN 2# 127 meeting we had made some agreements for overall design and relay discovery and (re)selection  for NR sidelink multi-hop relay which are listed as below. In this paper, we discuss the relay discovery and relay (re)selection aspects.

Agreements for overall design: - n-hop relaying involves n relay UEs by definition. - Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion - Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion - Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion - FFS if we will capture the requirements according to these terms when implementing CRs. - FFS link terminology. - RAN2 intend to align with SA2 terminology when it is stable - A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage. FFS if acting as a relay requires it to transition to RRC_CONNECTED. - Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage. - RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms. - RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19. Agreements for relay discovery and (re)selection: - From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

# Discussion

Rel-19 Multi-hop relay can further extend the coverage by supporting the multiple hop sidelink relay on top of Rel-17 U2N relay. Figure 1 shows the scenario where the remote UE connects with the network through two relay UEs, i.e., remote UE -> first relay UE -> last relay UE -> gNB.

Figure 1. Multi-hop relay with two relaying hops

In this paper, we use the terminology agreed in RAN2#127 meeting.

- last relay UE: the U2N relay UE directly connected to the gNB

- intermediate relay UE: the relay UEs except for the last relay UE

- first relay UE: the intermediate relay UE directly connected to the remote UE

## General design for topology

In the email discussion [Post127][401][Relay] [2], The possible topology for the multi-hop relay were discussed quite a lot. We believe this is the most important issue to be handled since different working assumption for the topology can lead to different design. In our understanding the multi-hop relays should have tree-like topology for the following reasons.

First, the Rel-17 U2N relay supports the tree-like topology. In Rel-17, the relay UE connects to one gNB (and to one serving cell) and can serve multiple remote UEs, while one remote UE only connects to one relay UE. As stated in the WID [1] as shown  below, the main aim of multi-hop relay is to solve the range limitation of a single hop sidelink relay and the enhanced mechanism should be considered on top of Rel-17 relay. Therefore, it is more straightforward to follow the tree-like topology design used in Rel-17, which allows  Rel-17 mechanism  to be reused as much as possible.

## 3 Justification 3GPP RAN completed a work item “NR Sidelink Relay” in Rel-17 and a work item “NR sidelink relay enhancements” in Rel-18 in order to cover the enhancements and solutions necessary to support the UE-to-network Relay coverage extension which consider a wide range of V2X, Public Safety and commercial applications and services. The 5GS has been also enhanced to support Proximity Services in Rel-17 and Rel-18 as specified in TS 23.304, based on the service requirements defined in TS 22.278, TS 22.261 and TS 22.115. However, single hop U2N relays that have been developed in Rel-17 and Rel-18 have limited applicability because of the range limitation of a single hop sidelink relay. Thus, additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay. A study item for ProSe phase 3 has been approved in SA in order to investigate further 5G system enhancements to support Proximity Services in Rel-19. RAN-side enhancements for sidelink relay, particularly multi-hop Layer-2 UE-to-Network relay, is necessary in accordance with the SA work. ## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

Secondly, the control plane aspects (like the local ID allocation, QoS split, RRC configuration) can be simpler since the gNB can control all the UEs if there is the tree-like topology. Considering the limited TUs allocated for the multi-hop relay topic, RAN 2 should agree to adopt the tree-like topology to simplify the discussion related to topology.

Based on the tree-like topology, the intermediate relay UE connects to only one parent relay UE, and it will take the cell of the parent relay UE as its serving cell.  In this architecture, the gNB can control all the UEs, including the remote UE and the relay UEs, which we think is the basic principle of the approach 1 in the email discussion [Post127][402][Relay] [3]. In the email discussion, we have clarified that approach 1 is a natural extension of the Rel-17 U2N relay mechanism, and can have several benefits such as  simplified local ID allocation, QoS handling, RRC State management.

Proposal 1: Considering the limited TUs allocated for the multi-hop relay, RAN2 agrees that the multi-hop relay operates in a tree-like topology, which involves just the hop extension based on the Rel-17 U2N relay mechanism. Consequently Rel-17 U2N relay mechanisms can be reused as much as possible.

## Conditions for performing Relay discovery

In RAN2#127 meeting, we have the following agreements for the Remote UE and last relay UE to perform discovery, but the condition for performing discovery for the intermediate relay UE are still not clear.

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

Based on proposal 1, we think there two possible cases for the intermediate relay UE.

Case 1: The intermediate relay UE has already established the PC5 connection with its parent relay UE as a remote UE, e.g., for its own data communication before serving as the relay UE.

Case 2: The intermediate relay UE has not established the PC5 connection with a parent relay UE yet.

In case 1, the intermediate relay UE can perform the discovery if it can provide the relaying service for multiple remote UEs. No AS condition is needed since there is available RRC connection with the network.

Case 2 would happen when the intermediate relay UE has just performed the relay (re)selection. In this case, we think the intermediate relay UE can first perform the discovery as a remote UE, when the intermediate relay UE connects to parent relay UE, it will perform the discovery as a relay UE. Based on the above procedure, the Rel-17 discovery procedure can be reused for the intermediate relay UE to find a parent relay UE and for the remote UE to find the first relay UE.

Proposal 2: Reuse the Rel-17 direct discovery procedures for the intermediate relay UE to find a parent relay UE first and then for the remote UEs to find the first relay UE in multi-hop scenario.

Technically, we should guarantee that the indirect link between the intermediate relay UE and the network should be in good condition. As least, the intermediate relay UE needs to make sure that the PC5 link between the intermediate relay UE and its parent relay UE is in good condition. If the link quality between the intermediate relay UE and its parent relay UE is not in good condition, the intermediate relay UE can trigger a relay reselection. Therefore, we think no AS condition need to be checked by the intermediate relay UE before performing the discovery message transmissions.

Proposal 3: The intermediate relay UE performs discovery procedure as a relay UE after establishing the PC5 connection with its parent relay UE. In this case it does not need to check the AS condition.

## Relay selection and reselection

Relay selection and reselection of the remote UE:

For Relay selection and reselection procedures, the legacy PC5 radio measurements mechanism can be reused, including:

- The U2N Remote UE uses the SD-RSRP measurements to evaluate the link quality if there is no PC5 connection between the U2N Remote UE and the candidate Intermediate Relay UE.

- The U2N Remote UE uses the SL-RSRP measurements to evaluate the link quality between the Remote UE and the serving Intermediate Relay UE. Serving Intermediate Relay UE is the Intermediate Relay UE that is providing the relaying service for the U2N Remote UE.

- It is left to Remote UE implementation whether to use SL-RSRP or SD-RSRP or both for measurement  if there is no data transmission from serving Intermediate Relay UE to U2N Remote UE.

Proposal 4: Release-17 PC5 link measurement and evaluation principles for relay selection and  reselection can be reused for multi-hop relay selection and reselection.

Similar to Release 17, the U2N Remote UE can trigger U2N relay selection in following cases .

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE

Hence, we propose the following

Proposal 5: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE  and the remote UE  is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE. The serving Intermediate Relay UE sends the notification message when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens.

- Upon receiving the notification message from the serving Intermediate Relay UE, which happens when the serving Intermediate Relay UE receives the notification message indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 6: The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 7: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens, or upon receiving the notification message from the U2N Relay that indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

There can be several candidate U2N Relay UE(s) or candidate Intermediate Relay UE(s) around the U2N Remote UE when performing relay selection and reselection. In the current spec, the only information the Remote UE has about  the candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, we think this information may  not be enough to select a good  relay UE. Considering the case that the Remote UE can discover the following candidate relay UE(s) in the following.

- relay UE 1: the SD-RSRP is value 1, and the relay UE 1 is connected to the gNB with a U2N Relay UE. Value 1 satisfies the relay selection criterion, i.e., above the configured threshold.

- relay UE 2:  the SD-RSRP is value 2, and the relay UE 2 is connected to the gNB with a U2N Relay UE and an Intermediate Relay UE. Value 2 satisfies the relay selection criterion, and Value 2 is larger than value 1.

Based on the PC5 link quality, the Remote UE will select the relay UE 2 with a high probability, since the PC5 link quality between the Remote UE and the relay UE is better. However, from the QoS perspective, the relay UE 1 can be better since the hop number between the relay UE 1 and the network is smaller. Therefore, we think it is better to indicate the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure, which can be used to select a better relay UE.

Proposal 8: RAN2 can discuss how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

Relay selection and reselection of the intermediate relay UE:

In the email discussion [Post127][401][Relay] [2], we also discussed the relay selection and reselection. Based on the tree-like topology, the intermediate relay UE performs relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario, i.e., as proposed in proposal 4 to proposal 8.

Proposal 9: In general, the intermediate relay UE can perform the relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario.

# Conclusion

In this paper, we discuss the relay discovery and relay (re)selection aspects and provide the following observations and proposals.

General design for topology

Proposal 1: Considering the limited TUs allocated for the multi-hop relay, RAN2 agrees that the multi-hop relay operates in a tree-like topology, which involves just the hop extension based on the Rel-17 U2N relay mechanism. Consequently Rel-17 U2N relay mechanisms can be reused as much as possible.

Conditions for performing Relay discovery

Proposal 2: Reuse the Rel-17 direct discovery procedures for the intermediate relay UE to find a parent relay UE first and then for the remote UEs to find the first relay UE in multi-hop scenario.

Proposal 3: The intermediate relay UE performs discovery procedure as a relay UE after establishing the PC5 connection with its parent relay UE. In this case it does not need to check the AS condition.

Relay selection and reselection

Proposal 4: Release-17 PC5 link measurement and evaluation principles for relay selection and  reselection can be reused for multi-hop relay selection and reselection.

Proposal 5: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

Proposal 6: The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 7: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens, or upon receiving the notification message from the U2N Relay that indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

Proposal 8: RAN2 can discuss how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

Relay selection and reselection of the intermediate relay UE:

Proposal 9: In general, the intermediate relay UE can perform the relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario.

# References

RP-242349, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

R2-2410305, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection

R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane.