3GPP TSG-RAN WG2 Meeting #128	R2-2410592

Orlando, FL,USA, 18 – 22 November 2024

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we focus on the issues related to functionality-based LCM for UE-side AI/ML beam management. First, we review agreements made in RAN1 in response to the LS sent out by RAN2. Then, we discuss the procedural impacts for reporting applicable configurations and for controlling configurations, e.g., activating, deactivating, switching, or falling back from configurations are detailed respectively. Next, we discuss the remaining issues concerning Step 4, data collection initiation, the retention of applicable configurations through RRC state transitions in the same cell, configuration performance monitoring, and the reporting of applicable configurations during handover.

## 2	Discussion

## 2.1 RAN1 agreement concerning LS

In RAN1#118bis, the following was made related to RAN2 LS:

Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

As stated in above agreement, a functionality is an AI/ML-enabled configuration constructed using conditions indicated by UE capabilities. As such, in the beam management use case, a functionality in general (including applicable, activated and supported) is represented by a CSI-ReportConfig.

Observation 1: A functionality (or configuration) in functionality-based LCM is a configuration constructed using conditions indicated by UE capabilities.

Proposal 1: In the beam management use case, an AI/ML configuration is signalled by CSI-ReportConfig.

## 2.2 Functionality-Based LCM

### 2.2.1	Procedures

The following agreements from RAN2#125bis [1] will be used as a basis for the next part of our discussion, namely functionality management for a UE-sided (functionality) based on the “network decision, network-initiated” approach, which was defined in the TR 38.843 [2], shown following the agreements.

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

The following figure, Figure 1, extracted from TR 38.843 [2], Section 7.2.1.1, illustrates “network decision, network-initiated” AI/ML LCM. Of note in this call flow is that the UE is not pre-configured with any sort of event triggered LCM actions and that all aspects of configuration originate from the network, including measurement and reporting configuration and functionality activation, deactivation and switching.

Note that from this point forward, we refer to “applicable functionalities” as “applicable configurations” as agreed and discussed during RAN2#127, but the term functionality is still used in the phrase “functionality-based LCM” because it has a specific meaning that is different from “model-based LCM”.

Figure 1: Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management [2]

Observation 2: In “network decision, network-initiated” AI/ML LCM, all aspects of configuration originate from the NW, including measurement and prediction reporting.

Sometime after an AI/ML BM measurement reporting configuration has been configured, it might become inapplicable. The change in applicability would have triggered a new applicable configuration report from the UE to the gNB, on which the gNB would need to act. Depending on whether the AI/ML BM measurement reporting is crucial for NW operation, the gNB could take one of a few different actions:

- De-configure an AI/ML BM configuration.

- De-configure a configured AI/ML BM configuration, simultaneously configuring a different applicable AI/ML BM configuration.

- De-configure a configured AI/ML BM configuration, simultaneously configuring a non-AI/ML BM configuration.

As written in our RAN1 contribution [3], CSI reports for both BM-Case1 and BM-Case2 based on configurations for aperiodic CSI reporting (e.g., CSI-AssociatedReportConfigInfo) can already be triggered on-demand using a DCI, and CSI reports based on configurations for semi-persistent CSI reporting can already be triggered on-demand using a MAC-CE for PUCCH and using a DCI for PUSCH. In both cases, measurement reporting is activated through a DCI or MAC-CE, measurement reporting is deactivated implicitly at the end of a single report or through a DCI or MAC-CE, and the switching or fallback of configurations is implemented implicitly through the choice of which report to activate.

Observation 3: The specifications already support measurement reporting activation and deactivation for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Periodic CSI reporting configurations can be activated or deactivated through RRC Reconfiguration, e.g., by adding or modifying, or removing a CSI-ReportConfig. The typical RRC Reconfiguration procedure requires the transmission of two messages: RRCReconfiguration and RRCReconfigurationComplete, while for the aperiodic and semi-persistent, only one message, a DCI or MAC-CE, which can be transmitted alongside normal data traffic, is required.

When using the RRCReconfiguration message to configure an AI/ML BM configuration, the ToAddMod and ToRemove structures can be used, wherein new CSI-ReportConfigs can be added or modified, or removed.

Observation 4: The specifications support CSI measurement reporting activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Because of the difference in procedures to activate CSI measurement reporting, we find it necessary to separately discuss the configuration of an AI/ML BM feature and the triggering of AI/ML BM prediction reporting since the behaviour is dependent on which type of reporting has been configured. The following proposal relates to the activation, deactivation, and switching of CSI reporting, but not to the initial configuration thereof.

Proposal 2: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement AI/ML BM reporting activation, deactivation, switching, and fallback.

### 2.2.2	Fallback

As written in Section 2.2.1, fallback means de-configuring one AI/ML BM configuration and activating a non-AI/ML BM configuration. The purpose of fallback is to mitigate performance losses by reverting to a non-ML method for beam management. However, the NW may fall into unnecessary ping-pong between non-AI/ML and AI/ML-enabled beam management configurations, which could lead to signalling overhead to disabling/enabling ML configuration on one hand and imposing unnecessary activation delays on the other hand.

Because the performance of the AI/ML BM configuration was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Observation 5: Because the performance of the AI/ML BM configuration was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Therefore, monitoring an AI/ML BM configuration during fallback could be useful to help the NW determine when to exit fallback and resume the use of the AI/ML BM configuration to make beam management decisions. During this type of fallback, the AI/ML BM reporting configuration would still be active, but it could be configured with a lower reporting frequency.

Proposal 3: Discuss signalling to enable fallback without deactivating an AI/ML BM configuration such that performance monitoring of an AI/ML BM configuration can continue during fallback.

## 2.3 Remaining issues concerning Step 4

RAN2#128 made the following agreement regarding applicability reporting:

Agreement - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We shdould aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

Once applicable CSI-ReportConfigs for inference configuration are reported, the gNB will be able to activate one or more of the applicable configurations, i.e., CSI-ReportConfigs.

Further study concerning reporting of applicable AI/ML configurations can be addressed by applicable configuration report considering:

- Updates on applicability: updates to applicability could arise from performance decrease or increase, loss of a model, acquisition of a model (e.g., a fully new model, an update to a model, etc), or change in consistency with UE-side and NW-side additional conditions.

- Content of report: applicability and/or non-applicability of provided configurations and any reasons, therefore.

Proposal 4: For AI/ML BM applicable configuration reporting, the CSI-ReportConfigId is signalled in the applicable configuration report.

The UAI is optionally used by the UE to signal updates to previously signalled capabilities.  As such, UAI can be reused to enable the UE to send applicability updates for various reasons such as changes of prediction accuracy level or evaluation of UE internal conditions. For instance, the inference configurations may change the accuracy level as the UE moves from two areas of the cell. In this case, it may be beneficial to include in UAI report such updates, so that the NW can more easily configure and activate the inference configurations with the better performance. The ongoing discussion in RAN1 about performance monitoring metrics (e.g., beam prediction accuracy or RSRP prediction accuracy) may also be reused for deciding which metric include in the applicability reporting.

Proposal 5: Further discuss content for reporting applicability of provided configurations and the specific reasons for applicability indication such as changes of prediction accuracy level. FFS on possible indication in the assistance information message of the reasons, e.g. accuracy information alongside applicability indication.

To enable efficient configuration of AI/ML BM inference reports, it is important for the UE to be able to report applicable configurations as soon as possible. Taking a semi-persistent reporting configuration type as an example, a UE which reported that a particular CSI-ReportConfigId corresponding to a CSI-ReportConfig with a reportType of semi-persistent on PUCCH could be configured to activate reporting from an AI/ML BM configuration with a MAC-CE immediately after the gNB received a UE’s RRCReconfigurationComplete message containing the report.

Proposal 6: Support the use of RRCReconfigurationComplete to signal the initial applicable configuration report following a configuration/reconfiguration.

As mentioned above, when the UE is performing inference (associated with an AI/ML BM configuration), it may send an updated applicable configuration report, e.g., triggered in response to changes in the UE internal conditions, information or indications associated with the NW-additional conditions, or performance assessments of AI/ML BM configurations. If the content of the applicable configuration report is small, then implicit reporting of non-applicable configurations through omission would be the most straight-forward approach because the overall report size would be small, and the extra field for non-applicability would not be required.

Observation 6: If the content of the applicable configuration report is small, then implicit reporting on non-applicable configurations through omission would be the most straight-forward approach because the overall report size would be small, and the extra field for non-applicability would not be required.

However, if the applicable configuration report contains significant additional information, it could be inefficient to signal all applicable configurations every time. Therefore, we limit our proposal to the assumption that an applicable configuration report is small.

Proposal 7: Assuming that an applicable configuration report is small, non-applicable configurations will be initially reported, through UAI or RRCReconfigurationComplete and updated through UAI by their omission, through UAI or RRCReconfigurationComplete. FFS how to handle the case when no configurations are applicable during Step 3.

## 2.4 Retention of Applicable Functionalities in IDLE and INACTIVE

Apart from identifying the basic framework for reporting applicable configurations between the network and the UE, changes to the UE’s RRC state, i.e., transitioning between CONNECTED, IDLE, and INACTIVE, may impact the procedure for applicable configuration reporting. As an example, it is unclear whether the UE should be mandated to perform the applicable configuration reporting procedure after Step3   when the UE transitions to IDLE or INACTIVE and returns to CONNECTED mode.

Proposal 8: RAN2 to discuss whether applicable configurations are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable configuration reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

## 2.5 Data collection initiation and configuration

The gNB, which is responsible for configuring UEs with AI/ML-based and non-AI/ML-based beam management configurations, is in the best position to determine configurations for data collection. The prominent component of CSI-based mechanisms are the CSI reference signals, which are configured along with CSI measurement reporting. It is these CSI-RS which the UE would measure during a data collection phase. Therefore, it is most logical that a gNB would be in control of determining data collection configurations.

Observation 7: The gNB configures CSI-RS transmissions and is, thus, the best entity for determining CSI-RS data collection configurations.

Proposal 9: The gNB is the entity which configures the UE for data collection for UE-side AI/ML models for beam management.

## 2.6	Performance Monitoring

### 2.6.1	Overview

During the Release 18 SI, two types were introduced for performance monitoring of UE-side functionalities.

Type 1, Option 1, enables the NW to make LCM decisions based on performance monitoring metrics calculated at the NW. Type 1, Option 2, also known as UE-assisted performance monitoring, enables the NW to make LCM decisions based on performance monitoring metrics calculated and reported by the UE to the NW. Type 1 performance monitoring, including Option 1 and Option 2, may require signalling enhancements such that the UE can be configured to report additional measurements for use by the NW in calculating a performance metric or such that the UE can be configured to calculate and report a performance metric to the NW.

Type 2 performance monitoring enables the UE to make LCM decisions based on performance monitoring metrics calculated by the UE. Type 2 does not require any RAN2-related signalling enhancements as it is mainly applicable to model-level performance monitoring.

In RAN1#118 [4], four alternatives were agreed as performance monitoring metric for further study. As such, any discussions on the specific signaling or triggering of additional measurement reporting or KPI calculation in RAN2 should be discussed after RAN1 progresses on deciding the performance monitoring metric.

Proposal 10: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

### 2.6.2	System-Level Performance Monitoring

Monitoring Configuration 0 (System-Level Performance Monitoring). Configure a BM configuration, which outputs spatial or temporal beam predictions and perform monitoring solely based on inference output and other metrics, e.g., system performance (Figure 2).

- L1-RSRP Measurements for Inference include measurements of Set B for BM Case-1 or Set A measurements within observation window for BM Case-2.

- Configuration output includes spatial beam predictions of Set A beams for BM Case-1 or temporal beam predictions of Set A beams for BM Case-2, which are required as inputs to the BM configuration.

- The NW uses the output of System-Level Performance Monitoring to make functionality-based LCM decisions.

Figure 2: System-Level AI/ML Functionality Performance Monitoring

Observation 8: There is no specification impact for the use of NW-based system-level performance monitoring to enable BM configuration performance monitoring.

### 2.6.3	Type 1 Performance Monitoring

The following Monitoring Configurations and block diagrams illustrate potential variations of NW-based monitoring with and without UE assistance. In RAN1, this is called Type 1 performance monitoring. In every case of Type 1 performance monitoring, an AI/ML configuration is activated, and additional information is used to determine the quality of the performance of the AI/ML configuration.

#### 2.6.3.1	Type 1, Option 1 Performance Monitoring

Monitoring Configuration 1 (Performance Monitoring Type 1, Option 1). Configure a BM configuration, which outputs spatial or temporal beam predictions and perform monitoring based on additional measurements of Set A considering a procedure as follows (Figure 3):

- L1-RSRP Measurements for Inference are configured by the NW and include measurements required as inputs to the BM configuration, e.g., measurements of Set B for BM Case-1 or measurements of Set A within the observation window for BM Case-2.

- The BM Configuration Outputs, e.g., L1-RSRP and/or RS index, are reported to the NW.

- L1-RSRP Measurements for Monitoring, required for NW-based KPI calculation, are reported by the UE.

- KPI calculation is based on the measurement results obtained in (3) and the configuration output obtained in (2).

- The NW uses the output of the NW-based KPI calculation to make NW-based functionality-based LCM decisions.

Figure 3: Type 1, Option 1 AI/ML Functionality Performance Monitoring

#### 2.6.3.2	Type 1, Option 2 Performance Monitoring

Monitoring Configuration 2 (Performance Monitoring Type 1, Option 2). Configure a BM configuration, which outputs spatial or temporal beam predictions and perform monitoring based on assistance provided by the UE, e.g., UE-based KPI calculation considering a procedure as follows (Figure 4):

- L1-RSRP Measurements for Inference are configured by the NW and include measurements required as inputs to the BM configuration, e.g., measurements of Set B for BM Case-1 or measurements of Set A within the observation window for BM Case-2.

- The BM Functionality Outputs, e.g., L1-RSRP and/or RS index, are reported to the NW.

- L1-RSRP Measurements for Monitoring are used for UE-based KPI calculation,

- UE-based KPIs are calculated and reported to the NW and used to make NW-based functionality-based LCM decisions.

Figure 4: Type 1, Option 2 AI/ML Functionality Performance Monitoring

Observation 9: Based on discussions on candidates for KPI calculation for AI/ML BM configuration performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated BM configuration are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 10: Based on discussions on candidates for KPI calculation for AI/ML BM configuration performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the configuration outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the configuration outputs.

Proposal 11: RAN2 only to support performance monitoring of an activated AI/ML BM configuration through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both BM-Case1 and BM-Case2.

## 2.7 Reporting of Applicable Functionalities for neighbouring cell

In [5], the Rapporteur provided an open question based on companies’ contributions on whether applicable configurations should be considered for neighbour cells. Highlighting our views, UAI framework can only report UE’s condition/information changes on serving cell (i.e. the UE is only required to monitor condition changes in serving cell in existing UAI framework). Evaluation of applicable configurations is per-cell, and we don’t consider necessarily keeping the same configuration active through handover (e.g., to be used during the handover process).

Since the existing UAI procedure can only work for reporting applicable configurations of the serving cell, if conditions in neighbour cells change, the UE will not be able to use UAI for applicable configuration reporting. Evaluation of applicable configurations is per-cell, and we don’t consider necessarily keeping the same configuration active through handover (e.g., to be used during the handover process). Additionally, we do not think that AI/ML configuration applicability should be used as a criterion for handover. As such, it is worth discussing the feasibility of using RRCReconfigurationComplete and RRCResumeComplete to report applicable configurations of a target cell in RAN2.

Proposal 12: RAN2 to discuss whether applicable configuration reporting to a target cell is supported during handover.

# 3	Conclusion

This document has made the following observations:

Observation 1: A functionality (or configuration) in functionality-based LCM is a configuration constructed using conditions indicated by UE capabilities.

Observation 2: In “network decision, network-initiated” AI/ML LCM, all aspects of configuration originate from the NW, including measurement and prediction reporting.

Observation 3: The specifications already support measurement reporting activation and deactivation for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Observation 4: The specifications support CSI measurement reporting activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Observation 5: Because the performance of the AI/ML BM configuration was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Observation 6: If the content of the applicable configuration report is small, then implicit reporting on non-applicable configurations through omission would be the most straight-forward approach because the overall report size would be small, and the extra field for non-applicability would not be required.

Observation 7: The gNB configures CSI-RS transmissions and is, thus, the best entity for determining CSI-RS data collection configurations.

Observation 8: There is no specification impact for the use of NW-based system-level performance monitoring to enable BM configuration performance monitoring.

Observation 9: Based on discussions on candidates for KPI calculation for AI/ML BM configuration performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated BM configuration are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 10: Based on discussions on candidates for KPI calculation for AI/ML BM configuration performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the configuration outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the configuration outputs.

And the following proposals:

Proposal 1: In the beam management use case, an AI/ML configuration is signalled by CSI-ReportConfig.

Proposal 2: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement AI/ML BM reporting activation, deactivation, switching, and fallback.

Proposal 3: Discuss signalling to enable fallback without deactivating an AI/ML BM configuration such that performance monitoring of an AI/ML BM configuration can continue during fallback.

Proposal 4: For AI/ML BM applicable configuration reporting, the CSI-ReportConfigId is signalled in the applicable configuration report.

Proposal 5: Further discuss content for reporting applicability of provided configurations and the specific reasons for applicability indication such as changes of prediction accuracy level. FFS on possible indication in the assistance information message of the reasons, e.g. accuracy information alongside applicability indication.

Proposal 6: Support the use of RRCReconfigurationComplete to signal the initial applicable configuration report following a configuration/reconfiguration.

Proposal 7: Assuming that an applicable configuration report is small, non-applicable configurations will be initially reported, through UAI or RRCReconfigurationComplete and updated through UAI by their omission, through UAI or RRCReconfigurationComplete. FFS how to handle the case when no configurations are applicable during Step 3.

Proposal 8: RAN2 to discuss whether applicable configurations are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable configuration reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

Proposal 9: The gNB is the entity which configures the UE for data collection for UE-side AI/ML models for beam management.

Proposal 10: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

Proposal 11: RAN2 only to support performance monitoring of an activated AI/ML BM configuration through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both BM-Case1 and BM-Case2.

Proposal 12: RAN2 to discuss whether applicable configuration reporting to a target cell is supported during handover.

# 4	References

[1] R2-2404102, “Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China”, ETSI MCC, April 2024

[2] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 3GPP, 2023-12

[3] R1-2409985, “AI/ML for Beam Management”, Nokia, November 2024

[4] R1-2407554, “FL summary #5 for AI/ML in beam management, 3GGP TSG RAN WG1 #118 – August 19th – August 23rd”, August 2024

[5] R2-2406381– Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2.