3GPP TSG RAN WG2 Meeting #128	 R2-2410709

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.4

Source: 	ITL

Title: 	Discussion on Conditional LTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to conditional LTM for intra-CU are as follows:

Agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, we discussed the conditional LTM for Intra-CU LTM.

Discussion

For Conditional LTM, the network can configure conditions for candidate cell(s). If the UE determines that any of these conditions are met, it initiates an LTM cell switch to the corresponding candidate cell.

In Conditional LTM, since the UE triggers an LTM Cell switch without an LTM Cell switch command MAC CE, the UE needs to receive or derive the information that would typically be included in the LTM Cell switch command MAC CE before performing the C-LTM.

Therefore, we consider it a key point of discussion to determine what information the UE needs to derive or receive, as well as how it should do so.

Figure 1. LTM Cell switch command MAC CE

In Figure 1, the LTM Cell Switch Command MAC CE contains information that enables the UE to perform either RACH-less or RACH-based LTM:

- Target Config ID: Indicates the candidate cell ID.

- Timing Advance Command: Specifies whether the timing advance (TA) is valid. Based on this, the UE determines whether to perform RACH-based or RACH-less LTM.

- TCI State ID: Indicates and activates the TCI state for the LTM target cell.

- Random Access Related Information (preamble index, SSB index, PRACH mask index, C, S/U, repetition number): Provides RACH-related information for CBRA and CFRA procedures.

On the below, we show our views on some aspects of the information that should be used and how it should be received by the UE to support both RACH-less and RACH-based Conditional LTM.

Timing advance command (TAC)

In R18 LTM, if the Timing Advance Command (TAC) indicated in the LTM cell switch command MAC CE is valid (i.e., not FFF), the UE can perform RACH-less LTM. Therefore, we think that the UE should obtain TAC information for candidate cell(s) to determine whether to proceed with RACH-less or RACH-based conditional LTM.

In RAN2#127bis, PDCCH-ordered early TA acquisition was agreed to support conditional LTM. However, the method for the UE to receive the TAC of candidate cell(s) has not yet been discussed.

To acquire the TAC of candidate cell(s) before performing conditional LTM, two options can be considered as follows:

- Option 1: Use RAR (Random access response)

- Option 2: Introduce a candidate cell TAC MAC CE

For Option 1, early synchronization via RAR was discussed in R18 LTM, as well as for inter-CU LTM. At that time, RAR-based TA acquisition for candidate cell(s) was not introduced, as it could interrupt data reception and transmission. However, in conditional LTM, since there is no LTM Cell switch command MAC CE, RAR-based acquisition could be considered as a possible method to obtain the TA of candidate cell(s).

For Option 2, since the TA of candidate cell(s) belonging to other gNB-DU(s) are transferred to source gNB-DU via F1 interface, source gNB-DU can transmit the TACs of candidate cell(s) to the UE via MAC CE.

We think that Option 2 is more straightforward, as it does not impact system performance.

Proposal 1: Candidate cell TAC indication MAC CE is introduced for conditional intra-CU LTM

RACH-less Conditional LTM

In RAN2#127bis, it was agreed to support Conditional LTM using both RACH-based and RACH-less methods. For RACH-less Conditional LTM, the CG-based option was agreed, but the DG-based option is still remaining FFS.

In Rel-18/19 RACH-less Intra/Inter-CU LTM, DG-based can be supported since network transmit the LTM Cell switch command MAC CE to the UE to initiates LTM Cell switch. In other words, since the source cell can notify the target cell that the UE has initiated an LTM, the target cell could allocate DG via PDCCH.

- CG-based RACH-less LTM: The first uplink transmission occurs on a configured grant that is configured before performing LTM.

- DG-based RACH-less LTM: The first uplink transmission occurs on a dynamic grant sent by the target cell

For the DG-based RAHC-less C-LTM, the network cannot be aware of the UE's C-LTM initiation unless the UE explicitly signals the C-LTM trigger to the network. However, indicating the C-LTM trigger leads to signaling overhead and further effort is required to define the procedures for how this indication would be sent to the network. Therefore, we think CG-based option is enough for RACH-less conditional LTM.

Proposal 2: For RACH-less conditional LTM, DG-based first uplink transmission is not supported

In R18 LTM, the TCI state ID indicated in the LTM Cell switch command MAC CE is used to determine the validity of a CG occasion. In other words, if one of the associated SSB(s) in the CG occasion matches the SSB corresponding to the TCI state ID, the UE considers the CG occasion valid and signals the SSB to the PHY.

Therefore, we propose to discuss how UE checks the validity of a CG occasion for CG-based RACH-less conditional LTM. In our understanding, there are several options can be considered as follows:

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the RS that satisfies the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

For Option 1, the network configures the TCI state ID of candidate cell(s) based on the measurement report from the UE. In this case, additional procedures may be required to confirm the validity of the TCI state IDs of the indicated candidate cell(s). Moreover, the UE should perform the reporting of LTM measurement results to the network and the checking of conditional LTM in parallel.

For Option 2, the UE reuses the RS that was evaluated and met the conditions during the triggered conditional LTM event. For example, If the beam satisfied by the conditional LTM event is SSB X, the UE considers the CG occasion associated with SSB X as valid.

For Option 3, similar to CG-SDT, if at least one SSB configured for CG with SS-RSRP above the threshold, the CG occasion is considered as valid.

We think Option 3 has less spec impact and is straightforward, and Option 1 and Option 2 have the advantage of skipping the procedure of checking RSRP.

Proposal 3: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the RS that satisfies the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

Conclusion

In this contribution, we discussed the conditional LTM for Intra-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: Candidate cell TAC indication MAC CE is introduced for conditional intra-CU LTM

Proposal 2: For RACH-less conditional LTM, DG-based first uplink transmission is not supported

Proposal 3: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Indicate early TCI state ID

- Option 2: Re-use the RS that satisfies the triggered conditional LTM event

- Option 3: Check the RSRP of associated SSB(s)

Reference

RAN2#127bis meeting report