3GPP TSG-RAN WG2 Meeting #128	R2-2410717

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.7.4.2

Source:	Samsung

Title:	DSR enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2#127bis meeting, RAN2 made the following agreements for DSR enhancements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this paper, we further discuss the essential aspects on the enhanced DSR.

Discussion

Threshold Configuration

Regarding the issue that whether we should introduce a restriction that triggering threshold should be always larger than the reporting threshold(s), we see no harm to let NW configure them based on its implementation, without such restriction.

In our understanding, NW may be interested in the holistic delay/buffer status, for optimizing the UL scheduling policy, even for non-delay-critical remaining time portion(s) in some cases. Note that it is always desirable to serve UL data in a way that the occurrence of delay-critical data can be avoided as much as possible.

Observation 1: Based on UL scheduling policy, NW may need to configure reporting threshold larger than triggering threshold, to obtain the delay status for the data that is not delay-critical yet, to prevent further occurrence of delay-critical data.

Moreover, if without the restriction on the configuration of triggering and reporting thresholds, one would have a concern that there could be too much spec impact. While, it might not be the case, since if there are any reporting thresholds configured larger than the triggering threshold, they only affect the enhanced DSR reporting, without introducing any impacts to the triggering and cancellation, as we already agreed that triggering is based on triggering threshold, and the definition of delay-critical data is reused, which is used in the cancellation condition. That is, the spec change is mainly expected in buffer size and remaining time determination for each remaining time portion when composing the enhanced DSR MAC CE. But such spec change is always needed, regardless of whether we introduce the restriction on the configuration of triggering and reporting thresholds or not.

Observation 2: There is no considerable difference in spec change whether we introduce the restriction on the threshold configurations, i.e., triggering threshold is always larger than the reporting threshold(s), or not.

Proposal 1. RAN2 to not introduce the restriction on the threshold configurations, i.e., the triggering threshold is always larger than the reporting threshold(s) for an LCG, for enhanced DSR.

MAC CE Format

Compared to the legacy DSR MAC CE, the most important extension for enhanced DSR MAC CE is to introduce the extension bit/field, since there could be more than one, varying number of such remaining time and buffer size pairs per LCG, due to the absence of corresponding data for some remaining time portion(s).

As illustrated in Fig. 1, the reserved bit in legacy DSR MAC CE is replaced by Extension (E) bit, which indicates whether further remaining time and buffer size pair follows for the same LCG or not. Note that we do not need to care about how to individually indicate which remaining time portion the presented information pair is associated to, since the reported remaining time implicitly implies the remaining time portion it belongs to.

Fig. 1. Proposed enhanced DSR MAC CE format.

Proposal 2. RAN2 to consider the MAC CE format in Fig. 1 as baseline of enhanced DSR, where Extension (E) bit indicates whether further pair of remaining time and buffer size is present for the associated LCG or not.

DSR Format Selection

It is clear that if there is at least one LCG with more than one remaining time portion to be reported, the enhanced DSR MAC CE should be selected.

The case for further discussion is whether to use legacy DSR or enhanced DSR MAC CE, when all the LCGs configured with multiple thresholds and with delay information to be reported, have one remaining time portion to be reported, respectively.

We think whether to use the legacy DSR or enhanced DSR MAC CE for the above case should be studied after the detailed MAC CE format for the enhanced DSR is confirmed, since before that we are not sure about whether extra signalling overhead is encountered when using enhanced DSR, compared to the legacy DSR. So, the discussion for this issue can be postponed until the MAC CE format for the enhanced DSR is determined.

Proposal 3. RAN2 to postpone the discussion on DSR format selection, until the MAC CE format of enhanced DSR is determined.

Non-Delay-Critical Data Reporting

An issue of whether non-delay-critical data should be included in the delay-critical data volume calculation, especially, for the case that the non-delay-critical data is ahead of delay-critical data (in the same or different LCH) from the scheduling/buffering order point of view. But we think such issue can be resolved by UE implementation without any spec change, e.g., reordering of the data within the buffer.

Observation 3: The issue of non-delay-critical data ahead of delay-critical data can be avoided by UE implementation without spec change.

On the other hand, if we introduce such spec change, it can be abused, such that some UEs can deliberately put all the non-delay-critical data prior to the delay-critical data, and thus reporting the whole buffer size as delay-critical data, causing fairness issue between different UEs with different implementations.

Observation 4: If non-delay-critical data ahead of delay-critical data is allowed to be included in buffer size calculation reported by enhanced DSR, such feature can be abused by UE implementation, and thus causing fairness issue between different UEs with different implementations.

Proposal 4. RAN2 to not consider the scheduling/buffering order of the data when calculating the buffer size reported by enhanced DSR.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: Based on UL scheduling policy, NW may need to configure reporting threshold larger than triggering threshold, to obtain the delay status for the data that is not delay-critical yet, to prevent further occurrence of delay-critical data.

Observation 2: There is no considerable difference in spec change whether we introduce the restriction on the threshold configurations, i.e., triggering threshold is always larger than the reporting threshold(s), or not.

Proposal 1. RAN2 to not introduce the restriction on the threshold configurations, i.e., the triggering threshold is always larger than the reporting threshold(s) for an LCG, for enhanced DSR.

Proposal 2. RAN2 to consider the MAC CE format in Fig. 1 as baseline of enhanced DSR, where Extension (E) bit indicates whether further pair of remaining time and buffer size is present for the associated LCG or not.

Proposal 3. RAN2 to postpone the discussion on DSR format selection, until the MAC CE format of enhanced DSR is determined.

Observation 3: The issue of non-delay-critical data ahead of delay-critical data can be avoided by UE implementation without spec change.

Observation 4: If non-delay-critical data ahead of delay-critical data is allowed to be included in buffer size calculation reported by enhanced DSR, such feature can be abused by UE implementation, and thus causing fairness issue between different UEs with different implementations.

Proposal 4. RAN2 to not consider the scheduling/buffering order of the data when calculating the buffer size reported by enhanced DSR.