3GPP TSG-RAN WG2 Meeting #128							R2-2410739

Orlando, USA, Nov.18th– 22nd, 2024

Agenda Item:	8.7.6

Source:	China Telecom

Title:	Discussion on Rate Control for XR

Document for:	Discussion

1	Introduction

During the RAN#105 meeting, a new objective was introduced in XR for NR Phase 3 WID to specify MAC layer XR rate control for uplink congestion adaption.

-	Specify uplink congestion signalling [RAN2]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#127bis meeting, rate control was discussed and the following agreements were made:

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

- RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications:

- Extend the Bit Rate field

- Define a new bit rate table to provide sufficient granularity for XR traffic

- Introduce new values for the bitRateMultiplier

In this contribution, we focus on open issues on XR rate control and provide our views.

2	Discussion

2.1 	DRB-level vs QoS Flow-level Granularity

In order to support codec adaptation for MMTEL voice or MMTEL video, RAN-assisted codec adaptation mechanism was introduced in Rel-15 to support the uplink/downlink bit rate increase or decrease. With the recommended bit rate MAC CE received, UE may initiate an end-to-end bit rate adaptation with its peer. In addition, UE may also send a query MAC CE to its local gNB. The existing recommended bit rate MAC CE is associated with the DRB/LCH level.

The XR services may comprise multiple different traffic flows. These traffic flows may be mapped to one DRB or multiple DRBs based on local RRM strategy. In the current protocol architecture, QoS flow information as high layer information cannot be perceived by MAC layer. From this point of view, at least the legacy DRB-level granularity recommended bit rate can be supported for XR services. Furthermore, the enhancement on Recommended Bit Rate MAC CE is needed as the mixed traffic flows have different recommended bit rates. As for how to support per QoS flow rate control in the MAC layer of gNB is an implementation issue as the gNB-CU could signal QoS flow level parameters to DU via F1AP message. From the perspective of UE, we understand per QoS flow rate control in uplink direction is a new UE capability. Therefore, both per-DRB granularity and per-QoS flow granularity recommended bit rate can be supported for XR service. And the network can configure whether per-QoS flow granularity recommended bit rate is applied via RRC message.

Proposal 1: both per-DRB granularity and per-QoS flow granularity recommended bit rate can be supported for XR service. And the network can configure whether per-QoS flow granularity recommended bit rate is applied via RRC message.

Proposal 2: per QoS flow rate control in uplink direction is a new UE capability.

As illustrated in Figure 1, the current Recommended Bit Rate MAC CE has a fixed size and consists of two octets. The Bit Rate field is 6-bit, comprising 56 utilized code points to indicate values from 0 to 8000 kbps, and X Bit Rate multiplier is 1-bit. In addition, in order to support per-QoS flow granularity recommended bit rate, a new MAC CE should be specified in Rel-19.

Figure 1 Recommended bit rate MAC CE

Proposal 3: a new MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

3 	Conclusion

Based on the above analysis, we have the following proposals:

Proposal 1: both per-DRB granularity and per-QoS flow granularity recommended bit rate can be supported for XR service. And the network can configure whether per-QoS flow granularity recommended bit rate is applied via RRC message.

Proposal 2: per QoS flow rate control in uplink direction is a new UE capability.

Proposal 3: a new MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

4	References

[1]	TR 38.838, Study on XR (Extended Reality) evaluations for NR, V17.0.0, 2022-04