3GPP TSG-RAN WG2 Meeting #128	R2-2410850

Orlando, Florida, USA

Agenda item:	8.7.4.1

Source:	Samsung

Title:	LCP enhancements for Rel-19 XR – outstanding issues

Document for:	Discussion & Decision

# Introduction

In the last RAN2#127-bis meeting, RAN2 made the following agreements for LCP enhancements for Rel-19 XR:

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

And additionally, on the topic of terminology, the following was agreed:

For now, we will use “LCH priority-adjusted data”

In this paper, we discuss the key remaining issues of the operations related with the additional LCH priority.

# Discussion

## Using LCH priority-adjusted data info in LCP Enhancements

One of the issues discussed in RAN2#127-bis was whether the additional LCH priority is applied during the entire resource allocation procedure of LCP, i.e., both 1st and 2nd Rounds, or part of it, where 1st and 2nd Rounds are highlighted below in yellow and green, respectively.

TS 38.321 The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

Regarding this issue, the key point is whether to allow the UE to check the availability of LCH priority-adjusted data per LCH when starting 1st Round of the LCP, or both 1st and 2nd Rounds, respectively, e.g., how to handle the case when all the LCH priority-adjusted data is exhausted by 1st Round, and no residual LCH priority-adjusted data remains when starting 2nd Round.

From the UE complexity point of view, it is more “light-weight” to prioritize an LCH during the whole resource allocation procedure in the same manner, as long as it has LCH priority-adjusted data when starting the LCP.

From the fairness and system capacity points of view, it may be more beneficial to prioritize an LCH (in 1st or 2nd Round) only when LCH priority-adjusted data is available for transmission, i.e., if the LCH priority-adjusted data of an LCH is exhausted in 1st Round, the LCH would not be prioritized in 2nd Round.

Accordingly, two possible options were discussed at RAN2#127-bis:

- Option 1: additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has LCH priority-adjusted data available for transmission when starting the 1st Round.

- Option 2: additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has LCH priority-adjusted data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual LCH priority-adjusted data available for transmission when starting 2nd Round.

And the following was agreed (as already quoted in the introductory section):

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

So the three main remaining options for RAN2 are: A) to adopt Option 2 instead of the agreed baseline (Option 1), B) to stick with Option 1, and C) to leave the decision on which LCH priority to use in 2nd round of LCP to UE implementation. In 2nd round, all channels (regardless of value of Bj) are allocated resources but in strictly decreasing priority order, and we had already agreed not to change the LCP procedure itself. The FFS above therefore refers to whether this priority in second round is same as first round, or subject to another check (whether there is any LCH priority-adjusted data left in a given LCH after 1st round).

Leaving it to implementation may be the simplest way to achieve agreement; however, it may go against one of the key goals of LCP, which is to provide a uniform way across different vendors of avoiding starvation. Therefore our preference is to specify either Option 1 (existing baseline) or Option 2, rather than leave the choice to implementation. Option 1 is simpler, since Option 2 would mean re-checking whether LCH still has LCH priority-adjusted data. In our understanding, Option 2 would not however add any significant processing time or complexity. Option 2 is more accurate (a better match of resources to criticality of data), and possibly fairer; Option 1 is simpler while still ensuring comparable quality across vendors (compared to implementation-based solutions i.e. leaving choice between Option 1 and Option 2 to implementation). We therefore propose to stick with the baseline:

Proposal 1. RAN2 to confirm the baseline procedure as adopted solution for Rel-19 XR i.e. priority of a LCH shall not change between 1st and 2nd round of the LCP procedure.

## Additional LCH Priority in Other Procedures

In general, LCH priority is used in 1) resource allocation in LCP, 2) BSR triggering, 3) truncated BSR LCG selection, and 4) intra-UE prioritization.

For BSR triggering and truncated BSR LCG selection, it is not essential to consider the additional LCH priority of the LCH with LCH priority-adjusted data, considering that DSR can handle the LCH priority-adjusted data.

According to 38.300, “in case a configured uplink grant transmissions or a dynamically allocated uplink transmission overlaps in time with a scheduling request transmission, the UE prioritizes the transmission based on the comparison between the priority of the logical channel which triggered the scheduling request and the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDU associated with the overlapping resource”.

Therefore, in this legacy intra-UE prioritization, when more than one UL grant(s) and/or SR(s) overlap in time domain, a priority-based comparison among the overlapped UL grant(s) and/or SR(s) is conducted to determine which UL grant or SR should be transmitted. The priority-based comparison is based on legacy (default) LCH priority, i.e., the priority of UL grant is defined as the highest (legacy) priority of LCHs multiplexed in the corresponding MAC PDU, and the priority of SR is defined as the (legacy) priority of the LCH triggering the SR.

In RAN2#127 meeting, during the discussion on Rel-19 LCP enhancement, an FFS was added on whether/how the additional LCH priority (originally intended to be used for LCP enhancement) will affect the intra-UE prioritization. The expected operation regarding this issue, would be, in the priority-based comparison elaborated above, when defining the priority of UL grant, the additional LCH priority can be used (instead of legacy priority) for LCH(s) with LCH priority-adjusted data (similar to the case of LCP enhancement). Similarly, the priority of SR can be defined as the (legacy) priority of the LCH if without LCH priority-adjusted data or the additional priority of the LCH if with LCH priority-adjusted data, for the LCH triggering the SR.

For intra-UE prioritization, we can expect similar benefit to that of LCP enhancements (i.e., preventing LCH priority-adjusted data from being delayed by non- LCH priority-adjusted data), via the prioritization of the LCH priority-adjusted data, by making use of the additional LCH priority in determining the priority of an uplink grant, and/or that of LCH triggering an SR.

Observation 1: Prioritization of uplink grant/SR associated with LCH priority-adjusted data in intra-UE prioritization can be used to achieve a goal consistent with that of LCP enhancements, i.e., to prevent LCH priority-adjusted data from being delayed by non- LCH priority-adjusted data.

Proposal 2. RAN2 to consider prioritizing the uplink grant/SR associated with LCH priority-adjusted data in intra-UE prioritization using the additional LCP priority.

## Prioritization of Data over MAC CE

For LCH priority-adjusted data, in-time delivery is important. For this, dedicated configured grant or resource separated by LCP have been introduced over multiple releases of NR. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in LCH priority-adjusted data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs.

Proposal 3. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to confirm the baseline procedure as adopted solution for Rel-19 XR i.e. priority of a LCH shall not change between 1st and 2nd round of the LCP procedure.

Proposal 2. RAN2 to consider prioritizing the uplink grant/SR associated with LCH priority-adjusted data in intra-UE prioritization using the additional LCP priority.

Proposal 3. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.