3GPP TSG-RAN WG2 Meeting #128	R2-2410862

Orlando, FL, USA, November 18th – 22nd, 2024

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

So far, RAN2 has reached several agreements regarding the shape/format of the intended service area as well as some of the related UE behaviours when located inside and outside the intended service area (ISA). However, the exact signalling container of the ISA is still under discussion. This contribution presents our views on the open issues and on supporting PWS in NTN cells.

# 2	Support for MBS broadcast

### 2.1	Signalling container for intended service area (ISA)

In RAN2#127bis, several options were discussed. In our previous paper [2], we proposed a solution without RAN impact based on existing functionality and the Service Announcement. Our contribution showed the advantages of the Service Announcement in terms of signalling efficiency, up-to-date notifications, and energy consumption. Indeed, this legacy mechanism can easily be extended to provide the Intended Service Area in an NTN cell. This approach is beneficial for both the UE and network. On the one hand, it allows the UE to decode MCCH whenever it is strictly needed (e.g., the UE is located inside the intended service area of a certain MBS service) helping to reduce battery consumption. Even for the rare case of changes to the intended service area, the UE can continue with its legacy behaviour and re-acquire the Service Announcement every now and then, up to application implementation. On the other hand, the network can spare unnecessarily updates and overhead in system information broadcast that may also affect non-MBS UEs. Provided that this solution reuses existing functionality, we believe it can ensure timely completion of the Work Item and speed up the deployment in real networks.

- Extending Service Announcement to include the Intended Service Area for NTN cells is a simple solution that has minimal specification impact and fulfils the energy consumption and signalling efficiency criteria.

- MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells.

In our view, using Service announcement is the most efficient approach. Unfortunately, despite its clear technical advantages, the proposal was met with a backslash in RAN2#127bis due to not aligning with the WID objective. RAN2 preferred to continue with a type of signalling that uses RAN2 protocols and agreed that the mapping between an ISA identifier and an MBS broadcast service should be placed in the MCCH. The next open issue is where to place the definition of the ISA. There are two options:

- MCCH: It has the clear advantage of a more self-contained signalling since both ISA definition and the mapping would be included in the same container. However, some companies showed concerns of its potential impact on legacy UEs, the restricted possibilities of MCCH acquisition relaxation, and the fact that it is not aligned with the work item objective.

- SIB: Provided the service area information rarely changes, using existing or a new SIB allows for more efficient MCCH acquisition relaxation. However, using SIB20 raised some concerns about the maximum SIB size, e.g., when there are multiple ISAs included. Even though SIB segmentation appears to be helpful, its use would require to change the legacy SIB20 which currently does not support segmentation. But, these drawbacks are avoided if a new SIB is used.

Based on the above analysis, if RAN2 were to follow the WID explicitly, we would prefer specifying the ISA definition in a new SIB. This approach has an advantage of having minimal impact to legacy UEs, while it enables MCCH acquisition relaxation whenever the UE is not located in any of the defined ISA.

If the ISA is to be sent using AS-protocols, the definition of the intended service area is provided in a new SIB.

### 2.2	ISA and MCCH acquisition relaxation

##### 2.2.1	Signalling

Even though including the ISA definition in SIB provides a simple way to skip MCCH acquisition, we see the potential benefits of optionally including the mapping between the ISA identifiers and MBS broadcast service also in the Service Announcement. Mainly, this method secures that the UE becomes aware of the ISA identifier of the specific MBS broadcast service the user is interested in. Therefore, a UE can relax MCCH acquisition based on the combination of the information in the new SIB and Service announcement. Specifically, including ISA ID in service announcement allows a UE to know whether it is within the ISA(s) of MBS service(s) that it is interested in and only consider those area(s) in MCCH-relaxation. Otherwise, the UE has to consider all the ISAs in the cell. Thus, providing a more granular approach.

- Providing the ISA ID in the Service Announcement can lead to a more granular MCCH acquisition relaxation, benefiting both energy saving and user experience.

Similar to existing functionality such as FSA ID, an entity in the MBS architecture, e.g., the AF or MBSF, can coordinate the assignment of unique ISA IDs to different ISAs and ensure the ISAs, together with ISA IDs, are signalled via AS signalling (e.g., SIBs) while the ISA IDs are included in the NAS signalling (i.e., Service announcement). Knowing that Service Announcement belongs to higher layer protocols, a short summary of its functionality is described in the Annex 1 to clarify the concept and the overall flow.

In conclusion, the provision of the ISA identifier in the Service Announcement provides a more granular MCCH relaxation with minimal specification impact. In other words, the UE can relax MCCH acquisition based on the specific MBS broadcast service (identified with its TMGI), and its associated ISA, it is interested to receive. We foresee that RAN2 needs to send an LS to SA2 to request this minor addition. A draft LS is provided in the Annex 2.

If ISA is sent in SIB, the ISA ID is provided in the MBS Service Announcement to facilitate MCCH acquisition relaxation. RAN2 sends an LS to SA2.

##### 2.2.2	UE behaviour

We understand that a UE interested in MBS must first acquire Service Announcement to become aware of the existing MBS services and their respective characteristics (e.g., start time). This may happen even before attempting to decode the MBS configuration provided in MCCH. In fact, the current RRC specification allows the UE to skip MCCH monitoring based on USD/Service Announcement:

[NOTE in TS 38.331 V18.3.0 - Section 5.9.2.2]

“It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.”

On a related note, Stage-2 specification allows the UE to decide when to start monitoring the MCCH based on the information signalled in the Service announcement:

[NOTE in TS 38.300 V18.2.0 - Section 16.10.6.2]

It is up to UE implementation to use the start and stop times in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

Similar to existing functionality, we consider that the use of the ISA identifier in the Service announcement to relax the acquisition of MCCH should be based on UE implementation.

A UE interested in an MBS broadcast service may skip MCCH acquisition when it is located outside the intended service area where this service is provided.

A UE may use, by implementation, its location, the ISA definition in SIB/USD, and mapping between ISA ID and MBS Service ID (TMGI) in the USD/Service Announcement to decide whether to skip MCCH acquisition.

### 2.3	Service continuity

#### 2.3.1 Service continuity for 5G MBS broadcast

Principles of service continuity for MBS broadcast are specified in TS 38.300 V18.2.0, Section 16.10.6.5. For UEs in RRC_IDLE or RRC_INACTIVE, service continuity is ensured through a combination of the following mechanisms:

- MCCH indicates a list of neighbour cells providing the same MBS broadcast services as provided in the serving cell.

- Information about which frequency is providing which MBS broadcast services via point-to-multipoint (PTM) transmission is obtainable through User Service Description (USD) or through a combination of USD and SIB21. Specifically, SIB21 contains MBS Frequency Selection Area ID (FSAI) which identifies a preconfigured area within, and in proximity to, which the cell(s) announces the FSAI and the associating frequency.

The first mechanism allows a UE to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using point-to-multipoint (PTM) transmission. The second mechanism helps the UE avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies. The UE can figure out the TMGI-Frequency mapping from USD or from the combination of USD and SIB21 and is allowed to make the frequency highest priority in cell reselection. Additionally, a UE in RRC_CONNECTED can send MBS Interest Indication (MII) to the gNB to indicate the UE’s interest in certain MBS broadcast services.

#### 2.3.2	Service continuity for NTN MBS broadcast

In previous RAN2 meetings, there has been an increased interest in enhancing service continuity considering the concept of Intended Service Area, which is specific to NTN. In our view, the introduction of ISA does not entail any necessary changes to legacy mechanisms for service continuity in 5G MBS broadcast, which can also be applied in NTN. That means, the list of neighbour cells providing the same MBS broadcast services (now with the associated ISA) in MCCH, the FSAI in SIB21, and the TMGI-frequency mapping or TMG-SAI mapping in the USD (which is delivered to the UE via Service announcement) can be reused.

- Legacy mechanisms for service continuity due to UE mobility in 5G MBS Broadcast can be reused for MBS broadcast with NTN.

Provided the large size of NTN cells (e.g., 100 km radius), the short time of service (e.g., 5 minutes), and the fact that there are no requirements for the UE to start or release MBS Radio Bearers (MRB) upon entering or leaving the ISA, we consider that RAN2 should focus on scenarios that are more common in an NTN system.

We see the need to address service continuity for another type of mobility in NTN, namely mobility due to satellite movement. Specifically, one aspect where NTN differs from terrestrial networks (TN) is that the time duration that an NGSO satellite serves a quasi-earth-fixed cell is just a few minutes. Each new serving satellite typically implies a new cell, therefore, an MBS UE in RRC_IDLE or RRC_INACTIVE needs to perform cell reselection and potentially acquire MBS related configuration and the list of neighbour cells that providing the same MBS services (carried in MCCH) quite often. This can be a challenge in terms of signalling overhead and service interruption since it is difficult from network perspective to align satellite switches with the MCCH notification period, and there is a risk the UE does not obtain the MCCH update in time, leading to service interruption. Therefore, we suggest RAN2 to discuss this issue and find potential solutions.

- The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

RAN2 focuses on service continuity for the scenario where the quasi-Earth-fixed cells are replaced due to satellite movement.

A potential solution to the foregoing problem is to allow a serving cell to signal a list of upcoming satellites/cells (e.g., indicated via satellite IDs) providing the same MBS services as the serving cell. This information can be complemented with the satellite assistance information provided in other System Information blocks (e.g., SIB19). The signalling may also contain information about the time when an upcoming satellite in the list starts serving the current cell or the intended service area associated with the MBS service and/or information about frequency or PCI. Similar to the legacy solution using SIB21 and FSAI, this small enhancement aims to enable UEs interested in MBS broadcast to know well in advance whether the same services are provided by upcoming satellites, thereby preparing the UEs to either continue using the same MBS configuration or request a unicast delivery of the same service in the new cells/satellites to ensure service continuity.

In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and services of the upcoming satellite that will serve the area.

### 2.4	Public warning systems in NTN

The scope of the work item to include a service area smaller than a cell in NTN is not limited to MBS but also encompasses other broadcast services such as Public Warning Systems (PWS). The two most prominent PWS are ETWS and CMAS. A flexible geo-fencing mechanism is already supported in CMAS, which can be repurposed for this NTN enhancement without necessitating further specification changes. During RAN2#127, the feasibility of applying this solution to ETWS was discussed. RAN2 concluded to send an LS to SA1 to inquire about the requirements and clarify whether such capability can be extended to ETWS.

Although the LS response from SA1 is not expected before RAN2#129, two recent developments suggest that further progress can be made within RAN2. Firstly, a new study item [3] was approved in SA#103 to further develop service requirements for emergency and mission-critical services over NTN. Secondly, a new work item revision [4] was approved in RAN#105 that includes the support of PWS for NB-IoT NTN and the potential extension of the intended service area concept. We believe these developments indicate a consensus that a geo-fencing solution should be adopted in ETWS to address the specific needs of NTN. RAN2 can begin confirming this understanding, while the detailed solution can await the reception of the LS response.

Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

# 3	Text Proposals

## 3.1	TP to TS 38.300 v18.3.0

#### 16.10.6.2	Configuration

MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

The following principles govern the MCCH structure:

-	MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

-	MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; a notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

NOTE:	It is up to UE implementation to use the start and stop times and the ID of the intended service area in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

-	When the UE receives an MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

## 3.2	TP to TS 38.331 v18.3.0

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list or the intended service area ID in the USD along with the UE's GNSS position to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Extending Service Announcement to include the Intended Service Area for NTN cells is a simple solution that has minimal specification impact and fulfils the energy consumption and signalling efficiency criteria.

Observation 2	Providing the ISA ID in the Service Announcement can lead to a more granular MCCH acquisition relaxation, benefiting both energy saving and user experience.

Observation 3	Legacy mechanisms for service continuity due to UE mobility in 5G MBS Broadcast can be reused for MBS broadcast with NTN.

Observation 4	The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

Observation 5	The Service Announcement contains the MBS Service Area.

Observation 6	MBS Service Area can be in the form of Cell ID list, TAI list, geographical area information or civic address information.

Based on the discussion in the previous sections we propose the following:

Proposal 1	MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells.

Proposal 2	If the ISA is to be sent using AS-protocols, the definition of the intended service area is provided in a new SIB.

Proposal 3	If ISA is sent in SIB, the ISA ID is provided in the MBS Service Announcement to facilitate MCCH acquisition relaxation. RAN2 sends an LS to SA2.

Proposal 4	A UE interested in an MBS broadcast service may skip MCCH acquisition when it is located outside the intended service area where this service is provided.

Proposal 5	A UE may use, by implementation, its location, the ISA definition in SIB, and mapping between ISA ID and MBS Service ID (TMGI) in the USD/Service Announcement to decide whether to skip MCCH acquisition.

Proposal 6	RAN2 focuses on service continuity for the scenario where the quasi-Earth-fixed cells are replaced due to satellite movement.

Proposal 7	In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and services of the upcoming satellite that will serve the area.

Proposal 8	Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

R2-2409184, Support for broadcast services in NR NTN, Ericsson, RAN2#127bis, Hefei, China, October 2024.

SP-240495, New SID: Study on satellite access - Phase 4, SA#103, Mar. 2024

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.

# 6	Annex

## 6.1	Service Announcement background

The Service Announcement provides essential information that enables users to discover available multicast and broadcast services. This includes identifying what services are available (identified with their TMGI), the type of content being offered, and how to access these services. For instance, it may include the MBS Service Area, in the format of Cell ID list, TAI list, geographical area information or civic address information, or the MBS FSAI and optional frequency information.

- The Service Announcement contains the MBS Service Area.

- MBS Service Area can be in the form of Cell ID list, TAI list, geographical area information or civic address information.

According to TS 23.247, which defines architectural aspects of MBS, to receive the data of broadcast communication service, the UE is either:

- Preconfigured with the needed configuration for the UE to receive MBS service (in this case the UE does not need interact with NW).

- Provisioned with the configuration of a Broadcast MBS session on the application level via the Service Announcement.

The Service Announcement may be provided to a UE by AF (Application Function) or MBS function (MBSF) or may be retrieved by the UE from those entities. The Service Announcement may be delivered to the UE via application specific means or as defined in TS 26.502, using one of the following methods:

  - Pre-configured at the UE side;

  - Via an MBS Session; or

  - Via a regular PDU Session.

Figure 1 illustrates different phases of broadcast data provisioning and Figure 2 depicts a more detailed signalling flow for MBS broadcast session establishment. It can be seen from Figure 2 that the Service announcement is delivered to the UE before the MCCH. In our understanding, the UE becomes first acquainted with the available MBS services (e.g., content, identifiers, start and stop times) through the Service Announcement. Once it has this information, based on user input, the UE may become interested in an MBS service and trigger RAN procedures such as MCCH acquisition or MRB establishment.

Figure 1: [Figure 4.2.2-1, TS 23.247 V18.6.0] Phases of Broadcast data provisioning

Figure 2: Broadcast session establishment flow (adapted from Figure 7.3.1-1 TS 23.247 V18.6.0)

## 6.2	Draft LS to SA2

3GPP TSG-RAN WG2 Meeting #128	draft-R2-XXXXXX

Orlando, FL, USA, November 18th – 22nd, 2024

Title:	[draft LS] Intended Service area for MBS NTN

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN2

To:	SA2

Cc: 	-

# Contact Person:

# Name:	Ignacio Pascual

E-mail Address:	 Ignacio.pascual.pelayo@ericsson.com

# Attachments:	None

# 1. Overall Description:

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” whose geographical definition is provided to the UE via system information broadcast and is associated with an identifier.

For the purpose of MCCH acquisition relaxation, RAN2 is considering the inclusion of the “intended service area” identifier in the Service Announcement. RAN2 understands that there may be an impact in SA2 specifications and would kindly request SA2 to consider the necessary updates.

# 2. Actions:

# To SA WG 1:

# RAN2 kindly request SA2 to take into consideration and update the corresponding specifications, if needed.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129		17th Feb. – 21st Feb. 2025			Athens, Greece

TSG RAN WG2 Meeting #129bis		7th April – 11th April. 2025				TBC, China