3GPP TSG-RAN WG2 Meeting #128	R2-2410878

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	7.4.2

Source: 	Sequans Communications

Title: 	Remaining open issues of satellite switch with resync

Document for:	Discussion and Decision

# Introduction

During the last meeting, the following was discussed and agreed:

R2-2409187	Remaining open issues	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	UEs configured with PDD report may (simultaneously) generate and send a PDD report upon satellite switch with resynchronization.

-	Sequans does not necessarily agree with Obs 1 and thinks the maximum PDD is 0.3 ms in the extreme case (of 100km cell size). Sequans thinks we might still need to discuss whether there is a need for the NW to update the SMTC, e.g. due to the feeder link, but still wonders if this is a real issue

-	LG also thinks there is no issue, smtc4 provides enough duration to perform neighbour cell measurement after completion of soft satellite switch.

Can come back in the next meeting

Proposal 2	RAN2 to revisit the impact on neighbour cell SMTCs upon satellite switch with resync.

In this contribution, we discuss further this topic.

# Discussion

## Hard satellite switch with resync

Service link PDD report

In connected, SMTCs are configured/adjusted by the NW, based on UE assistance information: service link propagation delay difference(s) between serving the cell and neighbour cell(s), but also initial coarse location information.

At the time of the satellite switch with resync, only serving satellite is changing. Neighbouring satellite serving neighbour cells would not change at the same time. So, the neighbour cell service link propagation delay is not impacted. Only the serving cell propagation delay may change.

In general, the SL PDD change would be related to the position of the UE in the QEF cell, and to the size of the QEF cell. The worst case is when the QEF cell is right below the satellite orbit (we neglect earth rotation compared to satellite speed). Then the max SL PDD jumps occurs when UEs are located at the extremities of the QEF cell, parallel to the trajectory of the satellite.

In 38.821 TR, the typical beam footprint considered is 100-1000km diameter. We assume typical cell sizes should be on the small side. Increasing the size of the cell requires more satellites to ensure a given elevation for all UEs in the cell. In the following we consider a cell diameter of 200km.

We consider the LEO 600 / Elevation >= 30° scenario. The worst SL PDD jump is around 0.5ms.

Even in scenarios where the worst SL PDD jump would be more significant (larger cells), this only occurs for UEs located at the extremities of the cell in the direction of the satellite movement. UEs located halfway always see a null SL PDD jump. So overall this does not seem to be an issue.

Observation 1: Service link PDD simultaneous reports at satellite switch seems avoidable by NW implementation

## Soft satellite switch with resync

The issue discussed earlier in ‎[1] and ‎[2] is different. It is related to the fact that current implementation of “Soft satellite switch with resync” eventually ends up in changing the SSB timing before/after the switch. That can be realized in different ways by the NW (changing the cell timing, changing the SSB location only, or a combination of both). But in the end, all those ways will have an impact on SMTCs, e.g. requiring reconfigurations at the time of the switch. This is expected as SMTC configuration is aligned on SSB occurrences.

For hard switch, there is no issue as the SSB timing does not change:

Figure 1 – Hard-switch (SSB half radio frames at gNB)

In our view, similarly, the SSB timing should not change before/after the soft switch. That is to say, the SSB offset should be applied only during the soft switch duration:

Figure 2 – Soft switch without SSB timing change

We encourage companies to revisit this issue.

Proposal 1: Consider soft switch where SSB-timeOffset is applied only between t-serviceStart and t-service

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Service link PDD simultaneous reports at satellite switch seems avoidable by NW implementation

Proposal 1: Consider soft switch where SSB-timeOffset is applied only between t-serviceStart and t-service

# References

- R2-2405680, “Soft satellite switch SSB time offset and SMTC impact”,	Sequans Communications

- R2-2405757, “Summary of [AT116][301][NR NTN Enh] SMTC impacts for soft satellite switch”, Sequans Communications