3GPP TSG RAN WG2#129bis	R2-2501716

Wuhan, China, 7th - 11th April 2025

3GPP TSG-RAN1 Meeting #120	R1-2501593

Athens, Greece, 17th – 21st February 2025

Title:	LS on Slot aggregation configuration with multi-PUSCH

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

Name:	Karri Ranta-aho

E-mail Address:	Karri.Ranta-aho@Nokia.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 specified a Multi-PUSCH feature in Release 16, and restricted the simultaneous configuration of Multi-PUSCH and PUSCH aggregation with the following statement in TS38.214 subclause 6.1.2.1:

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

RAN2 duplicated the configuration restriction in the field description of pusch-TimeDomainAllcationListForMultiPUSCH:

pusch-TimeDomainAllocationListForMultiPUSCH 
Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17.

In Rel-17 RAN1 specified that the UE can be configured with pusch-AggregationFactor simultaneously with pusch-TimeDomainAllocationListForMultiPUSCH-R17 when extendedK2 is also present in the configuration. However, in the end RAN2 did not introduce the new -r17 variant of the pusch-TimeDomainAllocationListForMultiPUSH to the RRC specification, but the Rel-16 parameter is used also in the Rel-17 configuration. Unfortunately this, if taken literally, prevents the Rel-17 configuration where pusch-AggregationFactor should be configurable simultaneously with pusch-TimeDomainAllocationListForMultiPUSCH-R17 when extendedK2 is configured.

RAN1#120 clarified that the Rel-16 restriction only applies if extendedK2 is not configured to differentiate the Rel-16 and Rel-17 behaviours.:

Agreement Fix the specification to enable the operation according to the RAN1#107-e agreement by clarifying that the pusch-AggregationFactor is not configured with pusch-TimeDomainAllocationListForMultiPUSCH (i.e. Rel-16 behavior), if extendedK2 (i.e. Rel-17 operation) is not configured. This would need to be captured in the first statement in TS 38.214: If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH but not configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor. And propagate the same change also to the TS 38.331 RRC parameter field description: pusch-TimeDomainAllocationListForMultiPUSCH Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor if extendedK2 is not configured. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17. Send an LS to RAN2 to lift the restriction on RRC specification.

2. Actions:

To RAN WG2

ACTION: 	RAN1 kindly asks RAN2 to make the simultaneous configuration restriction of pusch-AggregationFactor and pusch-TimeDomainResourceAllocationListForMultiPUSCH only applicable if the UE is not configured with extendedK2.

3. Date of Next TSG-RAN1 Meetings:

TSG RAN1 Meeting #120bis	7 - 11 April 2025	Wuhan, China

TSG RAN1 Meeting #121	19 - 23 May 2025	St. Julian, Malta