3GPP TSG-RAN WG2 Meeting #129bis	R2-2501772

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.0.2.1

Source:	vivo, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	Discussion on RACH-less HO Use Case Capturing in Stage-2

Document for:	Discussion and Decision

- Introduction

In the RAN2#125 meeting, it was agreed to generalize RACH-less HO supporting in Rel-18 based on the following way forward [1]. As a result, in the latest Stage-2 specification, a UE can be configured with RACH-less HO in case of intra-gNB HO procedure, applicable to both TN and NTN.

Potential way forward to be discussed offline: - Generalize existing rachLessHandoverNTN-r18 to rachLessHandover-r18 that may be supported by any release 18 UE (not just for NTN and IAB). - Restrict this to intra-gNB case - agree to the changes in TS 38.300 and TS 38.331 change requests for those listed above to support RACH-less HO with TA as source or TA equal to 0. - Early TA acquisition is not part of this discussion for now. Agreement: - We will generalize RACH-less HO without impact to RAN3 in Rel-18

38.300 sub-clasue 9.2.3.6 RACH-less handover During intra-gNB HO procedure, RACH-less handover can be configured for a UE. The RACH-less handover procedure applies the following functionality: - The UE uses the same timing advance value (i.e., NTA value) at the target cell as in the source cell or timing advance value (i.e., NTA value) of 0. - The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell. - The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell. - The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

At the same time, in mIAB session, a consensus was achieved that RACH-less HO can also be used during the mobile IAB-DU migration procedure for a UE that is migrated from the source logical mobile IAB-DU to the target logical mobile IAB-DU, without any impact to RAN3 (details in R2-R2-2306817, R2-2400035, R2-2401937). Later than that, in the RAN3#127 meeting, it was agreed that there is no issue with supporting inter-gNB RACH-less HO in NTN. That is supporting inter-gNB RACH-less HO in NTN will not bring any impact to RAN3 (details in R2-2501727).

Without a doubt, the latest Stage-2 specification needs to be updated to cover all the agreed RACH-less HO use cases. In this contribution, we would like to propose 2 candidates for specification capturing based on companies' views collected in a pre-meeting offline discussion.

- Discussion

During the pre-meeting offline discussion on how to capture the RACH-less HO use cases in Stage-2, companies are acceptable to add the inter-gNB HO procedure in NTN as one of the RACH-less HO use cases and further clarify that RACH-less HO can be configured for a UE in case of intra-gNb HO procedure in TN and NTN. The corresponding text proposal example is given below,

38.300 sub-clause 9.2.3.6 RACH-less handover A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN.

Some companies are holding the view that the new text proposal excludes the case that RACH-less HO can be used for mobile IAB-DU migration procedure. This is because those companies think that the IAB-DU migration procedure belongs to the inter-gNB HO case. In other words, the UE is handed over from the cell(s) of the source logical mobile IAB-DU that have F1 set up with the source F1-terminating IAB-donor-CU to the cell(s) of the target logical mobile IAB-DU that have F1 set up with the target F1-terminating IAB-donor-CU, which can be regarded as a kind of inter-gNB HO case based on the quoted Figure 1 from TS 38.401. On the other side, a few companies think this should be considered as intra-gNB HO case. Views on this are diverging.

Figure 1: mobile IAB-DU migration procedure

Thus, the first question needs to be checked among RAN2 is that whether the mobile IAB-DU migration procedure can be regarded as intra-gNB HO procedure when considering RACH-less HO configuration for a UE. If the answer is yes, then we can understand that the case “intra-gNB HO procedure in TN” covers both intra-gNB HO procedure for normal TN UEs and mobile IAB-DU migration procedure for mIAB UEs. The above-mentioned text proposal is sufficient to capture all the RACH-less HO use cases.

Proposal 1: For configuring RACH-less handover for a UE, RAN2 discusses whether the mobile IAB-DU migration procedure is regarded as intra-gNB HO procedure or inter-gNB HO procedure, or neither.

Further, if mobile IAB-DU migration can not be regarded as intra-gNB HO procedure in TN, based on companies' offline views, some companies suggest discussing in RAN2 whether we should do some clarification additionally for RACH-less HO configuration for mIAB, For example, in subclause 9.2.3.6 of 38.300, Revise the text “During intra-gNB HO procedure, RACH-less HO can be configured for a UE” to “A UE can be configured with RACH-less HO in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN, or mobile IAB-DU migration procedure”. Meanwhile, some companies think a separate correction for mIAB is not needed since the RACH-less HO configuration for mIAB is already captured in subclause 4.7.5.1 of the current Stage-2 spec. We only need to take care of the case of inter-gNB HO in NTN.

4.7.5.1 General <the unchanged part is omitted> RACH-less handover as specified in 9.2.3.6, in TS 38.321 [6] and in TS 38.331 [12] is supported in mobile IAB. RACH-less handover can also be used during the mobile IAB-DU migration procedure for a UE that is migrated from the source logical mobile IAB-DU to the target logical mobile IAB-DU.

Unfortunately, no consensus was made regarding the minimal acceptable text in the pre-meeting offline discussion. Consequently, the following proposal is made for online discussion and decision,

Proposal 2: RAN2 down-selects the following options as the text proposal for sub-clause 9.2.3.6 of 38.300 if it is confirmed that mobile IAB-DU migration procedure is not regarded as intra-gNB HO procedure:

Opt 1: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN, or mobile IAB-DU migration procedure;

Opt 2: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN.

The corresponding text proposals for Opt 1 and Opt 2 are given in Annex A and Annex B, respectively, for your reference. Meanwhile, you may refer to CR R2-2502605 for more details about Opt 2.

3 Conclusion

In this contribution, we list the concerns and potential solutions raised during the pre-meeting offline discussion on RACH-less HO use case capturing in Stage-2. All the proposals in this contribution are summarized below,

Proposal 1: For configuring RACH-less handover for a UE, RAN2 discusses whether the mobile IAB-DU migration procedure is regarded as intra-gNB HO procedure or inter-gNB HO procedure, or neither.

Proposal 2: RAN2 down-selects the following options as the text proposal for sub-clause 9.2.3.6 of 38.300 if it is confirmed that mobile IAB-DU migration procedure is not regarded as intra-gNB HO procedure:

Opt 1: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN, or mobile IAB-DU migration procedure;

Opt 2: A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN.

4 Reference

[1] R2-2403994, RAN2#125 Meeting Report, MCC.

Annex A: TP for Opt 1

Start of change

9.2.3.6	RACH-less handover

A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN, or mobile IAB-DU migration procedure. The RACH-less handover procedure applies the following functionality:

-	The UE uses the same timing advance value (i.e., NTA value) at the target cell as in the source cell or timing advance value (i.e., NTA value) of 0.

-	The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell.

-	The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell.

-	The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

End of change

Annex B: TP for Opt 2

Start of change

9.2.3.6	RACH-less handover

A UE can be configured with RACH-less handover in case of intra-gNB HO procedure in TN and NTN, or inter-gNB HO procedure in NTN. The RACH-less handover procedure applies the following functionality:

-	The UE uses the same timing advance value (i.e., NTA value) at the target cell as in the source cell or timing advance value (i.e., NTA value) of 0.

-	The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell.

-	The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell.

-	The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

End of change