3GPP TSG-RAN WG2 Meeting #129bis                                                           R2-2501777

Wuhan, China, April 7th– 11th, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Further Discussion on S&F Operation

Document for:	Discussion and Decision

- Introduction

In RAN2#129 [1], the support of the S&F operation was discussed and the following agreements were made.

Agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply.

The S&F operation issue was further discussed in RAN2#128 [2], and the following agreements were made.

2. It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

Besides, some companies propose to introduce the S&F cell lists in SIB4 and SIB5 to assist UE measurements and cell reselection [3], this issue was discussed but remains unsolved. Last but not least, the LS [4] from SA2 on Satellite IDs for S&F operation was received.

In the contribution, we will analyse all these issues and give our view.

- Time information from S&F mode to normal mode;

- Time information from normal mode to S&F mode;

- FFS new cause value within RRC Release upon feeder link is unavailable;

- FFS to introduce S&F cell list for cell reselection;

- SA2 response LS.

- Discussion

## 2.1 Time information from S&F mode to normal mode

In RAN2#129, RAN2 agreed to introduce the time information from S&F operation mode to normal mode in SIB31. However, whether to introduce a new IE to indicate the time information or reuse the existing t-service is FFS.

The following description extracted from the spec [5] is the meaning of the t-service.

t-Service Time information on when an NTN cell is going to stop serving the area it is currently covering, as specified in TS 36.304 [4]. This field applies for service link switches in NTN quasi-Earth fixed cells and feeder link switches for both NTN quasi-Earth fixed and earth-moving cells.

According to the spec, when it refers to the service link switch, it only applies to the EFC scenario. Besides, t-service means when the satellite stops serving the area it is currently covering. That is, it is associated with a specific area. As for the time info from S&F mode to normal mode, it means when the satellite will recover its feeder link with the ground MME. From such a perspective, the meaning of the t-service and the time info from S&F mode to normal mode is quite different.

Observation 1: The meaning of the t-service and the time info from S&F mode to normal mode is different, i.e., the t-service means when the satellite stops covering the area, while the time info from S&F mode to normal mode means when the satellite will recover its feeder link with the ground MME.

Moreover, one may argue that there may be some scenarios where two times have the same time point, i.e., the time point when the satellite stops serving the current area and covers the next area is the time when the satellite resumes its feeder link. However, we think such a scenario is quite a corner case, so there is no need to do any specific handling.

Observation 2: The scenario that the time point when the satellite begins to cover the next area is the time when the satellite resumes its feeder link is a coner case.

Proposal 1: Introduce new IE for the time info on the transition from S&F operation mode to normal mode in SIB31.

## 2.2 Time information from normal mode to S&F mode

If the cell is currently working on regeneration payload mode, both the S&F capable UEs and the S&F not capable UEs may work in the cell. Then, when the cell changes its operation mode from regeneration payload to S&F mode, UEs without S&F capability may not work well in the cell. If S&F not capable UE is in RRC CONNECTED, it can rely on NW implementation to handle these UEs, i.e., NW can trigger the handover of the UEs to cells in regeneration payload mode or release the UE. For S&F not capable UE in RRC IDLE, it is beneficial if UEs can camp on other cells before the cell changes to S&F mode. To enable this, we think the simple way is to reuse the new S&F operation indication, i.e., up to NW implementation, NW indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation, then the legacy UEs and R19 UEs not supporting the S&F operation consider the cell is barred and perform the cell reselection, while the S&F capable UE can continue work in the cell.

Observation 2: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 3: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 2: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 3: No additional information except the S&F operation indication needs to be provided by NW.

## 2.3 FFS new cause value within RRC Release upon feeder link is unavailable

If the cell is in normal mode upon UE access to the network, while later the feeder link is not available, i.e., the S&F mode, the network may release the UE to IDLE mode. Once the network gives UE the information that the release is due to the S&F operation, it will be beneficial for UE to perform cell reselection if the UE initiates the non-delay tolerant service.

Observation 4: There are benefits foreseen if NW indicates to UE the release is due to the cell transfer to S&F mode, i.e., UE may not reselect the cell if UE initiates the non-delay tolerant service.

Proposal 4: A new cause value “S&F” is indicated by the network if the RRC Connection release is due to the cell changes from normal mode to S&F mode.

After UE acquires the new cause value within the RRC Connection release message, UE can deliver the information to the NAS layer to assist the NAS layer procedure. Besides, from the AS layer perspective, the corresponding UE behaviour is UE may not reselect the cell, but we think this can just be left to UE implementation to realize.

Proposal 5: After UE acquire the new cause value “S&F”, UE forward the information to the NAS layer.

Proposal 6: How to avoid UE reselecting the S&F cell upon initiating the non-delay tolerant service based on the new cause value is up to UE implementation.

## 2.4 FFS to introduce S&F cell list for cell reselection

According to [2], introducing the S&F cell lists in SIB4 and SIB5 may be beneficial for UE to prioritise the S&F cell upon performing cell reselection. From our perspective, introducing such S&F cell list means the network has to change the SIB4/SIB5 frequently. Besides, there is also unclear motivation on why UE need to prioritise the S&F cell the S&F cell since the normal mode cell can provide better service. Finally, how to prioritise the S&F cell upon cell reselection can also depend on implementation. Based on these, there is no need to introduce such S&F cell lists for cell reselection.

Proposal 7:  No specific enhancement on cell selection/reselection for S&F is needed.

## 2.5 SA2 response LS

The following are the responses to the questions on the usage of the satellite within the satellite monitoring list [4]. It can be observed that there is no restriction that UE has to camp on, attempt to access or communicate with the satellite included in the MME-configured satellite list.

SA2 thanks RAN2 for the LS on Satellite IDs for store-and-forward operation and would like to provide the following feedback. - RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. [SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9. Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s): - the satellite has the UE context but does not necessarily support store-and-forward operation; or - the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)? [SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

Besides, how UE uses the satellites within the MME-configured satellite list is totally up to UE implementation. And such description has already been captured in the agreed running CR [6] on TS 23.401.

- The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with asatellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

Based on SA2 progress as we listed above, since the UE handling of the satellite within the satellite monitoring list provided by MME is up to UE implementation, there is no RAN2 impact.

Proposal 8: The usage of the S&F monitoring list is up to UE implementation, and no RAN2 impact is forseen.

- Conclusion

In this contribution, we have further discussed the support of the S&F operation. And we make the following observations and proposals:

Time information from S&F mode to normal mode

Observation 1: The meaning of the t-service and the time info from S&F mode to normal mode is different, i.e., the t-service means when the satellite stops covering the area, while the time info from S&F mode to normal mode means when the satellite will recover its feeder link with the ground MME.

Observation 2: The scenario that the time point when the satellite begins to cover the next area is the time when the satellite resumes its feeder link is a coner case.

Proposal 1: Introduce new IE for the time info on the transition from S&F operation mode to normal mode in SIB31.

Time information from normal mode to S&F mode

Observation 2: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 3: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 2: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 3: No additional information except the S&F operation indication needs to be provided by NW.

FFS new cause value within RRC Release upon feeder link is unavailable

Observation 4: There are benefits foreseen if NW indicates to UE the release is due to the cell transfer to S&F mode, i.e., UE may not reselect the cell if UE initiates the non-delay tolerant service.

Proposal 4: A new cause value “S&F” is indicated by the network if the RRC Connection release is due to the cell changes from normal mode to S&F mode.

Proposal 5: After UE acquire the new cause value “S&F”, UE forward the information to the NAS layer.

Proposal 6: How to avoid UE reselecting the S&F cell upon initiating the non-delay tolerant service based on the new cause value is up to UE implementation.

FFS to introduce S&F cell list for cell reselection

Proposal 7:  No specific enhancement on cell selection/reselection for S&F is needed.

SA2 response LS

Proposal 8: The usage of the S&F monitoring list is up to UE implementation, and no RAN2 impact is forseen.

- Reference

- Draft_RAN2_129_Meeting_Report_v1.

- Draft_RAN2_128_Meeting_Report_v1.

- R2-2409585, Discussion on Store and Forward operation, Xiaomi.

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- S2-2502513, Support of S&F Monitoring List in S&F Mode.