3GPP TSG-RAN WG2 Meeting #129bis	R2-2501788

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item		:	8.2.2 (Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on ambient IoT paging

Document for	:	Discussion and Decision

1.	Introduction

In the previous RAN2 meeting, some agreements were reached on ambient IoT (A-IoT or AIoT) paging as follows [1].

Agreement – RAN2#129 - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In WID [2], RAN2 aims to standardize the RAN aspects of A-IoT with the following objectives.

R19 RAN WID - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This contribution presents our views on A-IoT paging, considering the current progress in RAN2 and the WID. Among the unresolved (FFS) issues, we primarily address those not covered in the post-email discussion, such as paging identifier details, paging message content and format, and the paging procedure in multiple-reader scenarios.

2.	Discussion

2.1	Paging identifier

As the trigger for the A-IoT paging procedure, the A-IoT paging message includes identifier(s) (hereafter referred to as paging identifier(s)) representing the identities of target devices that must respond and proceed with subsequent steps. We need to discuss how the paging identifier should be stored in the A-IoT paging message, with a particular focus on their format, specifically their length.

According to TR 22.840 [3], there are various use cases for A-IoT. Among them, some assume that the length of the device identity could be variable. For example, in the use case for automated warehousing, the device identity is expected to be 96 or 128 bits long. In the use case for intralogistics in automobile manufacturing, the device identity is expected to be 96 or 240 bits. The EPC code, which is the identity used in RFID, also has a variable length ranging from 96 to 496 bits.

As discussed above, considering representative use cases, examples from reference technologies (e.g., the EPC of RFID), and the need for extensibility and forward compatibility, we believe it is natural to expect that the length of the device identity in the A-IoT system may be variable. In other words, different types of A-IoT device identities may have different lengths.

Observation 1: Since different types of A-IoT device identities may have different lengths, the type (and length) of the paging identifier included in the paging message may also vary.

Then, to enable A-IoT devices to read the paging identifier of variable length stored in the A-IoT paging message, a ‘type field’ for each paging identifier should be included in the A-IoT paging message, alongside the corresponding paging identifier. We therefore propose the following.

Proposal 1: A-IoT paging message should include a ‘type field’ for each paging identifier included.

Meanwhile, as outlined in WID, RAN2 aims to design a paging message format that allows multiple identifiers to be contained in a single paging message for forward compatibility purposes. In line with this, we propose the following Proposal 2. Additionally, to distinguish each identifier in multiple-identifier scenarios, we believe that the ‘type field’ for each identifier should also be included in the A-IoT paging message accompanying the related identifier.

Proposal 2: To ensure forward compatibility with scenarios where multiple paging identifiers can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple paging identifiers.

Proposal 2a: If a reserved field for multiple paging identifiers is defined, the A-IoT paging message should include a ‘type field’ for each identifier stored in the field.

2.2	Paging and random-access procedure

As summarized below, RAN1 decided to support frequency domain multiple access (FDMA) for device-to-reader (D2R) transmission, while FDMA is not supported for reader-to-device (R2D) transmission [4][5][6].

Agreement – RAN1#116bis - Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS Agreement – RAN1#118bis - Capture in the TR that for R2D: - Time-domain multiplexing is the baseline. - Code-domain multiplexing is not considered for device 1/2a/2b. - Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1 Agreement – RAN1#120 - Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

Based on the above agreements, the following scenario can be considered.

Figure 1. Example scenario of FDMA applied to D2R transmission

As shown in Fig.1, if FDMA is applied, the A-IoT device needs to support the transmission of D2R, e.g., Msg1 transmission, on specific frequencies. In other words, the reader should specify the frequency for D2R transmission to the A-IoT device. We think the simplest approach is to include a list of frequency information for D2R transmission in the A-IoT paging message. Then, when the A-IoT device receives the A-IoT paging message, the device can randomly select an access occasion and the frequency to transmit Msg1 (the same may apply to Msg3).

Proposal 3: A-IoT paging message should include a list of frequency information for D2R transmission, e.g., Msg1 transmission.

Meanwhile, during the RAN2#129 meeting, RAN2 agreed that two approaches could be considered for the R2D transmission, which determines the Msg1 resource(s) after the initial paging message [1].

Agreement - RAN2#129 From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

We think that it is also necessary to discuss whether, for the very first Msg1 resource, an additional R2D transmission (regardless of whether it is a new R2D message or a reused paging message) is needed, or if the initial paging message itself is sufficient to indicate the first Msg1 resource. Since receiving an R2D transmission consumes device energy, it may be more energy-efficient to have the initial paging message directly indicate the first Msg1 resource, rather than introducing an additional R2D transmission after the paging message. Moreover, since the first Msg1 resource occurs almost right after the initial paging message, there should not be significant scheduling issues for the first Msg1 resource. Therefore, we note that whether the initial A-IoT paging message can directly indicate the first Msg1 resource should be discussed. As mentioned above, we believe that the initial paging message can directly indicate it.

Proposal 4: A-IoT paging message directly indicates the first Msg1 resource (i.e., the start of the first access occasion) without requiring an additional R2D transmission.

2.3	Paging procedure in multiple-reader scenario

As outlined in WID, RAN has decided on Topology 1 as the general scope. According to TR 22.840 [3], even within Topology 1, there are use cases where A-IoT devices may be covered by multiple readers, such as in the flower auction scenario. In such cases, an A-IoT device located at the intersection of multiple readers' coverage areas could receive multiple A-IoT paging messages from different readers (hereafter referred to as the multiple-reader scenario).

The issues related to A-IoT device location, as described in TR 38.769 [7], further emphasize the need to study multiple-reader scenarios. If the A-IoT CN sends a location-related command to multiple readers, an A-IoT device may receive multiple A-IoT paging messages from these different readers. This introduces the necessity for RAN2 to consider the implications of the multiple-reader scenario.

TR 38.769 A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

In the multiple-reader scenario, the A-IoT device must determine how to respond to these paging messages. One situation may involve the device deciding whether to respond to only one paging message (e.g., the first) or to all paging messages from different readers. For example, in an inventory service where the device’s location relative to the readers is unknown, the CN may request the inventory service from all readers, prompting them to initiate the A-IoT paging procedure. In this case, it would be sufficient for the device to respond to the first received paging message.

On the other hand, there are scenarios where the A-IoT device may need to respond to all paging messages from different readers. As noted in TR 38.769, for accurate location purposes, it may be necessary to inventory an A-IoT device using multiple readers. In such cases, the A-IoT device should respond to each paging message from different readers. It may be also needed to avoid duplicate responses to the same reader while allowing responses to different readers. To achieve this, the device must be able to distinguish the source of each paging message.

During the RAN2#129 meeting, RAN2 agreed not to specify specific solutions for the multi-reader scenario as follows.

Agreement – RAN2#129 - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

However, concerns remain regarding whether the multiple-reader scenario can be adequately supported with just the transaction ID and implementation, as such an approach would require very tight coordination between readers. Given that the degree of overlap in coverage of different readers in the multiple-reader scenario, as well as the deployment information of nearby devices, may not be perfectly accurate from either the reader's or the CN's perspective, achieving flawless coordination could be difficult. Therefore, we believe that relying solely on the transaction ID may not be sufficient to achieve the desired device behaviour, especially for location purposes. In other words, unless there is very tight coordination between readers regarding transaction IDs—which is challenging—the transaction ID alone will be inadequate, and additional information, such as a reader ID or an indication of whether multiple responses are required, may be necessary.

One alternative approach for the multiple-reader scenario is to allow the device to know which reader the paging came from via a reader ID. Additionally, the device should be able to determine whether the paging corresponds to the same service (e.g., location) and identify whether multiple responses may be required. This method could be effective in situations where coordination on transaction IDs is imperfect. We therefore propose the following.

Proposal 5: A-IoT paging message should include a reader ID, which indicates the reader from which the message is transmitted.

3.	Conclusion

This contribution makes the following observations and proposals:

Observation 1: Since different types of A-IoT device identities may have different lengths, the type (and length) of the paging identifier included in the paging message may also vary.

Proposal 1: A-IoT paging message should include a ‘type field’ for each paging identifier included.

Proposal 2: To ensure forward compatibility with scenarios where multiple paging identifiers can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple paging identifiers.

Proposal 2a: If a reserved field for multiple paging identifiers is defined, the A-IoT paging message should include a ‘type field’ for each identifier stored in the field.

Proposal 3: A-IoT paging message should include a list of frequency information for D2R transmission, e.g., Msg1 transmission.

Proposal 4: A-IoT paging message directly indicates the first Msg1 resource (i.e., the start of the first access occasion) without requiring an additional R2D transmission.

Proposal 5: A-IoT paging message should include a reader ID, which indicates the reader from which the message is transmitted.

4.	References

- Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR)

- 3GPP TR 22.840, Study on Ambient power-enabled Internet of Things

- R1-2403821, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0

- R1-2409341, Final Report of 3GPP TSG RAN WG1 #118b v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR