3GPP TSG-RAN WG2 Meeting #129-bis	R2-2501789

Wuhan, China, Apr. 7th – 11th, 2025

Source:	vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital

Title:	Way forward on study of beam-level prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

The study of AI for mobility has the following objective regarding RRM measurement and event prediction, which includes beam-level measurement prediction.

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

At the RAN2#127 meeting, 3 sub-use cases for L3 filtered beam-level prediction were agreed. However, due to limited time, the study of beam-level prediction was concluded as lower priority than cell-level prediction.

Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

At the RAN2#129 meeting, whether to continue the study of L3 beam-level prediction was marked as FFS.

Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events

In this contribution, we will discuss the feasibility and necessity of L3 beam-level prediction, and then propose a way forward for the study of L3 beam-level prediction.

- Discussion

  - Feasibility analysis of L3 beam-level prediction

In the Rel-18 SI "Study on AI/ML for NR Air Interface", the performance of spatial domain and temporal beam predictions (i.e., BM-Case1 and BM-Case2) were investigated as representative sub-use cases.

- BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A. Note: Set A is for DL beam prediction. The codebook construction of Set A and Set B can be clarified by companies. - AI/ML model input consider: Alt 1): Only L1-RSRP measurement based on Set B; Alt.2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): CIR based on Set B; Alt. 4): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A (Set A and Set B are not the same). Alt. iii): Set A and Set B are the same. - AI/ML model input consider: measurement results of K (K≥1) latest measurement instances with the following alternatives: Alt. 1): Only L1-RSRP measurement based on Set B; Alt 2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - F predictions for F future time instances can be obtained based on the output of AI/ML model, where each prediction is for each time instance. At least F=1.

Simulation results in TR 38.843 [2] demonstrated that AI-based methods can improve Top-1 beam prediction accuracy compared to non-AI approaches. Based on the simulation results, the spatial and temporal domain beam predictions were included in the Rel-19 WI "AI/ML for NR Air Interface"[3].

BM-Case1, when Set B is a subset of Set A - (A)With measurements of fixed Set B of beams that of 1/4 of Set A of beams - Top-1 DL Tx beam prediction accuracy: - evaluation results from 9 sources indicate that, AI/ML can achieve about 70%~80% beam prediction accuracy - evaluation results from 9 sources indicate that, AI/ML can achieve about 80%~90% beam prediction accuracy - evaluation results from 7 sources indicate that, AI/ML can achieve more than 90% beam prediction accuracy BM-Case1, Set B is different than Set A - Top-1 DL Tx beam - evaluation results from 3 sources indicate that, AI/ML can achieve more than 80% beam prediction accuracy from 5 sources indicate that, AI/ML can achieve more than 55% beam prediction accuracy - 2 sources reported more than 80% beam prediction accuracy with 100% outdoor UEs, and more than 60% beam prediction accuracy with 20% outdoor UEs. - Evaluation results from 1 source shows that, with limited measurements (e.g., 1 or 4) of narrow beams in Set A=32, AI/ML can increase 15% or 30% beam prediction accuracy [respectively] compared with 55% beam prediction accuracy with measurement of wide beams only.

BM-Case2, when Set A = Set B - For 320ms prediction time, evaluation results from 7 sources show that AI/ML can increase about up to 3%~8% prediction accuracy, and evaluation results from 2 sources show that AI/ML can increase about 18.5%~23.5% prediction accuracy in terms of Top-1 beam prediction accuracy

In AI/ML for mobility, the intention for beam-level prediction is for RACH resource allocation, which also relies on the prediction of the best beam(s). Therefore, the feasibility conclusion on AI-based beam prediction of the Top-1 beam can be applicable to L3 beam-level prediction to some extent.

Observation 1: In the Rel-18 SI of AI/ML for air, it was concluded that AI-based methods can improve Top-1 beam prediction accuracy compared to non-AI approaches.

On the other hand, in the current AI/ML for mobility study, the following 6 sub-use cases are considered for L3 cell-level and L3 beam-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Figure 2-1: Measurement Model

Based on the measurement model in Figure 2-1, it can be observed that sub-use cases 1 and 4 share identical prediction mechanisms, i.e., predicting future L1 beam-level results based on historical L1 beam-level measurements. The distinction lies in the subsequent processing:

- Sub-use case 1 further derives L3 cell-level prediction at Point C;

- Sub-use case 4 further derives L3 beam-level prediction at Point E.

Observation 2: The L1 beam-level measurement prediction in sub-use case 1 of L3 cell-level prediction can be reused to derive the L3 beam-level prediction (i.e., sub-use case 4).

For temporal domain prediction Case A, although it has been agreed that "sub-use case 2 demonstrates the highest prediction accuracy among sub-use cases 1, 2, and 3", simulation results from 4 sources (7 sources in total with simulation results for both sub-use cases 1 and 2) reveal that the performance of sub-use cases 1 and 2 remains comparable, which means that sub-use case 4 is deemed with acceptable performance.

Table 2-1: Submitted simulation results of temporal domain prediction sub-use cases 1 and 2 at RAN2#128 meeting

( Average L3 cell-level Prediction error in dB)

|  | Sub-use case 1 | Sub-use case 2 | Note |
| --- | --- | --- | --- |
| vivo R2-2409667 | 1.6 | 1.47 | OW=PW= 400ms UE speed =60/90/120 km/h |
| SS R2-2409823 | 2.019 | 0.874 | OW=1600ms PW=400ms UE speed =30 km/h |
| MTK R2-2409869 | 2.49 | 0.92 | OW=160ms PW=640ms UE speed =60 km/h |
| DOCOMO R2-2410020 | 4 | 3.275 | OW=PW= 2000ms UE speed =60/90/120 km/h |
| OPPO R2-2410187 | 2.29 | 1.21 | OW=800ms PW=400ms UE speed =60 km/h |
| HW R2-2410539 | 2.98 | 2.83 | OW=PW= 400ms UE speed =60/90/120 km/h |
| ZTE R2-2410799 | 0.875 | 0.813 | OW=800ms PW=400ms UE speed =60/120 km/h |

For frequency domain prediction, the performance of sub-use cases 1 and 2 remains comparable as illustrated in Table 2-2.

Table 2-2: Submitted simulation results of frequency domain prediction sub-use cases 1 and 2 at RAN2#128 meeting

( Average L3 cell-level Prediction error in dB)

|  | Sub-use case 1 | Sub-use case 2 | Note |
| --- | --- | --- | --- |
| MTK R2-2409869 | 1.97 | 1.64 | OW=400ms UE speed =30 km/h |
| DOCOMO R2-2410020 | 1.45 | 0.7 | OW=200ms UE speed =90 km/h |
| HW R2-2410539 | 0.725 | 0.742 | OW=1000ms UE speed =30/60/90 km/h |

Observation 3: For L3 Cell-level prediction, some simulation results reveal that the performance of sub-use case 1 (based on beam-level prediction) is comparable to sub-use case 2.

Based on the above analysis, current conclusions on AI-based beam management and L3 cell-level prediction can already provide validation evidence for the feasibility of temporal and frequency domain L3 beam-level prediction, at least for sub-use case 4. Therefore, the simulation and discussion of L3 beam-level prediction can be pursued in a best-effort way.

Observation 4: Current conclusions on AI-based beam management and L3 cell-level prediction already prove the feasibility of L3 beam-level prediction (at least for sub-use case 4).

  - Necessity analysis of L3 beam-level prediction

Figure 2-2: Inter-gNB handover procedures

During the handover, the source gNB sends a HANDOVER REQUEST message to the target gNB passing an RRC container with the necessary information to prepare the handover at the target side. The HANDOVER REQUEST can include the reported measurement information from UE, which includes beam-related information, if available.

Figure 2-3: Potential Handover Modeling

Based on the above handover procedures, there exists a time gap between measurement reporting and handover execution (i.e., t0 to t1 in Figure 2-3). If the network utilizes prediction results (e.g., temporal domain Case A prediction) for the handover decision and the PW is longer than TTT, the time gap could be further extended since the handover command transmission to the UE may occur after a duration following handover preparation. If the target gNB configures the RACH resources based on the beam-level measurements at t0, the CFRA procedure during handover may fail due to the long time gap between t0 and t1. Alternatively, CBRA would have to be utilized, but this can increase HO delay due to potential preamble collisions and increase the chances of HO failure.

The same issue arises also for spatial and frequency domain predictions. That is, the network needs to receive the spatial/frequency domain beam prediction in the report from the UE to allocate random access resources in the target cell. We believe that the same considerations should be applied also to spatial and frequency domain beam predictions.

Consequently, it becomes critical to provide L3 beam-level prediction in the measurement report. Otherwise, the target gNB is not able to configure optimal RACH resources in the handover command.

Observation 5: L3 beam-level prediction is necessary for the target gNB to provide optimal RACH resources to the UE during handover.

  - Proposed way forward for study of beam-level prediction

Based on the above analysis of the feasibility and necessity of temporal L3 beam-level prediction, we propose:

Proposal 1: Consider L3 beam-level prediction in the specification impact analysis in the SI phase.

Furthermore, the detailed scope and priority of beam-level prediction can follow that of cell-level prediction. I.e., temporal, frequency and spatial domain can be considered. Among them, temporal and frequency domain prediction to be considered with higher priority for detailed specification impact analysis, if needed.

Proposal 2: L3 beam-level prediction includes temporal, frequency and spatial domain predictions. Temporal and frequency domain prediction to be considered with higher priority for detailed specification impact analysis, if needed.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Feasibility analysis of L3 beam-level prediction

Observation 1: In the Rel-18 SI of AI/ML for air, it was concluded that AI-based methods can improve Top-1 beam prediction accuracy compared to non-AI approaches.

Observation 2: The L1 beam-level measurement prediction in sub-use case 1 of L3 cell-level prediction can be reused to derive the L3 beam-level prediction (i.e., sub-use case 4).

Observation 3: For L3 Cell-level prediction, some simulation results reveal that the performance of sub-use case 1 (based on beam-level prediction) is comparable to sub-use case 2.

Observation 4: Current conclusions on AI-based beam management and L3 cell-level prediction already prove the feasibility of L3 beam-level prediction (at least for sub-use case 4).

Necessity analysis of L3 beam-level prediction

Observation 5: L3 beam-level prediction is necessary for the target gNB to provide optimal RACH resources to the UE during handover.

Proposed way forward for study of beam-level prediction

Proposal 1: Consider L3 beam-level prediction in the specification impact analysis in the SI phase.

Proposal 2: L3 beam-level prediction includes temporal, frequency and spatial domain predictions. Temporal and frequency domain prediction to be considered with higher priority for detailed specification impact analysis, if needed.

- Reference

- RP-242393	Revised SID on AIML for mobility in NR

- TR 38.843		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-243244	Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface