3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2501794

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.9.3

Source:	NTPU

Title:	Further considerations on Locating of CB-Msg3 Replicas for DSA

Document for:	Discussion and Decision

- Introduction

At RAN2#129, agreements were reached on supporting DSA for CB-Msg3 transmissions, alongside discussions on the potential of implicit pointers to improve network efficiency. Document R2-2501265 [1], contributed by multiple organizations, introduced a concrete proposal for such a mechanism, including algorithmic details and implementation considerations. There was preliminary support for this concept:

Proposal: RAN2 to specify a procedure to be followed by UEs to randomize placement of their replicas, enabling the use of an implicit pointer to infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

-	QC thinks this is useful also for DSA

-	HW is not convinced about the benefits and thinks this does not come for free.

-	IDC thinks that using a pseudo-random strategy is not something new.

=> Can come back to this in the next meeting

This document builds on the ideas presented in [1], offering complementary insights and reinforcing the value of implicit replica pointers as a practical enhancement in Release 19 for CB-Msg3 DSA.

- Discussion

  - Reinforcement of Pseudo-Random Replica Placement

Uncoordinated replica placement without proper guidelines may lead to increased collision probability and inefficient spectrum usage. Document [1] provides a concrete solution through a predefined linear congruential generator (LCG)-based algorithm that allows the UE to generate replica positions deterministically using a seed derived from the CB-Msg3 payload. This reinforces the importance of standardized pseudo-random replica placement mechanisms to ensure testable, consistent UE behavior.

  - Implicit Pointer for gNB Inference of Replica Positions

Enabling the gNB to infer all replica positions from a single successfully decoded transmission provides multiple advantages:

- Avoids redundant CB-Msg4 transmissions over PDCCH

- Reduces UE monitoring complexity by limiting reception windows

- Enhances contention resolution strategies at the network side

These benefits are supported in [1] through practical examples and potential algorithmic derivations.

  - No Modification to CB-Msg3 Format

Unlike solutions that require explicit pointers, the implicit pointer approach does not require changes to the CB-Msg3 structure, as confirmed in [1]. This minimizes overhead and aligns with the goal of enhancing DSA while maintaining compatibility with existing message formats.

  - No Expected RAN1 Impact

As emphasized in [1], the seed required for pseudo-random positioning can be derived from MAC-layer information already available to both UE and gNB. This supports the conclusion that enhancements for implicit replica pointers are contained within the RAN2 scope.

Observation 1: The use of a predefined pseudo-random algorithm for DSA replica positioning ensures consistent and testable UE behavior, while reducing systematic collisions.

Observation 2: Implicit replica pointers enable the gNB to determine the complete set of replica positions based on a single decoded transmission, leading to more efficient downlink signaling and resource utilization.

Observation 3: This mechanism avoids the need for CB-Msg3 format changes or additional signaling, aligning with the goals of low overhead and minimal complexity.

Observation 4: The approach has no expected impact on RAN1, as necessary parameters for replica positioning can be derived from existing MAC-layer payload content.

- Proposal 1: RAN2 to specify a standardized pseudo-random procedure for CB-Msg3 DSA replica placement, enabling the use of implicit replica pointers such that the gNB can infer all or part of the replica positions from a successfully decoded transmission.

- Note: Although the benefits of using implicit replica pointers are significant, we propose not to exclude the possibility of supporting explicit pointers in the next release and beyond. This would allow future flexibility and optional enhancements that may be beneficial in specific deployment scenarios.

- Conclusion

The use of implicit replica pointers offers a practical and efficient enhancement for supporting DSA-based CB-Msg3 transmission in Release 19. By allowing the gNB to infer the locations of all or part of the replicas from a single successfully decoded transmission, the mechanism avoids uplink and downlink overhead, improves resource utilization, and simplifies UE reception procedures—all without requiring modifications to the CB-Msg3 format or introducing impact on RAN1.

This document advocates for the standardization of a pseudo-random procedure for replica placement to enable such implicit inference. At the same time, it recognizes the value of preserving future flexibility: the option to introduce explicit replica pointers in subsequent releases should remain open, particularly for scenarios that may benefit from more direct signalling.

- Proposal 1: RAN2 to specify a standardized pseudo-random procedure for CB-Msg3 DSA replica placement, enabling the use of implicit replica pointers such that the gNB can infer all or part of the replica positions from a successfully decoded transmission.

- References

[1] R2-2501265, "Implicit pointer for locating CB-Msg3 DSA replicas", DLR, ESA, Toyota ITC, Inmarsat, Viasat, Thales, Aalyria Technologies, Echostar, Eutelsat Group, Sateliot, Novamint, 3GPP TSG-RAN WG2 Meeting #129, Athens, Feb. 2025.