3GPP TSG-RAN WG2 Meeting #129bis	R2-2501804

Wuhan, China, Apr 7th-11st, 2025

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

According to the RAN plenary #106 meeting, the approved work item on NTN for NR contains the following objective on DL coverage enhancements [1]:

R19 RAN WID The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

During RAN2#128,129 meetings, RAN2 made some progress with the following statements:

RAN2#128 meeting Agreements:

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

- As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

- RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

- For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states

RAN2#129 meeting Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

In this contribution, we provide our views on DL coverage enhancements.

2. Discussion

Access Control

From the last several meetings, RAN2 made some progress about the access control mechanism for DL-CE cell as below.

RAN2#127 meeting agreements: - From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects). RAN2#128 meeting agreements: - If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.).

During RAN#106, it was determined that the maximum additional SSB periodicity assumed by the UE during the initial cell search procedure is 160ms. And, during the RAN2#127bis meeting, it was agreed that SSB periodicity of 160ms does not cause any issues within current SSB configuration. For example, ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 already support SSB periodicity of 160ms. Therefore, there is no issue for the UEs not supporting DL-CE to operate in cells with 160ms SSB periodicity.

Furthermore, the system level enhancements of the DL-CE cell do not consider the Rel-18 network energy saving techniques, according to the updated WID [1]. So far, no improvements have been introduced for DL-CE cells other than SSB periodicity extension. So, considering the discussion so far, if a UE successfully camps on a DL-CE cell, there is no problem for the UE to operate on the DL-CE cell.

Observation 1	If the UEs not supporting DL-CE successfully camp on DL-CE cells, there is no issue for UEs not supporting DL-CE to operate on DL-CE cells with current specification.

At RAN1#119 meeting, the following observation has been made:

RAN1#119 meeting Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

According to observations from the RAN1#119 meeting, the legacy UEs are difficult to camp on DL-CE cells. Even if the legacy UEs success to camp on a DL-CE cell, the legacy UE has no issue to operate in the DL-CE cell. Therefore, due to the backward compatibility issue during the cell search process, there is no need to introduce a separate access control mechanism to bar UEs that do not support DL-CE.

Observation 2	Because of the backward compatibility issue during the initial access process, a separate access control mechanism for UEs not supporting DL-CE is not meaningful.

Proposal 1	The UE not supporting DL-CE does not need separate access control in DL-CE cells.

SMTC impacts due to extended SSB periodicity

In the RAN2#129, there was discussion on different SSB periodicity of neighbor cells in the same frequency layer, and RAN2 made the following agreement.

RAN2#129 meeting Agreements: - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer.

As RAN2 considers a scenario where intra-satellite transmits SSBs with an extended periodicity, neighbor cells may have different SSB transmission periodicities and timing offsets with a serving cell. However, the current SMTC lists for NTN, SMTC4list, cannot be configured with a SMTC periodicity different from SMTC1.

If UE performs measurement using SMTC with periodicity which is different from the cell’s SSB, there may have UE power consumption issue. For example, UE may perform measurements, even when the cell is not transmitting SSB. Or, the UE may not perform measurements, even if the cell transmits SSB. This causes the SS-RSRP measurement time to be longer, delaying the cell reselection procedure.

Observation 3	Configuring SMTC without considering SSB periodicity leads to unnecessary UE power consumption.

Proposal 2	The network should be allowed to configure different periodicities and offsets for the multiple SMTCs on a single frequency.

SSB beam hopping scenarios and SMTC impact

From the last RAN2 meeting, RAN2 agreed to support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC as shown below.

Figure1. SSB Beam hopping pattern across different cells [2]

Option a

In our understanding, the SSB configuration in current specification supports option a (clustered cells illuminated). Considering the case where neighboring cells adjacent to a serving cell belong to different clustered cell groups, there can be at most three clustered cell groups. Therefore, the current four SMTCs per frequency is sufficient. Also, we consider that the solution for option b can be applied to option a.

Observation 4	In the clustered cells illuminated scenario, UEs at the edge of the clustered cell group can be supported with the current SMTC configuration.

Proposal 3	From RAN2 point of view, no additional SMTC solution is required for the clustered cells illuminated scenario.

Option b

From the RAN2#129 meeting offline discussion, RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighboring cells), existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may not be enough.

The following options have been suggested as possible solutions during the past several meetings [3]:

- Configure multiple SMTCs (more than 4) and SMTC selection (e.g. location-based SMTC selection, UE autonomous selection)

- SSB-specific SMTC

- UE assistance information reporting

- Configure one SMTC with small periodicity, in each period (SMTC occasion) UE is indicated which PCI to use

Before discussing the possible solutions in detail, we should identify whether the UE needs to be configured with SMTCs for all the neighbor cells adjacent to the serving cell.

From our understanding, the network cannot know the exact location of UEs in RRC idle, inactive state. Additionally, the network cannot know whether UE in RRC idle, inactive state exists in the cell. Hence, the network cannot selectively configure SMTCs to measure only the closest neighbor cells among the six neighbor cells. Therefore, the network should transmit SMTC configurations that allow the UE to measure all six adjacent neighbor cells.

Proposal 4	The network should configure SMTCs considering all six adjacent neighbor cells.

From the above-mentioned perspective, option 3, which relies on UE assistance information reporting, is not a feasible solution. This is because signaling between the network and RRC idle/inactive UEs is limited to broadcasting from the network to the UE. To report the UE assistance information, the RRC idle, inactive UE should transition to RRC connected state. That is, whenever the UE needs to update its SMTC configuration due to reasons, such as UE mobility, satellite mobility, or neighbor cell change due to the EMC, UE should transition to RRC connected state and receive SMTC configuration via dedicated signaling.

Observation 5	From the RAN2 perspective, configuring SMTC using UE assistance information reporting is not a feasible solution.

However, the current SMTC specification does not support configuring six SMTCs per frequency. Even if the UE could be configured with six SMTCs per frequency, it may lead to increased UE power consumption. The UE consumes more power when measuring six neighbor cells using individual SMTC windows for each cell compared to measuring all neighbor cells using a single SMTC.

Proposal 5	RAN2 should consider SMTC configuration solutions that take into account the UE power consumption issues for the case where simultaneously active cells are scattered.

From our understanding, option 1 and 2 are viable solutions for minimizing UE power consumption. These options provide additional information that helps the UE identify which SMTC corresponds to the closest neighbor cell. And this enables the UE to prioritize nearest neighbor cell’s measurements, thereby reducing unnecessary power consumption.

For example, in option 1, the network configures SMTC with the associated neighbor cell’s PCI and reference location information. Using this reference location information, the UE can determine the closest neighbor cell and prioritize its measurements accordingly.

In option 2, the network configures SMTC with SSB beam index information. Specifically, each SMTC is associated with a physically nearby SSB beam. Since the UE can obtain SSB beam strength information during the initial cell search, UE can select the SMTC associated with the strongest SSB beam index for measurement.

Observation 6	The location based SMTC selection and SSB-specific SMTC are feasible solutions to minimize the UE power consumption.

However, option 4 is not related to minimizing UE power consumption. Option 4 does not provide information about which SMTC occasion is associated with the closest neighbor cell to the UE.

Additionally, in the case of option 4, aligning the SSB transmission timing of neighbor cells with the SMTC periodicity is required, which introduces additional constraints from a network deployment perspective. Moreover, using a single SMTC to measure neighbor cell SSBs with different transmission timing is essentially the same as configuring separate SMTCs for each neighbor cell’s SSBs transmission timing from the UE’s operational perspective. Configuring the network as in option 4 is related to the SMTC signaling optimization, but it does not provide any benefit in terms of UE power consumption. Therefore, option 4 does not offer any operational advantage from the UE’s perspective when measuring neighbor cell SSBs with a single SMTC. Furthermore, due to its limitations from a network deployment perspective, option 4 provides fewer benefits compared to option 1 and option 2.

Observation 7	Single SMTC with small periodicity is not a feasible solution due to UE power consumption.

Observation 8	Single SMTC with small periodicity does not provide any advantage from UE’s operational perspective compared to the configuring multiple SMTCs and SMTC selection, and the SSB-specific SMTC.

In the case of option 2, we have concerns that there has been no consensus on the cell-beam mapping relationship in NTN. Therefore, providing SSB beam and SMTC correlation information to allow UE to select SMTCs based on SSB beam strength introduces ambiguity. For example, there could be a scenario where the NTN cell has only a single SSB beam. Additionally, our understanding is that the correlation information between SSB beams and neighboring cells may not be accurate depending on the UE’s location within the serving cell or the number of SSB beams in a cell. Therefore, the SMTC selected by the UE for measurement may not correspond to the nearest neighbor cell.

Observation 9	SSB-specific SMTC has ambiguity and may have less accuracy regarding prioritizing the neighbor cell measurements.

Even though option 1 causes more signaling overhead compared to other options, prioritizing measurements based on the information of neighbor cell’s reference location has more accuracy compared to other options. Considering that prioritizing neighbor cells measurements should be implemented based on accurate information and UE power consumption has great importance to the RRC idle, inactive UE, we think option 1 is the most reasonable solution for the case where simultaneously active cells are scattered.

Proposal 6	RAN2 consider the configuring multiple SMTCs and SMTC selection, as a feasible solution for the case where simultaneously active cells are scattered and FFS on the details.

3. Conclusion

In this contribution, following statements are proposed:

Access Control

Observation 1	If the UEs not supporting DL-CE successfully camp on DL-CE cells, there is no issue for UEs not supporting DL-CE to operate on DL-CE cells with current specification.

Observation 2	Because of the backward compatibility issue during the initial access process, a separate access control mechanism for UEs not supporting DL-CE is not meaningful.

Proposal 1	The UE not supporting DL-CE does not need separate access control in DL-CE cells.

SMTC impacts due to extended SSB periodicity

Observation 3	Configuring SMTC without considering SSB periodicity leads to unnecessary UE power consumption.

Proposal 2	The network should be allowed to configure different periodicities and offsets for the multiple SMTCs on a single frequency.

SSB beam hopping scenarios and SMTC impact

Observation 4	In the clustered cells illuminated scenario, UEs at the edge of the clustered cell group can be supported with the current SMTC configuration.

Proposal 3	From RAN2 point of view, no additional SMTC solution is required for the clustered cells illuminated scenario.

Proposal 4	The network should configure SMTCs considering all six adjacent neighbor cells.

Observation 5	From the RAN2 perspective, configuring SMTC using UE assistance information reporting is not a feasible solution.

Proposal 5	RAN2 should consider SMTC configuration solutions that take into account the UE power consumption issues for the case where simultaneously active cells are scattered.

Observation 6	The location based SMTC selection and SSB-specific SMTC are feasible solutions to minimize the UE power consumption.

Observation 7	Single SMTC with small periodicity is not a feasible solution due to UE power consumption.

Observation 8	Single SMTC with small periodicity does not provide any advantage from UE’s operational perspective compared to the configuring multiple SMTCs and SMTC selection, and the SSB-specific SMTC.

Observation 9	SSB-specific SMTC has ambiguity and may have less accuracy regarding prioritizing the neighbor cell measurements.

Proposal 6	RAN2 consider the configuring multiple SMTCs and SMTC selection, as a feasible solution for the case where simultaneously active cells are scattered and FFS on the details.

4. References

[1] RP-243300	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3	Moderator (THALES).

[2] R2-2500689	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell

[3] R2-2501415	Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements (Huawei)