3GPP TSG-RAN WG2 Meeting#129bis	R2-2501808

Wuhan, China, 7th –11th, April 2025

Source:	vivo

Title:	Discussion on remaining issues of AI/ML enhanced positioning

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

RAN2#129 meeting has made progress on functionality-based LCM of UE-sided model for AI enhanced positioning as follows [1].

Agreements 1: Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation, 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

In this paper, we mainly focus on the leftover issues for RAN2 from the following perspectives:

- data collection;

- Signaling of performance monitoring/functionality management.

- .Discussion

## Data collection

RAN2#129 meeting has discussed some principles for UE-side data collection. For AI enhanced positioning Case 1, we would like to analyze whether these principles can be applied as well.

Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration.

### Configuration of data collection

Regarding the configuration of UE-side training data collection, and network control over the procedure of data collection, we made the following agreements.

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g., start/stop indication).

Configuration

For the positioning use case, since RAN1 has not made any decision on whether to use the associated ID as positioning assistance information from NW side, we think it is better for RAN2 to wait for their decision on the detailed content of assistance data for Case1 and include it accordingly.

- For AI enhanced positioning, wait for RAN1 progress about the detailed configuration for training data collection (i.e., RAN2 would not decide whether to include associated ID as a part of configuration).

NW control over data collection

It is a common sense that data collection may be initiated by UE for Case 1, where UE (UE side server) deploys and trained the model for the current stage. In this sense, UE may trigger MO-LR request towards LMF to connect with NW in order to get assistance data for training data collection. On the other hand, since the assistance data is provided by NW, it is up to NW to determine the start and stop the data collection by whether to provide the corresponding PRS for positioning calculation.

- UE can request for training data configuration via LPP Request Assistance Data, and the network can provide or release the data collection configuration (at any point in time).

### Consistency for training and inference/UE request for data collection

On whether UE is allowed to suggest a configuration for UE-side training data collection, it is still for further study.

- For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

For AI positioning, RAN1#118bis meeting came to the agreement that on-demand PRS procedure can be used for training data collection [2] in Case 1.

Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Since functionality is deployed at UE side, it has the knowledge of DL PRS configurations during training stage. When it comes to the stage of inference, to ensure the consistency of training and inference, UE may request for suitable PRS configurations via UE-initiated on-demand PRS scheme. In this way, when assigning DL PRS configurations for inference, LMF can forward such preference to gNB, and consistency can be ensured.

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

The current on-demand PRS scheme is designed to ensure the positioning QoS, therefore only considering certain characteristics (e.g., bandwidth, periodicity) of the PRS, without requesting specific cells to transmit PRS. However, from our understanding, except for PRS configuration itself, the consistency should be ensured by the same PRS transmission cells. Therefore, the target UE should send the area where the interested PRS is applicable to LMF. With this a priori knowledge, LMF is able to attach area and PRS configurations to specific NW-side additional condition. Furthermore, LMF may request the RAN-node near the UE and match with the condition(s) to perform DL positioning accordingly.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

## Signaling related to performance monitoring/functionality management

At the previous RAN1 meetings [2], the following agreements/conclusions were made regarding the performance monitoring of AI/ML positioning Case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Summarized from the RAN1 agreement, the performance monitoring and functionality management can be realized in the following options for positioning use case 1:

- Option 1: UE performs monitoring metric calculation and functionality management

- Option 2: LMF performs monitoring metric calculation and UE performs functionality management

- Option 3: UE performs monitoring metric calculation and LMF performs functionality management

- Option 4: LMF performs monitoring metric calculation and functionality management

For Option 1, with no LMF perception of which AI/ML functionality is enabled, UE autonomously decides to enable AI/ML based positioning and performs functionality management, UE may report AI/ML output of location prediction as the legacy location estimate. However, since RAN2 has agreed that “’UE-autonomous, UE’s decision is not reported to the network’ is not considered for Rel-19”, no further discussion on Option 1 is needed in WI phase.

As to Option 2, for the LMF-involved positioning (i.e., not UE autonomous positioning) procedure, we tend to believe that UE cannot perform AI functionality management under LMF decision on positioning method, where AI functionality is designed to be a positioning method on the trend. That is, if the target UE receives the request of location information attached to positioning method correspond to a specific functionality (FFS the granularity), it should consider the functionality is activated. Therefore, Option 2 is excluded naturally.

In this sense, functionality management can only be performed by LMF in terms of R19 scope and positioning system layout.

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

For Option 3, functionality management is realized by LMF requesting UE with AI-enabled location information just as illustrated in Option 2.

- Functionality management (e.g., activation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

Furthermore, based on RAN1 input, LMF may provide some assistance information to UE to achieve performance monitoring metric calculation at UE, e.g., ground truth label of the target UE (Option A-1), position calculation assistance data (Option A-2), PRU measurement or PRU location (Option A-3). UE can also obtain label from PRU in 3GPP-transparent way (Option A-4), which has no specification impact and is outside RAN2 scope. Logically speaking, if LMF configures UE to perform performance monitoring, or UE itself has intention to perform performance monitoring, the information can be provided via LPP Assistance Data Transfer procedure in both proactive/reactive way.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

For Option 4, based on RAN1 input, to achieve performance monitoring metric calculation at LMF, the model output based on either target UE’s measurement (Option B-1) or PRU’s measurement (Optionb B-2) is needed from UE.

Since the model output is location estimate, which is already the location information request by LMF as in the legacy positioning session, this can directly reuse LPP Provide Location Information to transmit. And for the PRU measurement part, LPP Provide Assistance Data can be reused.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

On the basis of the above analysis, LMF or UE is able to perform performance monitoring metric calculation. LMF can be of long-term decision while UE can make real-time deciding on whether AI/ML positioning accuracy is trustworthy. However, RAN1 has been, in parallel, discussing entities to perform performance monitoring, we would like RAN2 to wait for RAN1 progress on whether both options should be introduced in specification.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

### 5.5.1 General The purpose of the abort procedure is to allow the target device or location server to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from the target device). ### 5.5.2 Procedures related to Abort Figure 5.5.2-1 shows the Abort procedure. Figure 5.5.2-1: LPP Abort procedure 1. A procedure P is ongoing between endpoints A and B. 2. Endpoint A determines that the procedure must be aborted and sends an Abort message to Endpoint B carrying the transaction ID for procedure P. Endpoint B aborts procedure P. #### – Abort The Abort message body in a LPP message carries a request to abort an ongoing LPP procedure. -- ASN1START Abort ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { abort-r9 Abort-r9-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } Abort-r9-IEs ::= SEQUENCE { commonIEsAbort CommonIEsAbort OPTIONAL, -- Need ON ..., epdu-Abort EPDU-Sequence OPTIONAL -- Need ON } -- ASN1STOP

One company raises the issue for NW control of functionality deactivation. We consider functionality deactivation is already supported by the LPP signalling Abort Message [2] to end a positioning procedure and no explicit reason to give.

In our understanding, the Abort message does not include abort related information in a per-method way. When AI/ML positioning is introduced as a positioning method as we agreed, there is no further information required to provide on a granularity of positioning method.

- LPP Abort message is used for one endpoint to end the ongoing positioning procedure with no per-method related reason to provide therein.

However, the network can use this message to end the ongoing AI/ML positioning session with no indication of the abort reason for deactivating the corresponding functionality.

- RAN2 to confirm there is no further enhancement on NW deactivation of AI functionality, i.e., LMF can deactivate AI positioning via the current LPP Abort message.

- Conclusion

In summary, we make the following proposals on functionality-based LCM for AI/ML enhanced positioning:

Data collection

- Configuration

- For AI enhanced positioning, wait for RAN1 progress about the detailed configuration for training data collection (i.e., RAN2 would not decide whether to include associated ID as a part of configuration).

- UE can request for training data configuration via LPP Request Assistance Data, and the network can provide or release the data collection configuration (at any point in time).

- Consistency for training and inference

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

Signaling of performance monitoring/functionality management

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

- Functionality management (e.g., activation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

- LPP Abort message is used for one endpoint to end the ongoing positioning procedure with no per-method related reason to provide therein.

- RAN2 to confirm there is no further enhancement on NW deactivation of AI functionality, i.e., LMF can deactivate AI positioning via the current LPP Abort message.

- Reference

- RAN2#129meeting Chair’s Notes

- TS 37.355	LTE Positioning Protocol (LPP)

- RAN1#118bis meeting Chair’s Notes