3GPP TSG-RAN WG2 Meeting #129bis	R2-2501818

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.9.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues on S&F operation in IoT NTN

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest version is in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In previous meetings, some agreements have been reached. In this paper, we will further discuss the remaining issues for supporting S&F operation and give our proposals.

Discussion

In previous meetings, for a cell supporting both Normal mode and S&F mode, RAN2 has had discussion on whether and how to deliver the switching timing information about the transition between S&F mode and Normal mode. Some agreements have been achieved while some scenarios or issues still need to be further discussed.

In the following, we will give our further analysis for the two basic scenarios:

Scenario1: the serving satellite operating in S&F mode is going to switch to Normal mode

Fig.1

As illustrated in Fig.1, for quasi-earth fixed cell, the serving satellite 1 currently works in S&F mode at T1 and is going to switch to Normal mode at T2. According to SA2 agreements in [2], MME onboard may reject the NAS procedure when satellite works in S&F mode (e.g., between T1 and T2). In order to avoid this failure, in previous meeting, RAN2 has assumed that UE could delay some NAS procedures till after T2.

Furthermore, RAN2#129 meeting has reached agreements as below on provision of time information on transition from S&F operation mode to Normal mode (e.g., T2 in the Figure 1, with example name as t-SFtoN for Sat-1 in SIB31 in following discussion). This time information would be provided with an absolute UTC time format and the change to this time would not result in system information change notification.

RAN2#129 Agreements - The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) - Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). - A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In addition, a duration of Normal mode could be provided which can further indicates when the serving satellite is going to switch to S&F mode next time. There is an argument that the previously agreed “S&F operation” indication in SIB1 can also be used to indicate the start of S&F mode. However, the difference lies in that, before the R19 UE receives this indication, it cannot predict when the Normal mode might end. As a result, it is possible that the UE has just initiated a response-needed service and then received the “S&F operation” indication, meaning the transition to S&F mode. In such a case, the previously initiated service may become an unnecessary operation. On the other hand, since we assume that certain services are better performed in Normal mode, if the UE can know how long the Normal mode will last, it can make an early decision on whether to initiate certain services during this time duration. Furthermore, this time duration can be optionally provided, if it’s absent, the UE can assume that the Normal mode will continue until it receives “S&F operation” indication via SIB1 or the time when the satellite stops serving the current area is reached.

There may be other thinking that a time point information on transition from Normal mode to S&F mode can be provided, indicating same thing as that deduced by duration of Normal mode. However, considering such time point information would also be an absolute UTC time format, to use “start of Normal mode + the duration of Normal mode” to deduce the time point on transition from Normal mode to S&F mode can be more signaling efficient.

Proposal 1: Besides the agreed time information on transition from S&F operation mode to Normal mode, a duration of Normal mode could be provided to R19 UE, e.g., via SystemInformationBlockType31(-NB) message.

Besides the above discussion on the need of new parameter, in the following, we further discuss whether and how some enhancements are needed for the setting of the existing timing parameters for serving satellite and neighbor satellite.

As illustrated in the example in Fig.1, the serving satellite 1 is going to stop serving the area at T3. And the next neighbor satellite 2 is going to start serving the area at T4 but without the available feeder link, and switch to Normal mode at later T5. In this case, the legacy way for setting the t-Service in SystemInformationBlockType3(-NB) for the serving satellite 1 is to set to T3, e.g., the time when the serving satellite is going to stop serving the area.

- Case#1: Under the continuous coverage scenario, it’s possible that, before the serving satellite stops serving the area, the neighbor satellite already can switch to Normal mode, then the timing relationship is that T3 > T5 > T4 (“>” means later than). In this case, for quasi-earth fixed cell, there may be two options for setting the legacy parameter t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of neighbor satellite 2 (this parameter will be used by the UE to decide when to start measurements on this satellite 2):

  - Alt1 (Legacy): If as legacy, the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is set as the time when the satellite 2 is going to start serving the area, e.g., T4, for a legacy UE that is still under the coverage of satellite 1, it will start measurement satellite 2 at T4 and maybe switch to satellite 2 quickly after T4. However, during time period between T4 and T5, although satellite 2 can provide coverage to this area, it has no available feeder link. Therefore the legacy UE may not work normally under satellite 2. We think this is too early switching to satellite 2 due to the unsuitable setting of t-ServiceStartNeigh of satellite 2. Instead, if the legacy UE can stay at satellite 1 till T5, it’s still possible for the legacy UE to have normal connection (the only issue may be that the radio quality may be a bit worse than camping at satellite 2). For R19 UE supporting S&F mode, the drawbacks of too early switching to neighbor satellite 2 may not be as bad as they are for legacy UE, but we still think it is better to keep it in the current serving satellite operating in Normal mode.

  - Alt2 (New): If the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is set as the time when the satellite 2 starts to operate in Normal mode, e.g., T5, this would help to delay the switching to satellite 2 for the UEs (both legacy UEs and R19 UEs) and keep them in the current serving satellite 1 with Normal mode for as long as possible.

  - Hence, it’s preferred that the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of neighbor satellite could be set as the time when the neighbor satellite starts to operate in Normal mode. This timing relationship and the corresponding parameter setting can be seen in the following table:

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStartNeigh for Sat-2 in SIB33 |
| --- | --- | --- | --- | --- |
| T1: S&F mode |  |  |  |  |
| T2: Normal mode |  |  |  |  |
|  | T4: Start serving the area with S&F mode |  |  | Legacy: (too early) |
|  | T5: Normal mode |  |  | New: |
| T3: Stop serving |  | Legacy: |  |  |

- Case#2: Under the continuous coverage scenario, it’s also possible that the neighbor satellite will switch to Normal mode at a bit late time, e.g., after the serving satellite stops serving the area, then the timing relationship may be that T5 > T3 > T4. With similar analysis as above, in this case, for quasi-earth fixed cell, if still using the Alt2 in Case#1, that is, to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 as the time when the neighbor satellite starts to operate in Normal mode, e.g., T5, it may cause UEs to lose the coverage for a while (from T3 to T5). We think this is kind of too late switching to satellite 2, e.g., worse than camping on Sat-2 but not connecting normally. Hence, the more suitable process for Case#2 is to follow the legacy setting way, e.g., the t-ServiceStartNeigh is set as the time when the neighbor satellite starts serving the area, e.g., T4. This timing relationship and the corresponding parameter setting can be seen in the following table:

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStartNeigh for Sat-2 in SIB33 |
| --- | --- | --- | --- | --- |
| T1: S&F mode |  |  |  |  |
| T2: Normal mode |  |  |  |  |
|  | T4: Start serving the area with S&F mode |  |  | Legacy: |
| T3: Stop serving |  | Legacy: |  |  |
|  | T5: Normal mode |  |  | New: (too late) |

  - For R19 UE, an enhancement can be further considered, that is, a new time information when a neighbor satellite transits from S&F operation mode to Normal mode (e.g., with example name as t-SFtoN-Neigh for Sat-2 in SIB33) could be provided. Based on this new information, R19 UE determine the more suitable time when it can set up connection for real-time/response-needed services or some NAS processes. The setting for suggested new parameter can be seen in the last column in following table:

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStartNeigh for Sat-2 in SIB33 | New para t-SFtoN-Neigh for Sat-2 in SIB33 |
| --- | --- | --- | --- | --- | --- |
| T1: S&F mode |  |  |  |  |  |
| T2: Normal mode |  |  |  |  |  |
|  | T4: Start serving the area with S&F mode |  |  | Legacy: |  |
| T3: Stop serving |  | Legacy: |  |  |  |
|  | T5: Normal mode |  |  | New: (too late) |  |
|  |  |  |  | All the UEs can begin to measure the Sat-2 at T4 and may camp on Sat-2. But they may not be able to set up connection normally. | R19 UEs can measure and camp on Sat-2 at T4 but delay setting up connection until this time T5 (t-SFtoN-Neigh). |

- Case#3: Under the discontinuous coverage scenario, the only case is that the neighbor satellite switches to Normal mode after the serving satellite stops serving the area, so the timing relationship is that T5 > T4> T3. After the discontinuous coverage, it is likely that the registration or TAU process is triggered by UE. In order to ensure NAS process can be completed successfully at the first time for all the UEs, it is beneficial to set the t-ServiceStart in SystemInformationBlockType32(-NB) as the time when the neighbor satellite not only provides coverage but also operates in Normal mode, e.g., T5 (New), not setting the T4 when the neighbor satellite just starts serving the area but without the feeder link (as Legacy). Since the coverage anyway will lose after T3, it doesn't make much difference to the UE whether it connects to the next neighbor satellite earlier or later. Meanwhile, it’s better to ensure the UE can operate normally after camping the neighbor satellite.

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStart for Sat-2 in SIB32 |
| --- | --- | --- | --- | --- |
| T1: S&F mode |  |  |  |  |
| T2: Normal mode |  |  |  |  |
| T3: Stop serving |  | Legacy: |  |  |
|  | T4: Start serving the area with S&F mode |  |  | Legacy: (too early) |
|  | T5: Normal mode |  |  | New: |

Based on the above analysis, we have the following observation:

Observation 1: A scenario where satellites operating in S&F mode is going to Normal mode generally corresponds to that satellites move from an area without a ground station to an area with a ground station. In this scenario, the serving satellite typically is in Normal mode before it stops serving the areas. Meanwhile, neighboring satellites are likely in S&F mode when they start to cover the area. Therefore, the connection to serving satellite can be maintained as much as possible. Then the stop time of serving satellite can remain as per legacy settings, no need of optimization. At the same time, since the neighbor satellites generally have no connection to ground station when they start to cover the area, a possible enhancement for NW implementation is to set a later start time for the neighbor satellites in SIB33, e.g., at start time of Normal mode, allowing the UE to camp neighboring satellites a bit late till it can work normally.

Moreover, in the case of discontinuous coverage, the connection to serving satellite can be maintained as much as possible, so the stop time of serving satellite can remain as per legacy setting. Meanwhile, also considering neighbor satellites generally have no connection to ground station when they start to cover the area, a possible enhancement for NW implementation is also to set a later start time for the neighbor satellites in SIB32.

Please note, the above mentioned possibly optimized setting for the existing time parameters in SIB33/SIB32 can also be beneficial to the legacy UEs.

From specification perspective, for quasi-earth fixed cell, instead of always setting the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) as the time when the neighbor satellite starts serving the area, the following proposals are given:

Proposal 2a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) either the time when the neighbor satellite starts to operate in Normal mode or the time when the neighbor satellite starts serving the area.

Proposal 2b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

In additional, a new time information when a neighbor satellite transits from S&F operation mode to Normal mode can be provided via SIB33 (e.g., t-SFtoN-Neigh for Sat-2 in SIB33). If this time is present, for R19 UE, it is up to R19 UE’s implementation to delay setting up connection under the neighbor satellite till this time (e.g., later than the t-ServiceStartNeigh in SystemInformationBlockType33(-NB)).

Proposal 2c: It’s suggested to introduce a new time information when a neighbor satellite transits from S&F operation mode to Normal mode for each neighbor satellite in SystemInformationBlockType33(-NB).

Scenario2: the serving satellite operating in Normal mode is going to switch to S&F mode

Fig.2

As illustrated in Fig.2, for quasi-earth fixed cell, the serving satellite 1 serves the area and works at Normal mode at T1. It is going to switch to S&F mode at T2 and stop serving the area at T3. And the next neighbor satellite 2 is going to start serving the area at T4 with available feeder link.

- Case#1: Under the continuous coverage scenario, for quasi-earth fixed cell, the t-ServiceStartNeigh of satellite 2 in SystemInformationBlockType33(-NB) is generally set as the time when the neighbor satellite is going to start serving the area, e.g., T4. After the serving satellite switches to S&F mode, the neighbor satellite starts serving the area with available feeder link, then the timing relationship is that T3 > T4 > T2. In this case, for quasi-earth fixed cell, if the t-Service in SystemInformationBlockType3(-NB) that can be used by UE to decide when to start measurements on neighbor satellite is set as the time when the serving satellite starts to operate in S&F mode, e.g., T2 (New), instead of T3 (as Legacy), for all the UEs, it is helpful to trigger the UEs to early reselect to a neighbor satellite with Normal mode. Hence, in this case, it’s better to set t-Service as the time when serving satellite starts to operate in S&F mode.

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStartNeigh for Sat-2 in SIB33 |
| --- | --- | --- | --- | --- |
| T1: Normal mode |  |  |  |  |
| T2: S&F mode |  | New: |  |  |
|  | T4: Start serving the area with Normal mode |  |  | Legacy: |
| T3: Stop serving |  | Legacy: |  |  |
|  | T5: S&F mode |  |  |  |

- Case#2: In another continuous coverage scenario, it’s also possible that T3 > T5 > T4 > T2, similar suggestion as that in Case#1 for setting the t-Service for Sat-1 in SIB3 can be applied.

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStartNeigh for Sat-2 in SIB33 |
| --- | --- | --- | --- | --- |
| T1: Normal mode |  |  |  |  |
| T2: S&F mode |  | New: |  |  |
|  | T4: Start serving the area with Normal mode |  |  | Legacy: |
|  | T5: S&F mode |  |  |  |
| T3: Stop serving |  | Legacy: |  |  |

- Case#3: Under the discontinuous coverage scenario, timing relationship can be that T4 > T3 > T2. The t-ServiceStart in SystemInformationBlockType32(-NB) is generally set as the time when the neighbor satellite is going to start serving the area, e.g., T4. Meanwhile, in order to keep the UE in the coverage of satellite as much as possible, it may be no longer suitable to use the new setting way, e.g., to set the t-Service of serving satellite as the time when the serving satellite starts to operate in S&F mode, e.g., T2. This may cause the UE to stop the AS layer process too early. Instead, the more suitable way is to follow legacy way, e.g., to still set the t-Service in SystemInformationBlockType3(-NB) as the time when the serving satellite stops serving the area, e.g., T3.

| Serving satellite (Sat-1) | A neighbor satellite (Sat-2) | t-Service for Sat-1 in SIB3 | New para t-SFtoN for Sat-1 in SIB31 | Setting for t-ServiceStart for Sat-2 in SIB32 |
| --- | --- | --- | --- | --- |
| T1: Normal mode |  |  |  |  |
| T2: S&F mode |  | New: |  |  |
| T3: Stop serving |  | Legacy: |  |  |
|  | T4: Start serving the area with Normal mode |  |  | Legacy: |
|  | T5: S&F mode |  |  |  |

  - For R19 UE, an enhancement can be further considered, that is, a new time information when the serving satellite starts to operate in S&F mode (e.g., T2) can be provided to the R19 UE to facilitate R19 UE to stop some services at an earlier time point (even the coverage still exist till T3). Moreover, if Proposal 1 can be agreed, this time when the serving satellite starts to operate in S&F mode could be deduced by “start of Normal mode + the duration of Normal mode”, so no new parameter is needed.

Based on the above analysis, we have the following observation:

Observation 2: A scenario where satellites operating in Normal mode is going to switch to S&F mode generally corresponds to that satellites move from an area with a ground station to an area without a ground station. In this scenario, the serving satellite typically is in S&F mode before it stops serving the areas. Meanwhile, neighboring satellites are likely in Normal mode when they start to cover the area. Therefore, a possible enhancement for NW implementation is to set an earlier stop time for the serving satellite, e.g., at start time of S&F mode, allowing the UE to measure neighboring satellites also earlier. At the same time, since the neighbor satellites generally can connect to ground station when they start to cover the area, their start time can remain as per legacy settings, no need of optimization.

Moreover, in the case of discontinuous coverage, the connection to serving satellite can be maintained as much as possible, so the stop time of serving satellite can remain as per legacy setting. Meanwhile, the start time of neighbor satellites also can remain as per legacy setting as they generally can connect to ground station when they start to cover the area.

Please note, the above mentioned possibly optimized setting for the existing time parameters in SIB3 can also be beneficial to the legacy UEs.

From specification perspective, for quasi-earth fixed cell, instead of always setting the t-Service in SystemInformationBlockType3(-NB) as the time when the serving satellite stops serving the area, the following proposal is given:

Proposal 3a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-Service in SystemInformationBlockType3(-NB) the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

Proposal 3b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: Besides the agreed time information on transition from S&F operation mode to Normal mode, a duration of Normal mode could be provided to R19 UE, e.g., via SystemInformationBlockType31(-NB) message.

Proposal 2a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) either the time when the neighbor satellite starts to operate in Normal mode or the time when the neighbor satellite starts serving the area.

Proposal 2b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Proposal 2c: It’s suggested to introduce a new time information when a neighbor satellite transits from S&F operation mode to Normal mode for each neighbor satellite in SystemInformationBlockType33(-NB).

Proposal 3a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-Service in SystemInformationBlockType3(-NB) the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

Proposal 3b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Reference

[1] RP-250472, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. March, 2025

[2] TS 23.401, Evolved Universal Terrestrial Radio Access Network, (E-UTRAN) access (Release 19) V19.1.0, December 2024