3GPP TSG-RAN WG2 Meeting #129bis         	  R2-2501819

Wuhan, China, Apr. 7th – 11th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on UL capacity enhancements in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest one is approved in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous RAN2 meetings, RAN2 has discussed some general aspects for this objective and the feasibility of contention-based Msg3 transmission, and also gave comparison among different anti-collision schemes. The related RAN2 agreements are as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition RAN2#127b agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN. RAN2#128 agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started) RAN2#129 agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - - Frequency domain resources for (N)PUSCH occasions - - repetition number - - (N)PDCCH resource - - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature - Working assumption: - One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today - The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. - RAN2 assumes that a pointer solution is not needed in Rel-19

In this document, we will firstly give analysis on the remaining issues of Msg4 transmission, and further give analysis on remaining issues for supporting DSA schemes. Based on the analysis, some proposals are given.

Discussion

The configuration for CB-msg3 transmission

Compared with CB-msg3 without OCC, the decoding of CB-msg3 with OCC should rely on the enhanced DMRS and OCC sequence. If a shared resource pool is configured for CB-msg3 without OCC and CB-msg3 with OCC, the network needs to take considerable efforts on blind detection, which significantly increasing its complexity. Hence, a separate resource pool needs to be configured for CB-msg3 with OCC. Although only UEs supporting OCC can use the resource for CB-msg3 with OCC and couldn’t report the UE capability before the CB-msg3 procedure, the eNB can adjust the size of this resource based on its conflict situation. Besides, considering the flexible implementation of the network, only a resource pool for CB-msg3 with OCC could be configured. In this case, if the condition of CB-msg3 with OCC is satisfied, UE is able to utilize this resource pool for access. Otherwise, the PRACH procedure is preferred as an alternative means of access.

Proposal 1: A separate resource pool can be configured for CB-msg3 with OCC.

According to the information from RAN1, if two UEs without power imbalance select different OCCs and share the same time-frequency resources, the better gain could be achieved in the receiver. Conversely, if two UEs with power imbalance select different OCCs, and share the same time-frequency resources, the gain might decrease. As a result, the RSRP range was suggested to control the power imbalance in last meeting. However, setting a single RSRP range will greatly limit the number of UEs using CB-msg3 with OCC and reduce its usage rate, as the number of UEs meeting the RSRP range might be very small. In order to expand usage scenarios for CB-msg3 with OCC, it's advisable to set multiple RSRP ranges. Further, for UEs in two different coverage area, due to different transmission power, they may cause different levels of interference to UEs who select the same time and frequency resource in the receiver. For example, compared with UE with -140dBm RSRP, UE with -70dBm RSRP may cause more interference to UEs who select the same time and frequency resource. As a result, in order to suit different coverage area, the separate RSRP ranges can be flexibly configured for a CE level.

Proposal 2a: Multiples RSRP range could be introduced to control power imbalance for CB-msg3 with OCC. And the separate RSRP ranges can be flexibly configured for a CE level.

In additional, in order to enable UEs in the same RSRP range to select the same time and frequency resource, a RSRP range could link to a or multiple specific frequent occasions. For example, assuming there are 2 frequent occasions, a RSRP range {-100, -95} links to a frequency occasion, and a RSRP range {-95, -90} links to another.

Proposal 2b: A RSRP range could link to a or multiple frequent occasions for CB-msg3 with OCC.

Efficient delivery of Msg4

Contention resolution window

Taking legacy contention-based RA procedure or contention-based EDT procedure as reference, in order to guarantee the reception of Msg4, a contention resolution window is necessary.

Per our understanding, CB-msg3 will use similar Msg3 on (N)PUSCH and Msg4 transmission on (N)PDSCH (without Msg1 and Msg2) as that in PUR and the main difference is that CB-msg3 needs an additional contention procedure, so we assume the definition of Msg3 response window in PUR can be referred when defining the contention resolution window for CB-msg3.

The start time of the Msg3 response window in PUR is determined based on some factors, e.g., the RTT, the processing time of Msg3. Taking the following descriptions for NB-IoT NTN from RAN1 and RAN2 specs as reference, it’s suggested the contention resolution window of CB-msg3 in NB-IoT NTN can be also started at the end of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

TS 36.213: - 9.1.5 MPDCCH assignment procedure //skip unrelated parts If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts - 16.6 Narrowband physical downlink control channel related procedures //skip unrelated parts If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts TS 36.321: 5.4.7.1 Transmission using PUR //skip unrelated parts After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes. //skip unrelated parts

Proposal 3: For CB-msg3 procedure, a contention resolution window starts at the end of the Msg3 transmission plus 4 subframes plus UE-eNB RTT.

For the length of contention resolution window, we assume the length of Msg3 response window in PUR can be referred. The value of pur-ResponseWindowTimer are as below and seems long enough:

| for eMTC | pur-ResponseWindowTimer-r16 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240} |
| --- | --- |
| For NB-IoT | pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} |

Hence, we suggest the length of contention resolution window CB-msg3 could reuse that of the pur-ResponseWindowTimer.

Proposal 4: The same value range as pur-ResponseWindowTimer can be reused for the length of contention resolution window for CB-msg3 procedure.

CB-Msg4 transmission

Similar as that in legacy contention-based procedures, the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after reception of Msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 5: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 procedure, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

In the procedure for transmission using PUR for CP solution, L1 ACK and L2 ACK (e.g., TA command MAC CE) can be used to early terminate the procedure if there is no data to be sent to the UE. Similarly as PUR, for CB-msg3 for CP solution, upon the reception of the CB-msg3, the network is able to determine that there is no DL data for UE, thereby it can choose to terminate the procedure efficiently. Moreover, considering that the UE Contention Resolution Identity is essential to resolve the collision for msg4, a UE Contention Resolution Identity can be used as L2 ACK to terminate the CB-msg3 procedure.

Proposal 6: For CB-msg3 for CP solution, a UE Contention Resolution Identity can be used to early terminate the procedure.

Moreover, in legacy RACH procedure, after the reception of msg3, the network could send UE Contention Resolution Identity and RRC message together or separately. This allows the network to resolve the contention and complete the RRC procedure in two steps. The advantage of doing so is that the network can flexibly decide when to send the subsequent RRC message based on the arrival of data packets. For the same reason, the similar procedure could be adopted for CB-msg3 procedure. As a result, for CB-msg3 procedure, according to the information that needs to be carried in msg4, the potential procedures could be categorized into three cases as below:

Figure 1. the procedures for CB-msg3/msg4 procedure

After transmission of CB-msg3 carrying a RRC message, such as RRCEarlyDataRequest message, the following cases may occur:

- For case 1, upon the reception of a matching UE Contention Resolution Identity, UE could terminate the RRC procedure. This case is applicable to the scenario that there is no DL data.

- For case 2, upon the reception of a matching UE Contention Resolution Identity, UE could stop the contention resolution window, consider this contention resolution successful, wait for the subsequent RRC message and then complete the RRC procedure according to the received RRC message, such as RRCEarlyDataComplete, RRCConnectionReject, or RRCConnectionSetup.

- For case 3, upon the reception of a UE Contention Resolution Identity and RRC message (such as RRCEarlyDataComplete, RRCConnectionReject, RRCConnectionSetup), UE could stop the contention resolution window, consider this contention resolution successful and complete the RRC procedure according to the RRC message.

From the perspective of the network, it is flexible to support those cases.

Proposal 7: For Msg4 of CB-msg3 procedure, UE Contention Resolution Identity and RRC message could be send together or separately.

However, UE couldn’t distinguish the case 1 and case 2 when it receives the UE Contention Resolution Identity as it has no idea whether there will be following RRC message. Since for case2, a C-RNTI is needed to monitor the PDCCH for the following RRC message, it can be considered to let eNB provide a C-RNTI to the UE. This assigned C-RNTI can serve as an indication to let UE know that RRC procedure cannot be terminated and needs to continue. For example, for CB-msg3 procedure, if the Msg4 includes a matching Contention Resolution Identity and a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following RRC message by using the assigned C-RNTI. In some other cases, if the Msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminates the RRC procedure.

By this way, it can also address the issue of which RNTI to be used for monitoring the PDCCH after successful contention resolution for CB-msg3. In legacy RA procedure, upon successful contention resolution, UE will set the C-RNTI to the value of the Temporary C-RNTI which is assigned in Random Access Response message by the eNB. This C-RNTI will be used after successful contention resolution. However, for CB-msg3, there is no assigned Temporary C-RNTI as no Msg1 and Msg2 exchanging. There may be thinking that the RNTI derived based on the resource associated to the (N)PUSCH occasion for CB-msg3 transmission can be taken as C-RNTI. However, we think this is not a general way. By referring to the legacy RA procedure and also fallback case of PUR, we think it’s more suitable to let eNB assign a new C-RNTI to the UE if there will be further RRC signaling to be addressed.

Proposal 8a: For CB-msg3 procedure, if the Msg4 includes a matching Contention Resolution Identity and also a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following scheduling by using the assigned C-RNTI.

Proposal 8b: For CB-msg3 procedure, if the Msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminate the RRC procedure.

Multiplexing of Msg4

In last meeting, a work assumption that tends to agree the multiplexing of multiple Msg4 into a MAC PDU was agreed. First of all, UEs whose Msg4 are multiplexed in the same MAC PDU should monitor the same RNTI. According to the potential parameters in the calculation of RNTI, such as the time occasion, frequency occasion and OCC sequence, the Msg4 that can be multiplexed into a MAC PDU should target for UEs that transmit CB-msg3 in the same time occasion (in the different frequency occasions) or in the same time occasion and frequency occasion (using different OCC sequences). To be special, the analysis is as follows:

- If the Msg4 of the UEs that transmit CB-msg3 in the same time occasion and frequency occasion are all multiplexed into the same MAC PDU, considering the 2 available OCC sequences, the number of multiplexed Msg4 in a MAC PDU could be 2.

- If the Msg4 of the UEs that transmit CB-msg3 in the same time occasion are multiplexed into the same MAC PDU, due to many frequency occasions, many MAC PDUs for Msg4 may be generated. For example, if the number of available frequency occasion is 8 (same as PRACH), the number of available OCC sequences is 2, there are at most 16 Msg4 for all UEs transmitting in the same time occasion. Considering of the limitation of UE’s buffer, these 16 Msg4 are divided to generate multiple MAC PDUs. Each MAC PDU can contain several multiplexed Msg4, and multiple MAC PDUs can be transmitted separately. Especially, if the Msg4 carries large bits, such as RRC setup message, more MAC PDUs may be generated. In this case, for a UE, due to a shared RNTI, it has to decode all MAC PDUs until the decoded UE contention resolution identity carried in MAC PDU matches its own CCCH. This process inevitably leads to increased power consumption. Hence, considering of the great complexity, this case is not recommended.

Proposal 9a: For CB-msg3 procedure, if the OCC is enabled, the multiplexing of Msg4 could be supported.

Proposal 9b: For CB-msg3 procedure, if the multiplexing of Msg4 is supported, the Msg4 of the UEs that transmit CB-msg3 in the same time occasion and frequency occasion could be multiplexed into the same MAC PDU.

If the Msg4 of multiple UE are multiplexed into the same MAC PDU, the primary issue is how to let the UEs know which MAC CEs or MAC SDUs in the MAC PDU are intended for it. There are two solutions to resolve this issue:

- Alt1: introducing MAC subPDU in NR: a MAC PDU consists of one or more MAC subPDUs. Each subPDU could target to a UE, and include MAC subheader, MAC CE or MAC SDU. However, the new MAC subPDU format will introduce additional complexity. It requires UE to decode different MAC formats for different access processes. For example, for CB-msg3 procedure and PUR procedure, UE needs to decode different MAC formats for Msg4.

- Alt2: reusing the current MAC PDU but introducing the new rule: as above Proposal 5, a UE Contention Resolution Identity is essential to resolve the collision in Msg4. For the multiplexing of Msg4, the UE Contention Resolution Identity also could be reused to differentiate the MAC CE or MAC SDU targeting for different UE in a MAC PDU. To be specially, if the msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE. For a UE, as long as a decoded contention resolution identity carried in MAC PDU matches its own CCCH, the following MAC CE or MAC SDU until the next contention resolution identity or the end also belongs to this UE. As illustrated in Figure 2, two msg4 are multiplexed in a MAC PDU. For UE#1, if UE Contention Resolution Identity MAC Control Element#1 matches, MAC SDU#1 is targeted for it.

| MAC subheader #1-1 | MAC subheader #1-2 | MAC subheader #2-1 | MAC subheader #2-2 | UE Contention Resolution Identity MAC Control Element#1 | MAC SDU#1 | UE Contention Resolution Identity MAC Control Element#2 | C-RNTI MAC Control Element#2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 2. MAC PDU

In final, Alt2 has better compatibility and low complexity.

Proposal 10: For CB-msg3 procedure, if the msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE.

RNTI calculation

In previous meeting, RAN2 has agreed that RNTI for scheduling Msg4 transmission is derived based on the resource associated to the (N)PUSCH occasion used for CB-msg3 transmission.

According to our analysis in 2.1, the resource associated to the (N)PUSCH occasion can include the time domain resource, frequency domain resource and OCC resource. As above in 2.2.3, it can be seen that the multiplexing of Msg4 is closely related to the calculation of RNTI of Msg4. If the msg4 for multiple UEs is multiplexed together, the required same RNTI means that the RNTI calculation does not consider the frequency occasion or frequency occasion and OCC sequence. If Proposal 8 is agreed, the OCC resource isn’t counted into the calculation of RNTI.

In below, we will discuss one by one whether and how these resource information should be used in the RNTI calculation:

- Regarding the time domain resource, the calculation of RNTI has a close relationship with the length of contention resolution window. If UEs select different transmission occasions in time domain but use the same RNTI, and monitor the same RNTI within the overlapped contention resolution window, they cannot correctly distinguish the Msg4 via RNTI. Therefore, it should ensure that, within the length of a contention resolution window, for two UEs who select two transmission occasions in different periodicity, RNTI calculated by the UEs should be different. Due to the periodicity will be with the unit of tens of or hundreds of milliseconds, the subframe will keep unchanged in different periodicities and just SFN is used to distinguish the transmission occasion. Hence, SFN index should be used as a factor in the formula of RNTI, meanwhile, subframe index is not needed.

  - Besides, according to some previous discussion for RA-RNTI, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved. On one hand, since resources are configured according to periodicity, not all SFN indices will be used; that is, only SFN indices at certain intervals will be considered valid resource positions during resource selection. Hence, SFN index can be divided by a minimum periodicity in the formula. On the other hand, since each UE only monitors the DL for a limited time period, e.g., during the contention resolution window, to receive Msg4, the RNTIs of two UEs need only be different during this period. Hence, it also can be considered to modulo the SFN index by the length of contention resolution window.

- Regarding the frequency resource, for eMTC mode A, a UL resource allocation includes the start PRB index, PRB number. As long as UEs select the different start PRB index, Msg3 from UEs will not conflict. Hence, the selected start PRB index could be used as a parameter in the formula of RNTI. Meanwhile, for NB-IoT, a UL resource allocation includes subcarrier set index, RU number and carrier index. As long as UEs select the different subcarrier set and carrier, Msg3 from UEs will not conflict. Hence, the subcarrier set index and carrier index could also be considered in the formula of RNTI.

Based on the above discussion, the following proposals are given for RNTI calculation:

Proposal 11a: The following parameters can be taken into account in the RNTI calculation for CB-msg3:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: start PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

Proposal 11b: For the RNTI calculation for CB-msg3, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Backoff/Fallback schemes

Due to skipping Msg1/Msg2 transmission, if the channel quality is not stable enough, the Msg3/Msg4 transmission may face more failures. In UE side, such failure in RRC layer would be informed to NAS and NAS may trigger another access after a while. In order to avoid triggering higher layers to perform retransmissions which may increase the delay, it can be considered to prioritize remediation at the AS layer.

Generally, two schemes for improving the performance robustness in AS layer, e.g., Backoff and Fallback, can be con.sidered.

Regarding Backoff, it mainly refers to, upon failure occurrence, retriggering the same procedure after a certain time interval. Therefore, for CB-msg3, when the collision or congestion is detected by eNB on a certain resources occasion, the eNB can provide kind of backoff indication via the Msg4 to require backoff.

By using backoff scheme:

- On one hand, the UE can have additional opportunity to retransmit another CB-msg3, e.g, direct Msg3/Msg4 transmission without Msg1/Msg2, rather than immediately falling back to the 4-step access procedure. This is friendly and reasonable for the UE, as the previous failure was likely due to the selected resources by this UE is conflicted with another UE's choice. If given another chance to select resources, it would increase the likelihood of successful transmission.

- On the other hand, even UE is provided with another transmitting opportunity, UE still needs to delay a backoff time, then can choose another time domain resources within the shared resources for Msg3 transmission. This is also be helpful to avoid collisions possibly caused by another direct Msg3/Msg4 transmission. Here we think the range of backoff given/configured by the network would be important as network can suitably provide this based on the real collision situation.

- Furthermore, for the OCC-based SA scheme, with combined with Backoff scheme, it may have similar benefit as DSA that is brought by more transmission opportunities. But OCC-based SA + Backoff may be better, as in DSA, multiple replicas are always needed. Meanwhile, for OCC-based SA + Backoff, the additional transmission only occurs (may be seldom) when the previous transmission fails. So OCC-based SA + Backoff scheme can achieve a better balance between more transmission opportunities and reduced resource occupation.

Observation 1: OCC-based SA can be combined with Backoff scheme and it may have similar benefit as DSA or may be better than it as the additional transmission is also allowed but only occurs (may be seldom) when the previous transmission fails.

Proposal 12: RAN2 is suggested to support backoff scheme for CB-msg3. That is, when CB-msg3 fails, after a backoff time indicated by eNB, UE can choose an occasion to perform another CB-Msg3 transmission. FFS details.

If multiple attempts of the CB-msg3 also fail, it’s highly possible due to the bad channel quality of UE or the serious congestion on the current CB-msg3 resources. In order to ensure the successful access, the straightforward way is to select another resource to access. The candidate access resource could be the following:

- The resource of CB-msg3 in other carrier. The reselection leads to the carrier selection. For NB-IoT UE, the carrier selection needs a long switching time. Hence, it is not recommended to select to other carrier during access procedure.

- The resource of CB-msg3 in higher CE level. For the higher CE level, the requirement of received SINR decreases, thereby increasing the probability of the successful CB-msg3 reception. Moreover, due to the possibly larger configured TBS for higher CE level, the condition that the size of the MAC PDU of msg3 is smaller than or equal to the configured TBS is easier to be satisfied. Hence, fallback to higher CE level is feasible.

- The resource of 4-step PRACH. In fact, the decoding requirements of CB-msg3 are more stringent than PRACH. Even the deterioration of channel quality of the UE leads to the failed reception of CB-msg3, the reception condition of PRACH may still be satisfied. Hence, switching to RACH based procedure could increase the successful access probability. Hence, fallback to RACH based EDT procedure is feasible.

In a summary, if the CB-msg3 transmissions fail, fallback to higher CE level, or RACH based EDT procedure could be considered. The former has the advantage of shorter access time of CB-msg3. In contrast, the latter has the advantage of higher probability of successful access. In order to achieve balance between the access delay and access success probability, a stepped fallback mechanism can be considered. For example, if multiple transmissions of the CB-msg3 in a given CE level fail, firstly a higher CE level can be selected and CB-msg3 is triggered in this level. Until the CB-msg3 transmissions in highest CE level fail, a RACH based EDT procedure is triggered.

Proposal 13: For CB-msg3 procedure, a stepped fallback to higher CE level and then to RACH based EDT procedure can be considered.

DSA

RNTI calculation

For DSA, according to the discussion in last meeting, two alternatives as following can be considered for RNTI calculation:

- A specific RNTI for each replica: In DSA, UE could transmit multiple replicas on multiple time occasions. The RNTI can only be derived based on the transmission occasion for the corresponding replica. For example, in below figure 3, for DSA with 2 replicas, if eNB receives replica 1 and response Msg4, the RNTI 1 calculated based on SFN 4 is used for scrambling Msg4. if eNB receives replica 2 and response Msg4, the RNTI 2 calculated based on SFN 10 is used for scrambling Msg4.

- A common RNTI for different replicas: To avoid the complexity of the UE monitoring multiple RNTIs, in previous meeting, companies suggested to design a common RNTI related with the selected transmission window that corresponds to multiple different replicas. In this way, in a transmission window (usually it could contain multiple available transmission occasions), the UEs who select the same frequency occasion and different time occasions share a common RNTI. For example, for 4 replica, in a transmission window, there are 5 time occasions at least, 8 frequency occasions in a time occasion and 2 OCC sequence. In this case, there may be 32 UEs sharing a same RNTI. On the one hande, those UEs have to decode a large number of MAC PDU within the contention resolution window until they identity a matching UE Contention Resolution Identity. This exhaustive decoding process significantly increases UE power consumption. On the other hand, if eNB schedules retransmission by PDCCH, since the UEs cannot distinguish between different PDCCHs, all UEs that receives the PDCCH will retransmit. Due to these issues, a predefined common RNTI isn’t feasible.

In summary, for DSA, the common RNTI is infeasible. Hence, a specific RNTI for each replica is proposed. For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Proposal 14: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

UE capability

From the above discussion, it is evident that DSA represents an enhanced and more complex optimization function compared to SA. Hence, a UE capability to support DSA could be introduced. If a UE supports DSA, then it must necessarily support SA. Besides, if UE supports DSA, all functions to support DSA, including selecting the transmission window, transmitting multiple replicas and multiple contention resolution window, are supported. This capability could be reported to eNB, according to this, eNB could enable or disable the DSA function in the cell.

Proposal 15: A UE capability to support DSA can be introduced for CB-msg3 procedure.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: A separate resource pool can be configured for CB-msg3 with OCC.

Proposal 2a: Multiples RSRP range could be introduced to control power imbalance for CB-msg3 with OCC. And the separate RSRP ranges can be flexibly configured for a CE level.

Proposal 2b: A RSRP range could link to a or multiple frequent occasions for CB-msg3 with OCC.

Proposal 3: For CB-msg3 procedure, a contention resolution window starts at the end of the Msg3 transmission plus 4 subframes plus UE-eNB RTT.

Proposal 4: The same value range as pur-ResponseWindowTimer can be reused for the length of contention resolution window for CB-msg3 procedure.

Proposal 5: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 procedure, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 6: For CB-msg3 for CP solution, a UE Contention Resolution Identity can be used to early terminate the procedure.

Proposal 7: For Msg4 of CB-msg3 procedure, UE Contention Resolution Identity and RRC message could be send together or separately.

Proposal 8a: For CB-msg3 procedure, if the Msg4 includes a matching Contention Resolution Identity and also a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following scheduling by using the assigned C-RNTI.

Proposal 8b: For CB-msg3 procedure, if the Msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminate the RRC procedure.

Proposal 9a: For CB-msg3 procedure, if the OCC is enabled, the multiplexing of Msg4 could be supported.

Proposal 9b: For CB-msg3 procedure, if the multiplexing of Msg4 is supported, the Msg4 of the UEs that transmit CB-msg3 in the same time occasion and frequency occasion could be multiplexed into the same MAC PDU.

Proposal 10: For CB-msg3 procedure, if the msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE.

Proposal 11a: The following parameters can be taken into account in the RNTI calculation for CB-msg3:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: start PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

Proposal 11b: For the RNTI calculation for CB-msg3, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Proposal 12: RAN2 is suggested to support backoff scheme for CB-msg3. That is, when CB-msg3 fails, after a backoff time indicated by eNB, UE can choose an occasion to perform another CB-Msg3 transmission. FFS details.

Proposal 13: For CB-msg3 procedure, a stepped fallback to higher CE level and then to RACH based EDT procedure can be considered.

Proposal 14: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Proposal 15: A UE capability to support DSA can be introduced for CB-msg3 procedure.

Reference

- RP-243278, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#106, December 2024