3GPP TSG-RAN WG2 Meeting #129bis	R2-2501823

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.3.3

Source:	OPPO

Title:	Simulation results of measurement event prediction

Document for:	Discussion, Decision

Introduction

At RAN2#129, at-meeting email discussion reaches the following agreements.

- For evaluation purpose, it is assumed PW length = TTT length =320ms

- For evaluation purpose, Option 2 is agreed i.e. each counter can be updated only one time for related predicted and/or ground truth measurement event.

- To introduce “Total number of HOF per UE per second” as SLS metric

To make the simulation scenario more focused, we add one more metric for measurement event prediction for temporal domain case A i.e. the timing distance between 1st time instance of positive true prediction and corresponding ground truth label. Without it, there will be no difference between a powerful model e.g. 1stpositive true upon entry condition and weak model e.g. 1st positive true 80ms immediately before ground truth label

Accordingly, we have revised our simulations on measurement event prediction.

Discussion

Measurement event prediction based on FR2 temporal domain case A

For Measurement event prediction based on temporal domain case A, two options for handover decisions are available:

Agreement: For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

Note: In TR 38.744, handover decisions option 2 and option 3 here are renamed as handover model option 1 and handover model option 2.

The table below provide a detailed comparison of precision, recall, and F1 scores for the two methods used in prediction measurement event case A. Prediction starts when the enter condition of measurement event satisfies.

Table 1: Intermediate results of measurement event case A

| Metrics \ Method | Option 2 (handover model option 1 in TR) | Option 3 (handover model option 2 in TR) |
| --- | --- | --- |
| Precision | 0.96 | 0.97 |
| Recall | 1 | 1 |
| F1 | 0.98 | 0.99 |
| Average L3 cell RSRP difference (dB) | 0.69 |  |
| Last predicted point L3 cell RSRP difference (dB) | 1.23 |  |
| Timing distance between 1st time instance of positive true prediction and corresponding ground truth label | 320ms |  |

With a relatively short PW of 320ms and the condition to trigger predictions at the start of TTT, both options 2 and 3 achieve impressively high F1 score, nearing 1. These high F1 scores indicate that these models are effective in identifying true positives. This effectiveness is also reflected in the timing distance between the first instance of a positive true prediction and its corresponding ground truth label. The model predicts the happening of measurement events immediately upon triggering, leading to a recall value of one, meaning there is no missed even detection.

Observation 1: With a relatively short PW of 320ms and the condition for triggering predictions at the start of TTT, the prediction accuracy for measurement events based on the temporal domain Case A is remarkably good, with an F1 score approaching 1.

Observation 2: With a relatively short PW of 320ms and the condition for triggering predictions at the start of TTT, the AI/ML can predict the satisfaction of the TTT criteria as soon as they begin making predictions.

In the following table, we further check the system-level performance of these two SLS approaches and here are the results.

Table 2: System-level results of measurement event case A

| Method | HO failure rate (%) | HO attempts per UE per second | Total number of HOF per UE per second |
| --- | --- | --- | --- |
| baseline | 28.96 | 0.33 | 0.096 |
| Option 2 | 28.42 | 0.34 | 0.096 |
| Option 3 | 24.63 | 0.36 | 0.089 |

Both AI-driven options show a reduction in HO failure rates compared to the baseline method. Specifically, Option 3 demonstrates the most significant improvement with a failure rate of 24.63%, representing a decrease of 4.33 percentage points from the baseline's 28.96%. Additionally, the total number of HOF per UE per second of Option 3 is lower than that of the baseline. The improved performance of AI-driven handover mechanisms highlights the potential benefits of integrating advanced AI algorithms into network management systems. However, there is no clear difference between Option 2 and the baseline in terms of total number of HOF per UE per second.

Observation 3: AI/ML with handover decision option 3 (handover model option 2 in TR) results in a better performance than baseline (legacy) in terms of total number of HOF per UE per second.

Observation 4: AI/ML with handover decision option 2 (handover model option 1 in TR) shows a comparable total number of HOF per UE per second when compared with the legacy HO mechanism.

One possible explanation for these observations is that both Option 2 and Option 3 provide robust predictions of TTT satisfaction. However, Option 3 has a distinct advantage: it can issue the handover command as soon as the neighbouring cell's channel quality surpasses that of the serving cell. This immediate action reduces the likelihood of handover failure.

Measurement event prediction based on FR1 temporal domain case B

The previous agreement that measurement event prediction will focus on sub case 2 was initially proposed for Case A. However, when companies attempt to implement Case B, an issue arises: L3 measurements cannot be skipped at arbitrary intervals. To address this challenge, during RAN2#128, we reached a consensus that:

3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3.

Here are the details of each filtering option:

- Filtering option 1: L3 filtering is based on its L1 filtered result and the immediate last skipped measurement result ;

- Filtering option 2: L3 filtering is based on its L1 filtered result i.e. no L3 filtering if the immediate last result is skipped;

- Filtering option 3: L3 filtering is based on the L1 filtered result and last actual measurement result i.e. the skipped result(s) in between is ignored.

Among the three filtering options for sub case 2, filtering option 1 may not be practical. Filtering option2 actually use no L3 filtering and the channel variation may deteriorate the handover performance. We believe filtering option 3 is the most practical one among the three.

In this section, we stimulated two methods for the measurement event prediction of temporal domain case B, i.e.,

- Sub case 1

- Sub case 2 with filtering option 3

The detailed analysis of intermediate results can be found in our previous paper [1]. In this paper, we would like to discuss the system-level results, especially the total number of HOF per UE per second.

Table 3: System-level results of measurement event case B

| Method | HO failure rate (%) | HO attempts per UE per second | Total number of HOF per UE per second |
| --- | --- | --- | --- |
| baseline | 35.56 | 0.045 | 0.016 |
| Sub case 1 | 36.09 | 0.045 | 0.016 |
| Sub case 2 with filtering option 3 | 41.46 | 0.044 | 0.018 |

For both Sub-case 1 and Sub-case 2, the AI/ML-based handover mechanisms exhibit either minor or no degradation in terms of the total number of HOF per UE per second compared to the baseline.

Observation 5: With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no degradation in terms of the total number of HOF per UE per second compared with the legacy HO mechanism

Conclusion

In this contribution, we have shared our simulation results on measurement event prediction and have the following observations:

Measurement event prediction based on FR2 temporal domain case A:

Observation 1: With a relatively short PW of 320ms and the condition for triggering predictions at the start of TTT, the prediction accuracy for measurement events based on the temporal domain Case A is remarkably good, with an F1 score approaching 1.

Observation 2: With a relatively short PW of 320ms and the condition for triggering predictions at the start of TTT, the AI/ML can predict the satisfaction of the TTT criteria as soon as they begin making predictions.

Observation 3: AI/ML with handover decision option 3 (handover model option 2 in TR) results in a better performance than baseline (legacy) in terms of total number of HOF per UE per second.

Observation 4: AI/ML with handover decision option 2 (handover model option 1 in TR) shows a comparable total number of HOF per UE per second when compared with the legacy HO mechanism.

Measurement event prediction based on FR1 temporal domain case B:

Observation 5: With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no degradation in terms of the total number of HOF per UE per second compared with the legacy HO mechanism

Reference

R2-2500291	, Discussion on simulation results of measurement event prediction, OPPO