3GPP RAN WG2 Meeting #129bis	R2-2501828

# Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussion on A-IoT paging

Document for:	Discussion and Decision

1. Introduction

In RAN2, the following agreements were achieved [1]:

- Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we focus on the issues related to A-IoT paging.

2 Discussion

2.1 Subsequent A-IoT Paging procedure

In order to determine whether to respond the subsequent A-IoT paging message based on the transaction ID received in different paging messages, the device needs to store the previous transaction ID and the corresponding response state for the paging message. The response status corresponding to the transaction ID indicates whether to respond the paging message with the same ID.

Proposal 1: The device needs to store the transaction ID in paging message and the corresponding response status.

For the case of paging group IDs or paging without ID, the reader cannot page the specific devices to respond. In this case, it is necessary to include a transaction ID for the same service request in the paging message to avoid duplicate responses. After receiving the paging message, if the transaction ID for the service request has not been received in the previous paging message, the device will respond to this paging. If the transaction ID has been received in the previous paging message and RA for responding the pervious paging message was unsuccessful, the device will respond to this paging. Otherwise, the device will not respond to the paging message in which the transaction ID is same as the stored one from the previous paging message.

Proposal 2: If the transaction ID is different with the stored one, the device will respond to the received paging message.

Proposal 3: If the transaction ID is same as the stored one, the device will respond to the paging message if the RA for the pervious paging message was unsuccessful.

2.2 The content in paging message

Resources related

In the last meeting, it was agreed that the A-IoT paging message can include a number of msg1 resources and R2D message(s) which determines the Msg1 resource(s) can be sent after paging message. Since there may be a case to paging large number of devices, multiple R2D messages can follow a paging message.

Proposal 4: A paging message can be followed by one or multiple R2D message(s) which determines the Msg1 resource(s).

For the paging message with one or multiple R2D messages, the detailed Msg1 resources can be included in R2D message. For one hand, the paging message should include control information that enables the device to select which resource to access, such as the number of R2D messages or the number of access occasions. In such way, the overhead of paging message can be reduced. For another hand, the paging message can include multiple set of Msg1 resources, and the R2D message indicates which set of Msg1 resources should be used in this R2D message access. Since the RAN1 will discuss the detailed resource information, whether the paging message can include multiple set of Msg1 resources can be determined by RAN1.

Proposal 5: The paging message includes the number of R2D messages or the number of access occasions.

Paging identifier related

The WID restricts a paging message contains one identifier, and a paging message contains no identifier. In the last meeting, RAN2 confirmed “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”. Therefore, one indication can be included in paging message to inform whether the paging identifier is included.

Proposal 6: The paging message includes the indication to inform whether the paging identifier is included.

There is a note in WID “RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes”. Therefore, the information to identify whether the multiple identifiers are included in paging message should be introduced.

For the detailed information to identify whether the multiple device IDs is included, the following options can be considered:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of paging message.

For the option 1, each included device ID can correspond to an indication. Based on this indication, the device could derive whether is a following device ID in the paging message. For the option 2, the device can get the number of device IDs in the paging message. For the option 3, the device can decode the device ID based the length of the device ID and length of the paging message PDU. In summary, all the above methods work well, RAN2 can discuss which option can be supported.

Proposal 7：RAN2 considers the following options to identify whether the multiple identifiers are included in the paging message for the forward compatibility:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of the paging message.

CFRA related

The scope of the WID confirms the case of a paging message contains one identifier, one identifier for a single device in the paging message may be one use case. In this case, the CFRA will be used, and subsequent paging is not needed. The situation is the same for multiple device IDs with CFRA. ID for service request used to avoiding duplicated responses is not needed for these cases, which may reduce the size of paging message or reduce the complexity and power consumption by avoiding storing ID information obtained in the paging message. Thus, the ID for service request is not mandatory for paging message.

Proposal 8: The transaction ID in the paging message can be optional for the CFRA.

For the case of paging one device, the CFRA will be used. In this case, it is simpler to send a paging message to page the device and to be used to determine the MSG1 resource(s), which can reduce one message sending and receiving. The device can respond after receiving the paging message but not to wait for the following R2D message. However, a unified solution can reduce the complexity of device implementation. So, for the CFRA, RAN2 can discuss whether the paging message including paging identifier can be used to determine the Msg1 resource(s).

Proposal 9: For the CFRA, RAN2 can discuss whether the R2D transmission which determines the Msg1 resource(s) is needed.

3 Conclusions

Proposal 1: The device needs to store the transaction ID in paging message and the corresponding response status.

Proposal 2: If the transaction ID is different with the stored one, the device will respond to the received paging message.

Proposal 3: If the transaction ID is same as the stored one, the device will respond to the paging message if the RA for the pervious paging message was unsuccessful.

Proposal 4: A paging message can be followed by one or multiple R2D message(s) which determines the Msg1 resource(s).

Proposal 5: The paging message includes the number of R2D messages or the number of access occasions.

Proposal 6: The paging message includes the indication to inform whether the paging identifier is included.

Proposal 7：RAN2 considers the following options to identify whether the multiple identifiers are included in the paging message for the forward compatibility:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of the paging message.

Proposal 8: The transaction ID in the paging message can be optional for the CFRA.

Proposal 9: For the CFRA, RAN2 can discuss whether the R2D transmission which determines the Msg1 resource(s) is needed.

4. References

[1]	RAN2 #129 meeting report