3GPP RAN WG2 Meeting #129bis	R2-2501839

# Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.7.4.2

Source:	HONOR

Title:	Discussion on DSR enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#129 meeting, the following agreements were agreed [1]:

1. One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. 2. FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. 3. We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. 4. Different LCGs may be configured with different number of reporting thresholds. 5. If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. 6. Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). 7. Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In this document, we discuss the left issues of DSR.

2 Discussion

In the PDCP running CR, the following change is captured:

A delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold as its remaining time decreases. The transmitting PDCP entity provides a delay-reporting indication for the PDCP Data PDU to lower layers each time the delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold.

It means that the PDCP needs to notify the RLC with the dsr-ReportingThreshold the data belongs to every time it changes. However, how the PDCP notifies the RLC of the delayed-reporting PDCP SDUs is up to UE implementation. It is not needed to capture the procedure of this in the specification. For example, the PDCP can provide this indication when the MAC constructs the DSR MAC CE. In this scenario, the delay-reporting PDCP SDU may not change its associated dsr-ReportingThreshold. So, it is preferred that leaving this to UE implementation.

Proposal 1: Just capture “The transmitting PDCP entity provides a delay-reporting indication for the PDCP Data PDU to lower layers” in the running CR.

In the PDCP running CR, the following is captured for the definition of “Delay-reporting PDCP SDU”,

Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order. PDU Set remaining time: the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set.

The shortest remaining time among the data of PDU set are used to define which dsr-ReportingThreshold the SDUs of PDU set belong to. And this is captured as a new term of “PDU Set remaining time” in the running CR. Since it is not agreed to introduce a new term of “PDU Set remaining time”, the definition of this term can be directly used to define “Delay-reporting PDCP SDU”. And it seems not complex to add this in the description of “Delay-reporting PDCP SDU”. Therefore, it is suggested to use the description of “shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set” to define the delay-reporting PDCP SDU of PDU set.

Proposal 2: No need to introduce “PDU Set remaining time” and just use its description to define the delay-reporting PDCP SDU of PDU set in the running CR.

3 Conclusions

Proposal 1: Just capture “The transmitting PDCP entity provides a delay-reporting indication for the PDCP Data PDU to lower layers” in the running CR.

Proposal 2: No need to introduce “PDU Set remaining time” and just use its description to define the delay-reporting PDCP SDU of PDU set in the running CR.

4. References

[1]	RAN2 #129 Chairman notes