3GPP RAN WG2 Meeting #129bis	R2-2501841

# Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.7.6

Source:	HONOR

Title:	XR rate control

Document for:	Discussion and Decision

1. Introduction

In RAN#105 meeting, the following objective of Rel-19 XR work item was agreed [1]:

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#129 meeting, the following agreements on XR rate control have been achieved [2].

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

In this contribution, we discuss the granularity of XR rate control MAC CE, assistance from UE/CN to gNB, and detail fields of the rate control MAC CE.

2. Discussion

2.1. Per QoS flow vs. per DRB MAC CE

Rate indication based on QoS flow level has been agreed in RAN2 #128 meeting. Two options have been summarised for the detail of the rate indication.

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

Some companies argue that option1 needs RAN3 to modify F1 interface while option2 does not need to do that. However, according to the feedback from RAN3[3], both options have an impact on the F1 interface. It means that compared to option1, option2 does not reduce efforts.

RAN3 thanks RAN2 for the LS on XR UL bit rate control. RAN3 discussed it and agreed both options have an impact on the F1 interface

On the other hand, option1 is more flexible to indicate all or part of the QoS flows that need to be throttled, while option2 needs all the adjustable QoS flows adapt to the new rate indicated by gNB.

Thus, from the perspectives of workload and flexibility, we slightly prefer option1.

Proposal 1: Use MAC CE to indicate QoS flows that need to be throttled.

2.2. Assistance from UE/CN to gNB

SA2 has agreed that CN configures the NG-RAN to measure and report the available bitrate per QoS flow by sending QoS monitoring request including the Reporting threshold(s) [4]. NG-RAN is expected to report the UL and/or DL available bitrate to CN when it matches or exceeds a lower or a higher threshold value indicated in the Reporting threshold(s). It means that gNB has gotten the assistance information from CN about which QoS flows can be throttled. RAN2 should change the assumption to agreement that gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control.

Proposal 2: RAN2 confirm gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control.

2.4 Detail fields of the MAC CE

If proposal 1 is adopted by RAN2, we need discuss the detail fields of the MAC CE.

Firstly, RAN2 has agreed to specify a new table for XR rate control, so we need to define a new MAC CE for XR rate control instead of reusing legacy Recommended Bit Rate MAC CE.

Secondly, QoS flow ID is only unique within the PDU session ID/DRB ID, PDU session ID/DRB ID needs to be included in the new MAC CE to identify the QoS flow ID. From our point of view, DRB ID is better for AS layer.

Thirdly, as gNB configures DRB ID and its mapped QoS flow ID sequence in SDAP-Config before sending XR rate control MAC CE, we suggest using bitmap in the rate control MAC CE to indicate which QoS flows need to be throttled, with each bit associating to the index of one QoS flow. Compared with carrying QoS flow IDs, this method can significantly reduce the size of MAC CE. QoS flows corresponding to those bits which set to 1 have the recommended available bitrates.

Theoretically, the maximum number of the mapped QoS flow is 64, and the corresponding bitmap occupies 8 octets. However, the typical number of the mapped QoS flows is much less than 64. We suggest defining three MAC CE with different length of the bitmap field (e.g. one octet, four octets, eight octets).

The following figure1 illustrate the new MAC CE of one octet. Assume that the mapped QoS flow ID are QFI2(with index Q0), QFI3(with index Q1), QFI4(with index Q2), QFI5(with index Q3). gNB need to throttle QFI3 and QFI5.

Figure 1 fields of the new MAC CE of one octet

Proposal 3: For the MAC CE design, use DRB ID to identify QoS flows that need to be throttled.

Proposal 4: For the MAC CE design, use bitmap to indicate which QoS flows need to be throttled.

Proposal 5: Define multiple MAC CEs for rate indication with different number of mapped QoS flows.

The XR rate control MAC CE provides physical layer available bitrates to QoS flows, which include the overhead of RTP layer, IP layer and all the L2 layers of NR. UE shall perform appropriate value translation when mapping to the media codec, e.g. removing the overhead of each layer.

As the sizes of all protocol overheads change slightly during the session especially when ROHC is configured, there is no standardized way to map the recommended physical layer bitrate into the media codec. UE may determine the media codec based on the long-term overhead statistics.

Proposal 6: Add a note, namely, UE should consider the overhead of all layers when mapping physical layer bitrate to media codec.

2.3. Granularity of enabling UL rate query

Another left issue in MAC running CR is as follows.

It is FFS whether the granularity of ul-BitRateQueryEnabled is per MAC entity, per logical channel or per QoS flow.

In our view, UL rate query MAC CE is only designed for XR service. If the granularity of ul-BitRateQueryEnabled is per MAC entity, it would make UE confused that whether QoS flow that does not associate to XR service can be queried. If the granularity of ul-BitRateQueryEnabled is per logical channel, gNB should configure UE which QoS flows can be throttled, otherwise UE may query the QoS flow for XR service that does not need to be throttled, which may result in unexpected behaviours in gNB. So, it is straightforward to configure ul-BitRateQueryEnabled in QoS flow granularity.

Proposal 3: The granularity of ul-BitRateQueryEnabled is per QoS flow level.

3. Conclusions

This contribution provides the following proposals.

Proposal 1: Use MAC CE to indicate QoS flows that need to be throttled.

Proposal 2: RAN2 confirm gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control.

Proposal 3: For the MAC CE design, use DRB ID to identify QoS flows that need to be throttled.

Proposal 4: For the MAC CE design, use bitmap to indicate which QoS flows need to be throttled.

Proposal 5: Define multiple MAC CEs for rate indication with different number of mapped QoS flows.

Proposal 6: Add a note, namely, UE should consider the overhead of all layers when mapping physical layer bitrate to media codec.

Proposal 7: The granularity of ul-BitRateQueryEnabled is per QoS flow level.

4. References

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #129

[3] R2-2501508

[4] S2-2502675