3GPP TSG-RAN WG2 Meeting #129bis           	R2-2501847

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on the Random Access for A-IoT

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

This paper further discusses the random access related issues for ambient IoT [1]. In details, the following aspects are covered by our contribution,

- Further details on CBRA

- On the understanding of the NACK indication

Section 2 gives the detailed discussion, and followed by the Section 3 that summarizes the conclusion of this paper.

- Discussion

- Further Details on CBRA

- The modeling of Random access occasion for slotted ALOHA

During last meeting discussion, it was agreed that a new R2D transmission is used to determine the Msg1 resources for slotted ALOHA. The main controversy is whether L1 signaling can be used for this purpose.

From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

Firstly, as mentioned in our contribution on A-IoT paging [2], the determination of the Msg1 resource by this R2D message just corresponds to the start-timing information for the access occasion(s). The intention is that if the A-IoT paging assigns a large number of access occasions over the time domain, the devices may randomly select a certain access occasion at the end of the paging round. So the device may encounter a non-negligible timing-error due to SFO, which degrades the system performance.

To ovoid above problem, two candidate solutions can be used without further RAN2 involvement,

- One straightforward way is to handle it by the L1 sync signaling. That means up to reader implementation, the reader can send the L1 R2D message to adjust the timing alignment between the reader and device.

- The other way is to up reader implementation to assign a limited number of access occasions within one paging round with consideration of the SFO impact. And the reader just sends the L1 sync signaling along with the A-IoT paging.

From our perspective, either way is workable with relying on L1 sync signaling. We slightly prefer the first way as it does not put much restriction on the reader.

Observation 1: The L1 signaling can well handle the sync issue for the device selecting a certain access occasion.

Based on the Observation 1, our intention is just to simplify the random access procedure for slot ALOHA, which is shown in Figure 1.

Figure 1: Modeling of Random access occasion for slotted ALOHA

As shown in Figure 1, at each paging round, the A-IoT paging assigns a number of access occasions which does not differentiate between initial access and re-access. The device that determines to join the current round randomly selects the access occasions over the time domain (RAN2 also assume random selection over the frequency domain). We think this benefits the device complexity to a large extent.

Proposal 1: RAN2 to take the Figure 1 as the baseline framework for slot ALOHA.

Then we would like to further clarify the following agreement,

The A-IoT paging message can include a number of msg1 resources

Based on Figure 1, there is no need additional R2D message to assign the access occasions for initial access or re-access besides the A-IoT paging.

Proposal 2: Besides A-IoT paging, there is no need extra R2D message to assign access occasions for initial access or re-access.

- Whether to join the initial access round

From the device perspective, upon receiving the initial A-IoT paging (may be the initial A-IoT paging or the subsequent A-IoT paging sent by the reader, as the device may miss the initial A-IoT paging) that needs response, the device needs to firstly decide whether to join the paging round triggered by the this A-IoT paging.

Option 1 – Follow the similar principle as RFID

The A-IoT paging can provide the Q value, and once the device receives the initial A-IoT paging that needs response, it makes the random selection among [0 2^Q-1]. If the device satisfies a certain criterion (for example, select value 0), it joins the current round to perform random access. Note that the intention of this method is to allocate the devices in different paging rounds.

Option 2 – Join the round triggered by the initial A-IoT paging

To reduce the device complexity, the most straightforward way is to just let all devices that need to make response to the initial A-IoT paging join the access round, i.e., no need to further define the round selection mechanism.

We think option 2 is a simpler one, so we suggest that,

Proposal 3: Upon receiving the initial A-IoT paging, all responding devices should join the current access round to perform CBRA.

Note that whether to response the subsequent paging (i.e, not the very first A-IoT paging for a certain device) is discussed in the paging section.

- On the content/format/monitor of the Msg(s) during CBRA

On the format A-IoT Msg1

On the D2R format, we had the following agreements for further design,

FFS whether for D2R we need message type field, any length and need for padding Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

Regarding whether to need the message type for D2R, from our view we see no motivation. Currently the D2R message only includes the A-IoT Msg1 during RACH procedure and the A-IoT Msg3 (maybe also A-IoT MsgX) during data transmission phase. That means if the reader detects the signal in the access occasion assigned by the A-IoT paging, it means the A-IoT Msg1 sent by the device. Otherwise, it is the data transmission scheduled by the reader.

On the length indication, it is also not needed for A-IoT Msg1 as it is 16-bit fixed.

Proposal 4: There is no MAC header for A-IoT Msg1.

On the content of A-IoT Msg2

During the SI discussion, we had agreed that the A-IoT Msg2 can schedule the initial or re-transmission of A-IoT Msg3. So the content of A-IoT Msg2 should be able to include the scheduling information for A-IoT Msg3.

Proposal 5a: The content of A-IoT Msg2 should include the scheduling information for A-IoT Msg3.

On the AS ID, our view is that it is used for the D2R reception and R2D scheduling during data transmission phase. The very first data transmission occurs by A-IoT Msg3. As suggested by our Proposal 5a, the reader should detect the A-IoT Msg3 at the corresponding resource sent by the device associated with the echoed random ID. If the reader fails to receive the A-IoT Msg3, it should re-send the A-IoT Msg2 to schedule the same A-IoT Msg3 to the specific device that suffers from data transmission failure. In this moment, the random ID can be used to schedule such the specific device. But at the later phase after successful A-IoT Msg3 transmission, the AS ID is needed for further R2D reception (for example, command) or D2R data transmission (for example, the response to the command). So for CBRA, the A-IoT Msg2 is the only way to assign this AS ID by associating it with the random ID.

Proposal 5b: The A-IoT Msg2 should allocate the AS ID by associating it with the echoed random ID.

On the monitor of A-IoT Msg2

Regarding when to start monitor A-IoT Msg2, it was discussed by RAN1 with the following assumptions:

For Msg2 transmission in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device. Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Therefore, we can also wait for further RAN1 progress in this regards.

Proposal 6: Wait for RAN1 further progress on the details related to the monitor of A-IoT Msg2.

- NACK feedback for re-access

For the NACK based feedback for re-access purpose, it was agreed in last meeting that,

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback

Firstly, we would like to have a common understanding on the use of NACK feedback for re-access.

During the transmission of A-IoT Msg3, if the reader detects transmission failure at the corresponding resource allocated for scheduling this A-IoT Msg3, the intention of the reader is to re-schedule the failed device in order to trigger the re-send of the same data by the device. If the reader try many times with re-transmission scheduling for a certain failed device, then the reader might think the device is under a poor channel quality and thus using the NACK mechanism to trigger re-access. So from our point of view, sending the explicit NACK in that moment will still suffer from failure, which will make this mechanism out of work.

Companies may think it can be reader implementation to decide when to send such NACK to the device. But this is not friend to the device power as the reader does not know when the device will enjoy a better channel quality again.

Observation 2: Explicit NACK may be out of work as the NACK message may still suffer from failure due to poor channel quality.

The timer-based mechanism can well handle this issue. For example, for the A-IoT Msg2 reception, RAN1 discussed the potential time window [TD2R_min, TD2R_max],

TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions.

The similar way can be used for monitoring the expected R2D after sending A-IoT Msg3. If the device does not receive the expected R2D within the time window, it considers as implicit NACK and perform re-access.

Proposal 7: The timer-based mechanism is used to implement the NACK functionality to trigger re-access for A-IoT Msg3 transmission failure.

Regarding whether such an implicit NACK mechanism can be applied to the data transmission failure after A-IoT Msg3, we further provide our view as follows.

As suggested by our another contribution [3], we prefer the device ID is not segmented, i.e., it can be up to reader implementation to ensure the A-IoT Msg3 can convey the whole device ID. Then the D2R data after A-IoT Msg3 is the data responding to the command. If these data suffers from transmission failure, we can just resort to the service-level implementation. For example, the AF finds there is missing data for a certain device after this command service. It can re-trigger a new command to the specific device to re-collect the expected response from previous failed device.

Proposal 8: RAN2 to discuss whether it can resort to service-level implementation to re-trigger a new command to the specific failed device upon data transmission failure after A-IoT Msg3.

- Conclusion

In this paper, we discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: The L1 signaling can well handle the sync issue for the device selecting a certain access occasion.

Observation 2: Explicit NACK may be out of work as the NACK message may still suffer from failure due to poor channel quality.

Further Details on CBRA

Proposal 1: RAN2 to take the Figure 1 as the baseline framework for slot ALOHA.

Proposal 2: Besides A-IoT paging, there is no need extra R2D message to assign access occasions for initial access or re-access.

Proposal 3: Upon receiving the initial A-IoT paging, all responding devices should join the current access round to perform CBRA.

Proposal 4: There is no MAC header for A-IoT Msg1.

Proposal 5a: The content of A-IoT Msg2 should include the scheduling information for A-IoT Msg3.

Proposal 5b: The A-IoT Msg2 should allocate the AS ID by associating it with the echoed random ID.

Proposal 6: Wait for RAN1 further progress on the details related to the monitor of A-IoT Msg2.

NACK feedback for re-access

Proposal 7: The timer-based mechanism is used to implement the NACK functionality to trigger re-access for A-IoT Msg3 transmission failure.

Proposal 8: RAN2 to discuss whether it can resort to service-level implementation to re-trigger a new command to the specific failed device upon data transmission failure after A-IoT Msg3.

- References

[1] Draft_RAN2_129_Meeting_Report_v3

[2] R2-2501846 Discussion on Paging for Ambient IoT CATT

[3] R2-2501848 Discussion on data transmission for A-IoT CATT