3GPP TSG-RAN WG2 Meeting #129	bis					R2-2501866

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#129 and following agreements were made:

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

  - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

  - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

  - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution we discuss further details of on-demand SIB1.

# Discussion

## 2.1. UL carrier for SIB1 request

In legacy SI request is supported for both SUL and NUL. Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL. UE should select the UL carrier before transmitting SIB1 request. SIB1 request is based on random access and in legacy random access procedure, UL carrier selection is performed at the initiation of random access procedure. So both NUL and SUL can easily be supported for SIB1 request.

Not supporting SUL for SIB1 request has more specification effort:

- a) there is impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) need to be defined in MAC spec.

- b) we need to specify that cell supporting SUL does not provide SIB1 on demand. The reason is that if cell supporting SUL provides SIB1 on demand and UE is not in NUL coverage, UE will not be able to acquire SIB1 in the absence of SIB1 request configuration for SUL.

Proposal 1: SIB 1 request is supported for both SUL and NUL.

## 2.2 Msg1 repetitions for SIB1 request

Msg1 repetitions is supported for UL coverage in R18 for random access (including Msg1 based SI request). RACH based SIB1 request (similar to Msg1 based SI request) is agreed for on-demand SIB1. So, it is reasonable to support Msg1 repetitions for SIB1 request for better UL coverage.

Proposal 2: Msg1 repetitions is supported for SIB1 request.

## 2.3 SIB1 request in RRC_CONNECTED

Figure 1

Figure 1 illustrates the scenario of SIB1 request to the selected cell upon RLF. In RAN2#129, it was agreed that when T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. Inorder for this procedure to work, we need to specify the following:

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- RRC system information request needs to include SIB X

- RRC reconfiguration message needs to include SIB X

- Mechanism to validate that last acquired SIB X in current PCell is valid at the time SIB1 request is sent to cell selected for connection re-establishment.

  - Example, UE may send SIB1 request to Cell B, if the SIB1 request configuration acquired from Cell A includes SIB1 request configuration of Cell B and the SIB1 request configuration was acquired in the current modification period.

Proposal 3: Discuss and agree the following for SIB1 request to cell selected for connection re-establishment

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- SIB X can be requested using RRC system information request in RRC_CONNECTED

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X cannot be broadcasted in active DL BWP)

- UE may send SIB1 request to selected cell (say cell B), if the SIB1 request configuration acquired from current serving cell (say Cell A) includes SIB1 request configuration of Cell B and the SIB1 request configuration was acquired in the current modification period.

In the RRC CONNECTED while T311 is not running, UE needs SIB1 to acquire other SIBs (pink highlighted conditions below). SIB request is supported in RRC_CONNECTED using DedicatedSIBRequest message. SIB1 request can be easily supported by adding a new code point for SIB1 in DedicatedSIBRequest.

Alternate approach is that if PCell supports on demand SIB1, network can ensure (using SI change notification and dedicated SIB1 delivery in RRCReconfiguration message) that UE always has the latest SIB1. UE can apply the si-BroadcastStatus from latest acquired SIB1 in the PCell, when the SIB1 is provided on demand in the Cell.

“5.2.2.3.1              Acquisition of MIB and SIB1

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

:

2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

“

Proposal 4: Discuss and agree one of the following

Option 1: DedicatedSIBRequest is updated to include request for SIB1 in RRC_CONNECTED.

Option 2: Network ensures (using SI change notification and/or dedicated SIB1 delivery in RRCReconfiguration message) that UE always has the latest SIB1 in in RRC_CONNECTED. UE can apply the si-BroadcastStatus from latest acquired SIB1 in the PCell, when the SIB1 is provided on demand in the Cell.

## 2.5 Camping on a cell supporting on demand SIB1 by legacy UE

In RAN2#127bis it was discussed and agreed that a cell supporting on demand SIB1 can be dynamically switch between broadcasting SIB1 and providing SIB1 on demand.

- Broadcast status for OD-SIB1

  - A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

  - If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

During the discussion, understanding of the company was that when the cell supporting on demand SIB1 broadcasts SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell after acquiring the broadcasted SIB1. There is no reason to bar the legacy UEs if SIB1 can be acquired by the legacy UEs.

Proposal 5: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request)), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1.

# Conclusion

Proposal 1: SIB 1 request is supported for both SUL and NUL.

Proposal 2: Msg1 repetitions is supported for SIB1 request.

Proposal 3: Discuss and agree the following for SIB1 request to cell selected for connection re-establishment

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- SIB X can be requested using RRC system information request in RRC_CONNECTED

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X cannot be broadcasted in active DL BWP)

- UE may send SIB1 request to selected cell (say cell B), if the SIB1 request configuration acquired from current serving cell (say Cell A) includes SIB1 request configuration of Cell B and the SIB1 request configuration was acquired in the current modification period.

Proposal 4: Discuss and agree one of the following

Option 1: DedicatedSIBRequest is updated to include request for SIB1 in RRC_CONNECTED.

Option 2: Network ensures (using SI change notification and/or dedicated SIB1 delivery in RRCReconfiguration message) that UE always has the latest SIB1 in in RRC_CONNECTED. UE can apply the si-BroadcastStatus from latest acquired SIB1 in the PCell, when the SIB1 is provided on demand in the Cell.

Proposal 5: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request)), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1.

# References