3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501875

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Discussion on Multi-hop Discovery and (Re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, the issue on Rel-19 multi-hop SL U2N relay discovery and (re)selection had been discussed[1]. In the current contribution, we further discuss the key leftover issues:

- FFS on Uu upper bound for intermediate relay UE.

- PC5 AS conditions for discovery model B.

- FFS on the relay reselection triggering condition for the Remote UE.

- UE behavior for the idle/inactive remote UE after intermediate relay UE triggers relay reselection as a remote UE.

Discussion

FFS on Uu upper bound for intermediate relay UE

In RAN2#128 meeting, whether an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is needed or not was discussed and reached the below conclusion:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The first FFS was solved with the below agreement in RAN2#129 meeting:

The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.  FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive.

For the second FFS, in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, some additional restriction should be introduced to avoid the scenario related to the above agreement. RAN2 can leave this issue to SA2, or the UE role can be used as one AS condition to judge that whether a UE can be operated as an intermediate relay UE or not.

Proposal 1: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 2: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

PC5 AS conditions for discovery model B

In the summary of post email discussion[2] before RAN2#128 meeting, the below status was reached:

[DISCUSSION] Proposal 5b: In the case of discovery model B, the following two options can be further discussed:

(option 1) For the discovery response message transmission (in the case of model B), the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first) intermediate Relay UE.

(option 2) As the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS conditions for transmitting the announcement/response discovery message. The same principle can be applied to the last Relay UE.

In RAN2#129 meeting, based on the SA2’s LS reply, the below further agreements were reached:

For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient).

For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.  FFS if the PC5 link is not already established.

For model B discovery, there is still one FFS in case the PC5 link is not already established. We further discuss this issue. In case that the PC5 link is not already established, this is familiar with the case that for U2U relay case, the below agreements had been reached for Rel-18 U2U relay case:

For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.

For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

With the above agreements, the following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

The leftover issue is that whether any PC5 AS conditions for discovery model B at the last relay UE is needed or not. It seems reasonable to keep the principle as agreed for Rel-18 U2U relay case. Hence, we propose:

Proposal 3: The following discovery model B forwarding condition for Rel-19 multi-hop U2N relay can be baseline for further discussion:

- For discovery model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For discovery model B, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first)intermediate Relay UE.

- For discovery model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

- For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

FFS on the relay reselection triggering condition for the Remote UE

In RAN2#129 meeting, the below agreements had been agreed for remote UE’s relay reselection triggering:

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current first relay UE.

-	When U2N Remote UE detects PC5 RLF with the current first relay UE.

-	Indicated by upper layer.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

For the first FFS about the link quality degradation of the upstream links, considering that RAN2 had already agreed that the intermediate relay UE can also adopt the same AS triggering condition of relay (re)selection of the Rel-17 remote UE. That is to say, the similar link quality was considered. Hence, it is reasonable that the link quality degradation of the upstream links may trigger a notification to its child relay UE.

Proposal 4: Link quality degradation of the upstream links may trigger a notification to its child relay UE.

For the second FFS, according to the current agreed proposals, we will analysis them one by one:

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

For the last relay UE case, there is no doubt that the notification to the remote UE is needed which is following the legacy.

-	release of RRC connection between an intermediate relay UE and the network.

We fail to see the needs of sending notification to the idle/inactive remote UE once this case happens, in case that the remote UE is in RRC_CONNECTED state, the RRC connection release at the intermediate relay causes RRC Reestablishment at the remote UE, which may further trigger relay reselection. But the current agreement is clearly focused on the idle/inactive remote UE. Hence, we propose to revisit the below agreement:

Proposal 5: RAN2 revisits the agreement that L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

This depends on the discovery model, in case for the discovery model B, besides the PC5-S connection with the first relay UE, the remote UE should also care about the path information, in case the intermediate relay UE occurs PC5 RLF or PC5-S connection release then perform a successful relay reselection, the notification to the remote UE is always needed. But for the discovery model A, the non-RRC connected remote UE doesn’t care about the path information, it just only focuses the PC5-S connection with the first relay UE. In case this happened, the case without notifying the remote UE can happen, but we fail to see any specification impact to support this case.

Proposal 6: RAN2 deletes the FFS that if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

UE behavior for the idle/inactive remote UE after intermediate relay UE triggers relay reselection as a remote UE

In RAN2#129 meeting, RAN2 agreed that the intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the Rel-17 remote UE.

Agreements:

The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

FFS if the notification is required in all cases

FFS the content in the notification

FFS the child node behaviour after receiving this notification

FFS other condition/timing that the intermediate relay UE notifies its child node

For the first FFS about the detailed specification impact, there are two options: modelled as a separate relay UE behavior vs. through functioning as a remote UE. According to the agreed SA2 CR[3], the current wording is list as below:

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

For the above green marked part, there are two different understandings:

- Option1: the intermediate relay UE has U2N relay data transmission requirement. Then we treat the intermediate relay UE as a remote UE to access to the gNB.

- Option2: no need to consider whether there is U2N relay data transmission requirement or not by the intermediate relay UE. The intermediate relay UE just takes the role of the remote UE in the procedure.

From our point of view, this issue had been discussed during the scenario. There is no limitation that the intermediate relay UE has U2N relay data transmission requirement. But once we talk about the intermediate relay UE acting as a remote UE, we also don’t limit that there must have U2N relay data transmission requirement for the intermediate relay UE. For approach1, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. The connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE, currently, there is no requirement that the intermediate relay UE has U2N relay data transmission requirement at all.

Proposal 7: RAN2 clarifies the wording that intermediate relay UE takes the role of the remote UE in the procedure is not equivalent to the intermediate relay UE has U2N relay data transmission requirement.

Next, for the detailed specification impact, we prefer to reduce the specification impacts as much as possible. Hence, we propose that the specification impact models the intermediate relay UE through functioning as a Rel-19 remote UE.

Proposal 8: The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the Rel-17 remote UE. The specification impact models the intermediate relay UE through functioning as a Rel-19 remote UE.

For the second FFS if the notification is required in all cases, according to the agreed SA2 CR[3], the related chapter is under 6.4.3.8.3 of TS 23.304, that is to say, the case that allow the intermediate relay UE triggers the relay reselection can just only happen based on Model A discovery.

Next, according to the understand of SA2 progress, at least for L3 multi-hop U2N relay case:

- When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

- When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay.

Based on the above understanding, in case that the intermediate relay UE occurs relay reselection, different intermediate relay UE’s behavior should be considered. For discovery model A, the non-RRC connected remote UE doesn’t care about the path information, it just only focuses the PC5-S connection with the first relay UE.

Proposal 9: RAN2 agrees the case that intermediate relay UE occurs relay reselection acting as a remote UE can only happen under Model A Discovery.RAN2 sends one LS to SA2 to further check this understanding.

Next, in SA2’s agreed CR[3], it stated that the relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure, when its connection with a parent Relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold. If the 5G ProSe Intermediate UE-to-Network Relay cannot complete the reselection, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path. After receiving the re-selection indication, the 5G ProSe Remote UE may trigger the reselection of the relays as described above.

That is to say, once the intermediate relay UE satisfies the AS triggering condition of relay reselection as a remote UE, it depends on the intermediate relay UE’s implementation whether to trigger relay reselection or send notification to its child relay UE and/or remote UE.

Proposal 10: Once the intermediate relay UE satisfies the AS triggering condition of relay reselection as a remote UE, it depends on the intermediate relay UE’s implementation whether to trigger relay reselection or send notification to its child relay UE and/or remote UE.

For the third FFS on the child node behavior after receiving this notification, considering the case that the child nodes include other intermediate relay UEs, we further distinguish the UE role. If child intermediate relay UE receives its parent UE’s notification, it can decide by itself to perform relay reselection or further send this notification to its child UE. If the remote UE receives its parent UE’s notification, the relay reselection can be triggered.

Proposal 11: During the notification message transfer to its child UE, if the intermediate relay UE receives this notification, it is left to intermediate relay UE implementation to trigger relay reselection or further forward this notification message to its child UE.

Proposal 12: During the notification message transfer to its child UE, if the remote UE receives this notification, relay reselection will be triggered.

For the fourth FFS on other condition/timing that the intermediate relay UE notifies its child node. As what we discussed previous, we prefer there is no condition or timing for the intermediate relay UE notifies its child node in Rel-19.

Proposal 13: RAN2 agrees that no condition or timing is introduced for the intermediate relay UE notifies its child node in Rel-19.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 2: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Proposal 3: The following discovery model B forwarding condition for Rel-19 multi-hop U2N relay can be baseline for further discussion:

- For discovery model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For discovery model B, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first)intermediate Relay UE.

- For discovery model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

- For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

Proposal 4: Link quality degradation of the upstream links may trigger a notification to its child relay UE.

Proposal 5: RAN2 revisits the agreement that L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating release of RRC connection between an intermediate relay UE and the network.

Proposal 6: RAN2 deletes the FFS that if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

Proposal 7: RAN2 clarifies the wording that intermediate relay UE takes the role of the remote UE in the procedure is not equivalent to the intermediate relay UE has U2N relay data transmission requirement.

Proposal 8: The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the Rel-17 remote UE. The specification impact models the intermediate relay UE through functioning as a Rel-19 remote UE.

Proposal 9: RAN2 agrees the case that intermediate relay UE occurs relay reselection acting as a remote UE can only happen under Model A Discovery.RAN2 sends one LS to SA2 to further check this understanding.

Proposal 10: Once the intermediate relay UE satisfies the AS triggering condition of relay reselection as a remote UE, it depends on the intermediate relay UE’s implementation whether to trigger relay reselection or send notification to its child relay UE and/or remote UE.

Proposal 11: During the notification message transfer to its child UE, if the intermediate relay UE receives this notification, it is left to intermediate relay UE implementation to trigger relay reselection or further forward this notification message to its child UE.

Proposal 12: During the notification message transfer to its child UE, if the remote UE receives this notification, relay reselection will be triggered.

Proposal 13: RAN2 agrees that no condition or timing is introduced for the intermediate relay UE notifies its child node in Rel-19.

Reference

- R2-2501334 Report from session on positioning and sidelink relay	Session chair (MediaTek)	report

- R2-2410981 Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.	discussion	Rel-19

- S2-2501292 Clarification on Multi-hop Relay Reselection Huawei, HiSilicon, China Telecom