3GPP TSG-RAN2 Meeting#129bis												R2-2501884

# Wuhan, China, April 7th - April 11th, 2025

Agenda Item:	6.1.3.2

Source:	OPPO

Title:	Discussion on Paging Capability Issue

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of missing paging capability, based on the 129 conclusion as follows

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

- Discussion

Reading the current paging related capability

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v15e0-IEs                      OPTIONAL

}

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v1700-IEs          OPTIONAL

}

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1840-IEs                   OPTIONAL

}

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                              OPTIONAL

}

Logically, all paging capability(ies) which have not been included in the container of ue-RadioPagingInfo-r17 would have this issue.

Which can be further divided into three types:

- Type-1: as highlighted as green above, without these capability, the impact is mainly at network side, i.e., the network has less freedom in terms of paging related PDSCH scheduling, but no impact to the UE, and there would be no paging message loss. (For 2Rx XR, since it does not change the PO location, it is not clear whether there is a difference when network sends paging message to 4-Rx and 2-Rx UE).

- Type-2: as highlighted as blue above, where (e)Redcap would only enter into (e)Redcap-capable cell, which would generated capability in a correct manner, so that no misunderstanding of (e)Recap related paging capability.

- Type-2: as highlighted as yellow above, without these capability, for a UE in RRC INACTIVE supporting this feature camps a cell supporting this feature but not aware of UE’s paging capability, network would send RAN paging to the UE in POs which are derived from the RAN configured UE-specific DRX cycle while the UE would monitor paging in POs which are derived from the CN configured UE-specific DRX cycle, the POs derived by network and UE may be different,, i.e., there would be paging message loss.

There is inter-operability issue for at least one legacy paging capability that would lead to paging message loss.

To solve this issue, there are in general two method directions.

Direction-1: Non-specified solution, i.e., rely on network upgrading.

In this method,

- The cons is it has to be done mandatorily for all legacy RAN nodes, while

- The pros is that it helps legacy UEs in the field (if any that has implemented the paging related feature).

Direction-2: Specified solution, i.e., rely on specified solution

- The pros is it relax the network burden, i.e., it allows network to keep the legacy RAN node implementation, but limit the effort to new RAN node implementing the features

- The cons is that it cannot solve the issue for legacy UE. And it requires additional spec/implementation effort at UE side.

- R2 decide whether to rely on network implementation (no specified solution), or specified solution to solve the issues (for paging capability before R17, e.g, inactiveStatePO-Determination-r17).

If R2 tends to go for specified solution.

Most importantly, it has to avoid paging message loss at legacy RAN node, and to reach that, the key point is that both UE and RAN node reached a common understanding regarding how to deliver the paging message.

- For a RAN node not supporting the paging enhancement feature, there is no such concern.

- For a RAN node supporting the paging enhancement feature, there is such concern.

For case-2 above: it is always preferred that UE can enjoy the benefit from the paging enhancement feature.

To achieve that, the concerned RAN node supporting the paging enhancement feature has to know the UE capability – otherwise, if it is just refer to the capability profile generated by a legacy RAN node, the concerned UE may be misunderstood as not supporting the related paging capability.

To achieve that, the easiest method is that UE trigger a capability update oriented TAU towards AMF, when it arrives at RAN node supporting the concerned feature, and thus the whole paging capability profile can be re-generated, assuming the RAN node supporting this paging feature should be able to generate the capability correctly.

In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE shall use the same i_s as for RRC_IDLE state. Otherwise, the UE determines the i_s based on the parameters and formula above.

- If R2 decides to adopt the specified solution, UE trigger a MRU (Mobility Registration Update) to update paging capability (if has not done since last NAS registration) upon camping on a cell indicating ranPagingInIdlePO.

- Conclusion

Proposal 1	R2 decide whether to rely on network implementation (no specified solution), or specified solution to solve the issues (for paging capability before R17, e.g, inactiveStatePO-Determination-r17).

Proposal 2	If R2 decides to adopt the specified solution, UE trigger a MRU (Mobility Registration Update) to update paging capability (if has not done since last NAS registration) upon camping on a cell indicating ranPagingInIdlePO.