3GPP TSG-RAN2 Meeting#129bis	R2-2501886

# Wuhan, China, April 7th - April 11th, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

Based on the latest R1 progress

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

For the FFS point on R2, considering the following R2#127bis conclusion

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

It is already confirmed by R2, no further conclusion is needed from R2 side.

R2 has already confirmed the feasibility of RRC-based deactivation.

Furthermore, for deactivation: R1 confirmed the two options above, i.e., MAC-CE may or may not be used for OD-SSB deactivation. In 129, one concern raised by companies is that there could be cases where

- For cell-A, the OD-SSB deactivation is controlled by MAC-CE, so a bit 0 in MAC-CE means deactivation;

- For cell-B, the OD-SSB deactivation is controlled by RRC, so a bit 0 in RRC means nothing;

And thus, concern is raised regarding how to the two different types of bit 0 co-exist in the same MAC-CE. In our view, it can be easily solved by a per-cell RRC configuration, i.e., regarding whether it is controlled by MAC-CE or not.

For activation: R1 concluded that

Agreement

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

I.e., R2 has to firstly to conclude that MAC-CE can be used for SSB burst adaptation, i.e., change SSB burst periodicity without deactivation first. Note that there is no difference between the two cases from MAC-CE design perspective

- MAC-CE is to activate the OD-SSB, which is currently deactivated

- MAC-CE is to change the periodicity of an OD-SSB, which is current activated

Since in both cases, a bit-1 would indicate the presence of associated configuration / parameter (including periodicity) field, and OD-SSB is (re)activated as a result of the MAC-CE transmission.

- R2 agree that MAC-CE can be used for SSB periodicity change, i.e., change the periodicity of an OD-SSB that is being transmitted.

For the MAC-CE format design, R2 agreed that

=> Will discuss it later.

=> NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

There are in general two methods: one is to include cell-index of OD-SSB explicitly while the other is to include cell-index implicitly via an ordered bitmap. In general, the latter one is more signaling efficient, considering the saved overhead for cell-index indication due to the cell-index indication (especially in case of simultaneous multi-SCell (de)activation. For the meaning of the bit

- In case of option-1 (Explicit deactivation via MAC-CE): 1/0 is for activation/deactivation, and presence/absence of (at least) periodicity parameter.

- In case of option-2 (Implicit deactivation): 1/0 is for presence/absence of (at least) periodicity parameter.

- For OD-SSB MAC-CE, adopt the bitmap-based method to indicate OD-SSB status for multiple SCells simultaneously, where 1) 1 means either OD-SSB activation or periodicity change of an activated SSB (for both cases, the configuration/parameter field for the concerned cell is present), and 2) 0 means either OD-SSB (de)activation or nothing, depending on per-cell RRC configuration (for both cases, the configuration/parameter field for the concerned cell is absent).

Besides, R1 had an FFS point regarding which parameter(s), besides periodicity, has to be carried by MAC-CE

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

So that R2 cannot conclude on the detailed design of the MAC-CE format.

- R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

  - Issue-2: L3 Measurement of OD-SSB

Comparing Case#1 and Case#2,

- For OD-SSB, hopefully the conclusion of Case#1 can be reused as much as possible to OD-SSB of Case#2, i.e., for the issues of OD-SSB itself, R2 can prioritized the discussion of Case#1 to simplify the discussion.

- On the other hand, R2 can focus on the relationship between OD-SSB and AO-SSB in Case#2, to clarify the Case#2 specific issues, as complementary part of conclusion of OD-SSB.

    - Case#1-Specific Issues

For serving cell measurement, 127bis discussed that

3. L3 RRM Framework

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Need more time to think of.

Based on R4#112bis conclusion:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

Based on R4#113 conclusion for case-1

Agreement:

OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement

For the case of CSSF = 1:

- OD-SSB periodicity regardless of measCycleSCell and DRX cycle.

  - FFS whether a lower bound will be specified.

FFS for the other cases of CSSF.

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

i.e., R4 agreed on a hybrid approach, i.e., UE will perform a hybrid fast + slow mode measurement

1) In the fast mode window, the measurement requirement is defined based on SSB periodicity.

2) After the fast mode window, if the OD-SSB is still activated, the measurement requirement is defined as in legacy, i.e., based on measCycleSCell and DRX cycle.

R4 agreed on a hybrid L3 measurement framework for OD-SSB Case#1 for deactivated SCell, where the fast mode measurement relies on SSB periodicity ignoring measCycleSCell and DRX cycle, and the slow mode measurement relies on measCycleSCell and DRX cycle ignoring SSB periodicity.

If the measurement requirement is based on SSB periodicity, there is no additional benefit by introducing an adaptive SMTC, which exactly copied the SSB pattern, without additional functionality introduced.

For fast mode measurement, adaptive SMTC does not bring additional functionality, on top of SSB pattern based L3 measurement, considering both need to follow SSB periodicity-based requirement.

In other words, “fast mode window” already implies that the UE shall be able to perform L3 measurement on SCell based on SSB pattern directly, ignoring SMTC. After the “fast mode window”, the measurement is not related to the SSB periodicity, but relies on the measCycleSCell and DRX cycle, i.e., the corresponding L3 measurement does not change with SSB periodicity

- Firstly, it is feasible for UE to perform L3 measurement based on SSB pattern (but following measCycleSCell and DRX cycle), since anyway SMTC window is more meaningful for neighboring cell for which no prior information on the SSB configuration and synchronization at UE side.

- Secondly, another approach is relying on legacy SMTC, but since dynamicity of OD-SSB goes against the static of SMTC, it is not feasible to do that, unless restrict network implementation to ensure OD-SSB can at least partially overlapping with statically configured SMTC.

For slow mode measurement, legacy SMTC, which was essentially introduced for neighboring cell measurement, goes against dynamicity of OD-SSB since it is statically configured.

Furthermore, this issue can be further extended from L3 measurement on deactivated SCell, to L3 measurement of SCell during activation / activated already.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity

Agreement:

- In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity.

  - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity.

- For OD-SSB Case#1, for L3 measurement of SCell, R2 confirm it is performed based on OD-SSB pattern but ignoring SMTC, to align with R4 conclusion.

    - Case#2-Specific Issues

For case-2, R1 is still working on the feasibility of different frequency scenario

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

R1 already confirmed the feasibility of same-frequency scenario for AO/OD-SSB in Case-2 (when AO-SSB is NCD-SSB), and FFS when AO-SSB is CD-SSB.

Considering that, R2 can proceed for the same-frequency scenario in the coming meeting, e.g., regarding whether to allow combined measurement on always-on SSB and OD-SSB.

Figure 1 An example of Case-2 (AO-SSB and OD-SSB are partially overlapping)

Based on R1 conclusion, Figure-1 gives an example of partially overlapping AO/OD-SSB, e.g., AO-SSB/OD-SSB is of 40ms/20ms periodicity, with same offset, so it leads to 3 SSB bursts which are overlapping between each other. In this case, it is hard to differentiate between AO-SSB and OD-SSB, at the overlapping places.

Furthermore, R2 defined a L3 measurement framework in the following manner, I.e., the configuration of AO-SSB and OD-SSB have to be carried via a same MO.

The network applies the procedure as follows:

[…]

- to ensure that, in the measConfig associated with a CG:

- for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

Combining the two, i.e., same MO and hard to differentiate the SSB attributive (AO-SSB or OD-SSB), it seems easier to perform a combined L3 measurement, and generate a single MR. But that is to be further evaluated by R4, and R2 only needs to focus on MO configuration and MR report.

Considering R1 conclusion, R2 can firstly conclude on C1. And wait for R4 conclusion on C2.

- For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 wait for R4 conclusion on the feasibility of combined measurement, or R2 firstly conclude the support of L3 measurement configuration based on a single MO for C1 as concluded by R1.

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

For different frequency case, R1 has already concluded that the combined measurement is not applicable. And it is up to R4 regarding whether to support this scenario.

- For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 wait for R4 conclusion on the feasibility of supporting this scenario in R19, or R2 firstly conclude the support of L3 measurement configuration based on separate MO (if this scenario is confirmed to be supported by R4 later).

If R2 would like to progress more, some key issues need to be considered.

- servingCellMO configuration granularity: to stick to the principle of single-MO for single SSB frequency, it is straightforward to use two separate MOs for inter-frequency case. While the remaining issue is whether to apply this to per-cell MO (for which both intra- and inter-frequency scenarios need to be considered) only or per-BWP MO (for which only intra-frequency scenario needs to be considered so far) as well.

Table 1 AO-/OD-SSB scenarios based on latest R1 conclusion

| AO-SSB Case | AO-SSB is CD or NCD-SSB | AO-SSB is on sync raster or not | Is it applicable to servingCellMO in ServingCellConfig | Is it applicable to servingCellMO in BWP-DownlinkDedicated | RAN1 concluded scenario for AO-SSB and OD-SSB |
| --- | --- | --- | --- | --- | --- |
| Case-1 | CD | Yes | Yes | No | Inter-frequency |
| Case-2 | NCD | Yes | Yes | Yes | Intra-frequency |
| Case-3 | CD | No | Yes | No | Intra-frequency |
| Case-4 | NCD | No | Yes | Yes | Intra-frequency |

- MO Applicability: And when both SSB types are available, for separate frequency case, it is straightforward to rely on the OD-SSB, similar to the legacy scenario when CD-SSB and NCD-SSB co-exist. Following this principle, the applicable SSB would be used to define the intra-/inter-frequency measurement definition.

- Gap applicability: there is no remaining issue for gap necessity, since that no matter OD-SSB is configured as per-cell or per-BWP, it has to stick to the R1 conclusion that AO-SSB and OD-SSB are located in the same BWP. So that the gap necessity can follow legacy rule.

- For OD-SSB Case#2, R2 confirm introduce OD-SSB configuration (e.g., MO) into ServingCellConfig, and R2 further discuss whether to include it in BWP-DownlinkDedicated as well (if this scenario is confirmed to be supported by R4 later).

- For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 confirm to rely on OD-SSB for the measurement and report triggering when OD-SSB is being transmitted (if this scenario is confirmed to be supported by R4 later).

    - Common Issues for Case#1/Case#2

For L3 measurement, since it is possible that serving cell and neighbouring cell share the same MO configuration, e.g., in an intra-frequency scenario, it is good to understand the behavior for neighboring and serving cell scenario.

For neighbouring cell, it anyway needs to rely on SMTC window to identify the time location of SSB searching. Besides, when it comes to OD-SSB, it is anyway infeasible for UE to know the neighboring cell OD-SSB (de)activation status. Combing the two, it seems more realistic to rely on Case-2, if network would like to configure the UE to perform neighboring cell measurement. Anyway, there seems not much work that R2 can do for the neighbouring cell measurement under OD-SSB objective, and it aligns with R4 conclusion below

Issue 1-1-2: OD-SSB impact on neighbour cell measurement

Agreement:

- For the RRM measurement on neigboring cell, the legacy meaurement framework is applied.

  - Note: The discussion of SMTC configuration is a separate issue.

- This agreement can be revisited if it is not aligned with further agreement in other WGs.

- For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Another issue is whether to rely on legacy event-based MR triggering or rely on MAC-CE based MR triggering.

We understand that MAC-CE based MR triggering was introduced for FR2 unknown SCell, while OD-SSB can be sent before SCell activation, and thus enable known case, as long as the L3 MR triggering event has been configured properly (regardless of whether the AO and OD-SSB are on the same or different frequencies), which would lead to an even quicker SCell activation compared with the “semi-known” case of MAC-CE triggered MR. Considering that, the motivation of introducing MAC-CE based MR triggering is not strong.

Issue 1-4-1: The known cell condition – measurement reporting

Agreement:

- In case 1, known/unknown condition should be defined based on only OD-SSB.

- In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB.

  - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue.

  - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB.

  - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

- R2 not pursue MAC-CE triggered MR for OD-SSB.

- Conclusion

Proposal 1	R2 agree that MAC-CE can be used for SSB periodicity change, i.e., change the periodicity of an OD-SSB that is being transmitted.

Proposal 2	For OD-SSB MAC-CE, adopt the bitmap-based method to indicate OD-SSB status for multiple SCells simultaneously, where 1) 1 means either OD-SSB activation or periodicity change of an activated SSB (for both cases, the configuration/parameter field for the concerned cell is present), and 2) 0 means either OD-SSB (de)activation or nothing, depending on per-cell RRC configuration (for both cases, the configuration/parameter field for the concerned cell is absent).

Proposal 3	R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

Proposal 4	For OD-SSB Case#1, for L3 measurement of SCell, R2 confirm it is performed based on OD-SSB pattern but ignoring SMTC, to align with R4 conclusion.

Proposal 5	For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 wait for R4 conclusion on the feasibility of combined measurement, or R2 firstly conclude the support of L3 measurement configuration based on a single MO for C1 as concluded by R1.

Proposal 6	For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 wait for R4 conclusion on the feasibility of supporting this scenario in R19, or R2 firstly conclude the support of L3 measurement configuration based on separate MO (if this scenario is confirmed to be supported by R4 later).

Proposal 7	For OD-SSB Case#2, R2 confirm introduce OD-SSB configuration (e.g., MO) into ServingCellConfig, and R2 further discuss whether to include it in BWP-DownlinkDedicated as well (if this scenario is confirmed to be supported by R4 later).

Proposal 8	For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 confirm to rely on OD-SSB for the measurement and report triggering when OD-SSB is being transmitted (if this scenario is confirmed to be supported by R4 later).

Proposal 9	For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Proposal 10	R2 not pursue MAC-CE triggered MR for OD-SSB.