3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2501889

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Discussion on conditional LTM

Document for:	Discussion and Decision

Introduction

In latest meeting, RAN2 discussed conditional LTM and the following agreements were reached:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

Early sync TA maintenance

In previous meeting, it has agreed that PDCCH ordered early TA acquisition is supported for conditional LTM. Additionally, early sync with RA response is supported for CLTM as there is no LTM cell switch MAC CE delivered to UE. Regarding the RA response format, it has agreed the network can inform the candidate cell’s TA information to UE via new MAC CE.

For network-triggered LTM, the TA obtained through the early synchronization procedure is explicitly indicated by the LTM cell switch command, and the UE starts the TA timer associated with PTAG upon receiving the LTM cell switch command. For CLTM, the TA timer associated with PTAG should also be started when executing the cell switch. Regarding the CLTM early synchronization procedure, it has been agreed that the candidate cell TA will be maintained by a CLTM TAT. If the candidate TA MAC CE including TA information is provided to the UE, the UE will start the CLTM TAT associated with the candidate cell. This raises questions about how to handle the CLTM TAT associated with the target cell upon CLTM execution. During the last meeting, several options were discussed concerning the handling of the CLTM TAT:

CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session:

-	Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18;

-	Option 2 (keep the offline agreement): discuss the options below in the MAC running CR

o	Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR;

o	Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running.

o	Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer.

TAG-based serving cell TA maintenance and management was introduced since R15. The early sync TA maintenance should not affect the legacy TAG-based TA maintenance. Additionally, the early sync TA timer for target or candidate cells is used for candidate cell TA maintenance. Therefore, it is logical for the UE to start the PTAG TAT upon executing a cell switch. And as UE already maintained the TAT for PTAG, there is no need to keep a redundant TA timer running.

Furthermore, option 1 is aligned with the TAT handling of NW triggered LTM and the spec impact is simpler and less than option2. Thus, we would like to go for option 1, that is, UE stop the CLTM TAT associated with target cell and start the timeAlignmentTimer associated with PTAG as in Rel-18.

And regarding the ongoing CLTM TATs for other candidate cells, similar principle should be followed. upon CLTM execution or other HO execution, UE shall stop all running TA timers which maintained for the CLTM candidate cells and start the corresponding TA timer associated with PTAG of the target cell.

As CLTM TAT is used for maintaining the TA of candidate cells, once the candidate configuration is released, there is no need to maintain the corresponding TAT anymore.

Upon CLTM execution, UE stops the CLTM TAT associated with target cell and start the timeAlignmentTimer associated with PTAG.

The CLTM TAT timer of other candidate cell(s) is stopped upon cell switch execution, including both LTM/CLTM cell switch execution and L3 HO execution.

The CLTM TAT(s) is stopped upon the corresponding candidate cell is released.

Besides PDCCH ordered early sync, there is another alternative for triggering the early sync procedure, i.e. event triggered early sync. As defined in the WID, conditional LTM is introduced in Rel-19 which aims to achieve high robustness by enabling the procedure to be executed without explicit signalling exchange with source cell beforehand. With event triggered early sync, UE can evaluate the candidate cell/RSs and trigger early sync to a fulfilled cell/beam to acquire the TA value. Furthermore, companies have also reached consensus that the event triggered L1 measurement reporting can be used for early sync scenario since UE have the real-time L1 measurement of the candidate cells/RSs in R19 inter-CU LTM. The same principle can be used for event triggered early sync. UE can trigger the early sync to candidate cell(s) if the candidate fulfils the condition.

Event triggered early sync to candidate cell(s) is supported for CLTM.

Additionally, one more open issue is that whether CLTM support UE-based TA measurement. In R18, UE can obtain the TA by performing DL measurement between source and candidate. And we think the benefits of UE-based TA measurement can be extended to intra-CLTM.

UE based TA measurement is supported for CLTM.

Execution condition evaluation

In TS38.300, how UE performs the condition evaluation for CHO is defined once the L3 HO is triggered.

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed.

For CLTM, the start/stop condition for execution condition evaluation should be same as CHO, which is that UE starts evaluating the execution condition(s) upon receiving the CLTM configuration and stops evaluating the execution condition(s) once a handover is executed. In last meeting, we have confirmed that UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

The issue of when to stop execution condition evaluation for CLTM remains unresolved. Once a cell switch is executed, monitoring other candidates is no longer necessary as UE is accessing the selected cell. If UE continues to monitor and evaluate other candidate cells, it is necessary to further specify UE behaviour in the case that another candidate triggers a cell switch execution while UE is accessing a CLTM candidate.

UE stops evaluating the execution condition(s) once a cell switch/handover is executed.

And if any HO command/ LTM cell switch command is received before any CLTM execution condition is satisfied, UE shall execute the received HO command/LTM cell switch command, regardless of any previously received CLTM configuration.

Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration) or LTM cell switch command MAC CE, the UE executes the HO procedure as described in clause 9.2.3.2 or LTM cell switch procedure as described in clause 9.2.3.5, regardless of any previously received CHO configuration.

Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

LTM, CLTM and L3 HO coexistence

The issue on coexistence between different types of HO mechanism should also be studied. As shown below, Rel-18 LTM support the coexistence with all other L3 HO mechanisms except DAPS.

| Coexistence(Y/N) | Normal HO | DAPS | CHO | CPAC | SCPAC |
| --- | --- | --- | --- | --- | --- |
| LTM | Y | N | Y | Y | Y |

For CLTM, we can follow similar agreements made in Rel-18, CLTM can be coexisted with other L3 HO mechanism.

CLTM and L3 HO coexistence are supported.

CLTM and DAPS coexistence is not supported.

According to existing RRC signalling structure, LTM and CLTM use same IE (i.e. LTM-Candidate IE is re-used to model an CLTM candidate cell configuration) to provide candidate configuration, which implies LTM and CLTM coexistence is supported. And in last meeting, it has agreed that for intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

As subsequent LTM/CLTM is supported, there may be cases where an intra-CU CLTM candidate becomes an inter-CU LTM candidate following an inter-CU LTM execution. Conversely, an inter-CU LTM candidate may become an intra-CU (C)LTM candidate after an inter-CU LTM execution. To avoid frequent network reconfiguration when switching LTM candidate types, supporting a single candidate configuration with two triggers (LTM and CLTM) seems necessary. And even an intra-CU CLTM is associated with L1/L3 execution condition, we see no need to restrict NW to trigger the cell switch execution with cell switch MAC CE.

Network is allowed to trigger LTM cell switch execution for a candidate cell which is associated with CLTM execution condition.

Coexistence

In last meeting, RAN2 initially discussed the coexistence between R19 inter-CU LTM and other features, and the following conclusion has been reached. For CLTM, similar conclusion can be followed.

Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

Same as Rel-18 intra-CU LTM, Rel-19 intra-CU CLTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU.

In R18, LTM support the coexistence with R18 MIMO 2TA. For CLTM, we understand there is no need to prohibit a candidate cell configuring mTRP.

Support the coexistence of R18 MIMO two TA and CLTM.

Currently we have agreed NW triggered early sync procedure and early TCI state activation/deactivation are supported for CLTM, while CLTM execution is triggered by UE based on pre-configured execution conditions. It should further study on how to align the beam of early sync, TCI state and CLTM execution since UE and NW may not be synced on the selected TRP/TAG.

In R18 LTM supporting MIMO 2TA, UE determines the TA group of the accessed TRP according to the pointer associated with the indicated TCI state in LTM cell switch command. For PDCCH ordered early sync in CLTM, UE follows NW indication to perform RACH for TA acquision, and NW can also provide the TAG info within the candidate TA MAC CE. Upon CLTM execution, UE could select the beam which associated with the same TAG. For early TCI state activation/deactivation, it can leave for NW implementation to indicate the TCI state which pointer to same TAG as the one indicated in PDCCH order for early sync.

The beam alignment issue for CTM execution, early sync, TCI activation can be resolved with following way: a) TAG info is indicated in candidate TA MAC CE b) for CLTM execution, UE selects the beam which pointer to same TAG of the available TA.

R19 MIMO further extend 2TA to UL-only TRP scenario, which require a dynamic adjustment on PL_offset for supporting UL transmission on UL-only TRP. Based on RAN1 discussion, the PL_offset is obtained by SRS measurement in NW side, which is not supported for LTM candidate. And whether to support coexistence of CLTM and R19 MIMO 2 TA is also studied in MIMO session. We can wait for their conclusion and align views between Mobility session and MIMO session later.

FFS on the coexistence of R19 MIMO two TA and CLTM.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	Upon CLTM execution, UE stops the CLTM TAT associated with target cell and start the timeAlignmentTimer associated with PTAG.

Proposal 2	The CLTM TAT timer of other candidate cell(s) is stopped upon cell switch execution, including both LTM/CLTM cell switch execution and L3 HO execution.

Proposal 3	The CLTM TAT(s) is stopped upon the corresponding candidate cell is released.

Proposal 4	Event triggered early sync to candidate cell(s) is supported for CLTM.

Proposal 5	UE based TA measurement is supported for CLTM.

Proposal 6	UE stops evaluating the execution condition(s) once a cell switch/handover is executed.

Proposal 7	Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Proposal 8	CLTM and L3 HO coexistence are supported.

Proposal 9	CLTM and DAPS coexistence is not supported.

Proposal 10	Network is allowed to trigger LTM cell switch execution for a candidate cell which is associated with CLTM execution condition.

Proposal 11	Same as Rel-18 intra-CU LTM, Rel-19 intra-CU CLTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU.

Proposal 12	Support the coexistence of R18 MIMO two TA and CLTM.

Proposal 13	The beam alignment issue for CTM execution, early sync, TCI activation can be resolved with following way: a) TAG info is indicated in candidate TA MAC CE b) for CLTM execution, UE selects the beam which pointer to same TAG of the available TA.

Proposal 14	FFS on the coexistence of R19 MIMO two TA and CLTM.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx