3GPP TSG RAN WG2 #129bis                                                                         	  R2-2501925

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Signalling feasibility of AIML model transfer

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we continue to discuss the signalling feasibility of AIML model transfer in Uu interface.

- Discussion

  - Background of RRC segmentation

To transmit large AI training data over CP, the TR37.873 [2] about segmentation of UE capability information could be considered as background:

There are two options to split and transmit large data in Uu control plane based on study on optimizations of UE radio capability signaling:

6.2.1.3.1 Segmentation at RRC level In the segmentation of UE capabilities in the RRC level, for each segment, a type (e.g. notLastSegment, lastSegment) as well as a segment number is added. The RRC segmentation can be done in different ways as described below. Option 1: "hard" split RRC Segmentation: This approach is similar to ETWS/CMAS in LTE and NR. The total UE radio capability information is encoded as one structure (message or information element) similar to non-segmented case. This is then split into multiple segments in bits. The network can only decode the complete capabilities after assembling all parts, rather than decoding each part. Option 2: "soft" split RRC Segmentation: UE radio capability information is encoded separately in parts and carried by multiple messages or information elements (smaller than 9000 bytes for NR or 8188 bytes for LTE). The network can decode and process every segment and combine the content after all the segments are received.

This study is for UL UE capability, but both the options are also applicable for DL large data transmission.

Considering that it is meaningless to decode each segment separately of the UE capability information, the UL UE capability transmission uses the “hard” split option similar as ETWS/CMAS at last. And network can only decode the complete capabilities after assembling all parts.

Proposal 1: To analyze the AIML model transfer at RRC level, the 2 split segmentation options discussed for UE radio capability optimization can be considered.

  - Signalling feasibility of AI/ML model transfer

We discuss the RRC segmentation method of UL training data transmission in [3], and propose to use a “soft” split RRC segmentation method similar as logged MDT for solution3 over CP to avoid the scalability problem. Both the AI/ML data collection report in UL and the model transfer in DL are large scale data transmission. Therefore we could consider whether to use the similar “soft” split RRC segmentation method for signaling/mechanism design for UL and DL.

For AI/ML model transfer, RAN1 has sent a LS [1] to RAN2 for the signalling feasibility of dataset and parameter sharing.

For alleviating/resolving issues related to inter-vendor training collaboration, RAN1 concluded that the standardized signaling may be:

- over-the-air, or

- other approaches

RAN1 asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}, may be roughly in the order of 225 MB;

- Encoder parameter sharing, may be roughly in the order of 11.6MB;

- Encoder parameter sharing + dataset sharing consisting of {target CSI}, may be roughly in the order of N2 * (2000 bits) / (8bits/Byte) + 11.6 MB, N2 is the number of samples which may equal to or smaller than 600K.

All options from RAN1 listed above need a large scale of data between UE and gNB. In legacy specification, the PDCP SDU size limit is about 9000 bytes, and current supported max value of DL message segments is only 5. RRC buffer size is defined in TS38.306 for RRCResume or RRCReconfiguration messages:

4.2.14 RRC buffer size The RRC buffer size is defined as the maximum overall RRC configuration size that the UE is required to store. The RRC buffer size is 45Kbytes.

To send the dataset and parameter sharing related data, more than 1000 (11.6 MB / 9KB) segments are needed, which greatly exceeds the RRC buffer size defined for current UE capability.

Observation 1: The model transfer size of all the 3 options provided by RAN1 needs more than 1000 segments which greatly exceeds RRC buffer size defined for current UE capability.

To transmit the large scale of model related information, consider the 2 options in the segmentation of UE capability TR [2]:

  - “Hard” split option: The advantage is that the codec is only needed once, and the disadvantage is that the UE capability is required to explicitly support the large RRC buffer size;

  - “Soft” split option: Using a logged MDT report similar downlink transmission mode, the advantage is that a large RRC buffer size may not be required, and the UE decodes each segment in sequence and sent it to e.g. storage node of the AI model. The disadvantage is that it requires multiple decoding.

However, compared with the cleartext record of AI data collection, the encoder parameter sharing and the dataset sharing defined by RAN1 are both relatively large and cannot be used independently after decoding. It needs to all be connected to make a complete and effective model. So a “Hard” split option without multiple decoding seems more applicable for AI model transfer if the RRC buffer size can be expanded.

Observation 2: “Hard” split RRC segmentation without multiple decode is more applicable for AI model transfer if the RRC buffer size can be expanded.

Proposal 2: If the UE capability of RRC buffer size for AI model transfer can be expanded to the order of magnitude of 100MB, the “hard” split RRC segmentation is used for model transfer in the air-interface.

- Conclusion

In this contribution, we provide our views on the UE side data collection, and the observation and proposals are summarized as follows:

Proposal 1: To analyze the AIML model transfer at RRC level, the 2 split segmentation options discussed for UE radio capability optimization can be considered.

Observation 1: The model transfer size of all the 3 options provided by RAN1 needs more than 1000 segments which greatly exceeds RRC buffer size defined for current UE capability.

Observation 2: “Hard” split RRC segmentation without multiple decode is more applicable for AI model transfer if the RRC buffer size can be expanded.

Proposal 2: If the UE capability of RRC buffer size for AI model transfer can be expanded to the order of magnitude of 100MB, the “hard” split RRC segmentation is used for model transfer in the air-interface.

- References

- R1-2410922 LS on signalling feasibility of dataset and parameter sharing RAN1

- TR37.873 Study on optimizations of UE radio capability signalling; (Release 16)

- R2-2501924 Consideration on UE side data collection CATT