3GPP TSG RAN WG2 #129bis	                                                                             R2-2501928

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT, Turkcell

Title:	Specification impact for AIML mobility

Agenda Item:	8.3.5

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, the following agreement was made [1].

- Agreements on priority Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

In this contribution, we analyze the spec impact of RRM measurement prediction and measurement event prediction from the following aspects.

- Applicability reporting;

- Performance monitoring;

- Data collection;

- Inference;

- Discussion

  - RRM measurement prediction

In this section, we discuss the spec impacts on RRM measurement prediction for both UE sided model and network sided model respectively.

UE-sided model

      - Applicability reporting

In BM case, the following definitions were agreed:

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

We think these definitions can be reused for AI mobility.

Proposal 1: For AI mobility, the definitions including supported functionalities, applicable functionalities and activated functionalities are reused.

For BM case, the following signaling procedure was agreed for applicable functionality reporting for UE sided model. During the procedure, the UE can report its supported functionalities via UE capability.

Proposal 2: For AI mobility, UE reports supported functionalities via UE capability.

For applicable functionalities, the decision can be made by the network side based on additional conditions if configured, UE side additional conditions if any, and whether the model is available in the UE. For AI mobility, the procedure can be similar to that of BM. Consequently, it is proposed that:

Figure 1: signalling procedure on applicable functionality reporting for BM UE-sided model

Regarding network additional conditions for BM, they are used for the UE to understand specific conditions under which the AI/ML model is applicable, such as scenarios, sites, and datasets. For AI mobility, the RSRP can be predicted from measured RSRP. And some network additional conditions are helpful to improve the prediction accuracy, e.g., the antenna configuration, scenarios, etc. Hence, the concept of network additional condition can be used in AI mobility. Like BM, some of the network conditions may not be preferred to be known by the UE. Therefore, RAN2 needs to discuss the content of the network additional conditions, e.g. whether an associated ID is needed for the AI mobility use case, just like for BM.

Proposal 3: RAN2 studies the content of network additional condition for UE-sided model for AI mobility, e.g., whether an associated ID, like that used for BM, is reused for AI mobility.

According to current procedure of BM, the following 2 options are agreed for the applicability report:

- A) The network configures full inference configuration in Step 3;

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported;

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- B) One set or multiple sets of inference related parameters for applicability report only (not for inference);

  - Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

For AI mobility, the applicability report procedure can follow that of BM as baseline and postpone the discussion about the procedure until further progresses are achieved in BM.

Proposal 4: RAN2 postpones the discussion on  applicability report procedure for AI mobility until further progresses are achieved in AI-PHY.

      - Performance monitoring

Regarding BM, RAN1 defines the following options for UE-sided model.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

And RAN1 has agreed to support Type 1 performance monitoring including both Option 1 and Option 2 for performance monitoring of UE-sided model.

For Positioning, RAN1 identified the following options for positioning Case 1.

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Furthermore, RAN1 agreed to support at least Option A for model performance monitoring of AI/ML positioning Case 1.

Considering the monitoring can be performed by network and UE side and it is feasible that the UE can indicate/request/report to the network for performance monitoring, it is proposed that:

Proposal 5: RAN2 starts the study of performance monitoring for AI mobility of UE-sided model including the following options:

-	Performance monitoring is triggered by the network:

-	Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

-	Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

-	Performance monitoring is indicated/requested/reported by the UE.

      - Data collection

The UE-sided model data collection is still under discussion in AI-PHY WID. For AI mobility, it can wait for the progresses in AI-PHY.

Proposal 6: RAN2 postpones the discussion on UE side data collection procedure until further progresses made in AI-PHY.

      - Inference

For AI mobility simulation, the following 3 cases were agreed for cell level measurement prediction model [2]:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Regarding these 3 Sub-use cases, the simulation results show that at least Sub-use case 2 brings benefits on the measurements reduction and the handover performance. And for the other 2 sub-use cases, simulation results from some companies [3]-[5] also show they are beneficial for mobility performance. From spec study point of view, all these 3 sub-use cases should be studied in the SI.

Proposal 7: All the 3 sub-use cases, i.e., Sub-use case 1/2/3 are considered for the specification impact study phase.

Regarding these 3 Sub-use cases, the inputs and outputs of the inference can be summarized in Figure 1.

Figure 1: Input/output association in different sub-use cases

For L1-beam level measurement result(s) is used as input or output of inference, the CSI framework can be used as the configuration of inference input which is similar to the BM case.

Proposal 8: The CSI framework is used as the configuration of inference input for Sub-use case 1/3.

For Cell-level measurement results, it is obtained at Point C based on Figure 2 [6], which is a measurement after processing in the layer 3 filter. It is straightforward that the RRM measurement framework is used as the configuration of the input and output.

Proposal 9: The RRM framework is used as the configuration of inference input for Sub-use case 2.

Figure 2: Measurement model in TS 38.300

Regarding the output of inference, the Cell-level measurement results are predicted via AI/ML procedure for Sub-use case 2/3. The predicted measurement result can be reported to the network via the current measurement report procedure with additional information, e.g. time information for Case A.

It is a bit of complex for Sub-use case 1, since the measurement result(s) is indirectly generated by predicted L1 beam-level measurement result. For instance, the UE selects the top-K beams among all the predicted L1 beam-level results and generates the cell-level results based on the top-K beams. In this case, the RRM measurement framework can be reused for measurement result report..

Proposal 10: The RRM measurement framework is used for the configuration of inference output of Sub-use case 1/2/3.

If Proposal 10 is agreed, the content of the output can follow that of the measurement result which consists of RSRP, RSRQ and SINR. Considering the time information is essential to FR2-FR2 Case A, it can also be included in the measurement report which can be discussed in the WI.

Proposal 11: The inference output includes at least RSRP, RSRQ and SINR. The other information, e.g. time information can be discussed in WI.

Regarding RRM measurement framework, the measurement report can be reported periodically or based on the event. In the following, we analyze the report procedure of predicted measurement report for FR1-FR1 Intra-frequency temporal domain case B, FR2-FR2 Intra-frequency temporal domain case A, inter-frequency domain and spatial domain.

FR2-FR2 Intra-frequency temporal domain case A

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW and the measurement results in the future can be predicted to improve the handover performance.

Generally, the predicated measurement results can be reported periodically or based on event trigger. But different UE behaviors on when to trigger inference brings impacts on how to report the measurement result. Regarding AI mobility, there are 2 options on when to trigger inference:

- Option A: the inference is performed when the real measurement result is obtained as shown in Figure 3;

Figure 3: Inference pattern of Option A

- Option B: the inference is performed when actual measurement event is triggered. For example, when the A3 entry condition is satisfied by actual measurement result, the inference is performed. And the prediction on the future result is performed which is shown in Figure 4.

Figure 4: Inference pattern of Option B

For Option A, predicted measurement results can be obtained periodically considering the inference is performed periodically based on the measurement period. While for Option B, the inference is only performed when TTT is started due to the actual measurement result satisfying the event entry condition. Hence, it can be seen that different trigger mechanisms of inference brings different impacts on measurement report of prediction. Thus, RAN2 needs to study the inference trigger for AI mobility.

FR1-FR1 Intra-frequency temporal domain case B

In intra-frequency temporal domain case B, the measurement is skipped to reduce the measurement overhead and measurement results in PW are predicted by historical measurement result(s) in OW. The UE needs to predict the measurement result for the skipped time instance based on the network configuration. In this case, the UE can report the predicted measurement result periodically or based on the event trigger.

Inter-frequency domain

For inter-frequency, the measurement result of one frequency are predicted by the actual measurement result of another frequency to reduce the measurement overhead of different frequencies. Since the measurement results are continuous predicted by the actual measurement results of the other frequency, the measurement results can either be reported periodically or triggered by measurement event.

Spatial domain

For Spatial domain, AI is applied to predict the skipped beams. As a consequence, the measurement overhead in the spatial domain is reduced. The skipped beams can be predicted continuously. In this case, the UE can report the predicted measurement result periodically or based on the event trigger.

Observation 1: For FR1-FR1 Intra-frequency temporal domain case B, Inter-frequency domain and spatial domain, the UE can report the predicted measurement result periodically or based on event triggers.

Observation 2: For FR2-FR2 Intra-frequency temporal domain case A, when the inference is triggered impacts how to report the predicted measurement results.

Proposal 12: RAN2 to study the inference trigger conditions for Intra-frequency temporal domain case A.

Network sided model

      - Performance monitoring

This is up to the network implementation.

      - Data collection

For network-sided model of BM, data collection for model training is performed according to network control. I.e., the network determines when to start or stop the data collection. And the immediate MDT is used as baseline for data collection. For AI mobility, it can follow the same procedure as agreed in AI-PHY.

Proposal: For data collection of network-sided model, the procedure agreed in AI-PHY can be reused.

For AI mobility, it is complicated to study the content of data collection,, considering there are four cases including FR2-FR2 Intra-frequency temporal domain case A, FR1-FR1 Intra-frequency temporal domain case B, inter-frequency and spatial domain. RAN2 can further study the content logged in the data collection for AI mobility case by case.

Proposal 13: RAN2 further studies the content logged in the data collection for AI mobility case by case.

  - Measurement event prediction

In this section, we discuss the spec impact for measurement event prediction for UE sided model. Both indirect measurement event prediction and direct measurement event prediction are considered, and for indirect measurement event prediction, all the following 4 listed will be considered which have been agreed in previous meeting.

Agreements 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Data collection

In measurement event prediction use case, for all the 4 indirect measurement event prediction, considering that the RRM measurement prediction (temporal, frequency, and spatial domain RRM prediction) model is used, the data collection for training including e.g., data collection procedure, signaling, and collection contents etc. for indirect measurement event prediction are same as RRM measurement prediction.

Proposal 14: For indirect measurement event prediction, the data collection for training, which including e.g., data collection procedure, signaling, and collection contents, is the same as that for RRM measurement prediction.

Data collection for training for UE-side model is currently under discussion in beam management use case in AI PHY WID, and the following agreements were achieved. The data collection configuration in AI/ML based beam management mechanism can be used also to direct measurement event prediction.

Agreements Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

It has been agreed that the CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report i.e., L1 CSI framework is used for data collection configuration for AI/ML based beam management, similarly, for direct measurement event prediction, L3 RRM framework can be used for data collection configuration without measurement reporting, the details of configuration can be further discussed.

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Another issue about data collection for UE-side model for direct measurement event prediction is the collected contents and reporting method. For the collected contents, we understand at least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, whether other information (e.g., UE location) is needed can be further discussed. For reporting method, considering that the data collection reporting for UE-side model is still under study in AI PHY WID, we can wait for the progress.

Proposal 15: For direct measurement event prediction, for UE side data collection,

- Reuse AI/ML based beam management mechanism for UE request and network configuration;

- RRM framework can be used for data collection configuration , details are FFS;

- At least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, other information is FFS;

- Wait for the study progress for the UE-side model data collection reporting in AI-PHY.

Applicability reporting

Similar as data collection for training for indirect measurement event prediction, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the applicability reporting of indirect measurement event prediction is same as RRM measurement prediction.

Proposal 16: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

For direct measurement event prediction, the applicability reporting can take the applicability reporting of AI/ML based beam management as baseline. For applicability reporting for BM UE-sided model, the following steps were proposed.

Figure 1: signalling procedure on applicability reporting for BM UE-sided model

In direct measurement event prediction, for step1 and step2, new UE capability is needed, and the granularity needs to be further considered.

For step 3, RAN2 has agreed to support two options in beam management use case, i.e., Option A (full inference configuration in Step3) and Option B (inference related parameters in Step3), both the two options can also be considered in direct measurement event prediction. Similar as BM use case, for Option A, one or more of ReportConfigNR for inference configuration can be configured in RRM framework, and for Option B, it can be further discussed whether UAI or RRM framework is used. In step4, similar as BM use case, for Option A, the applicable/non-applicable functionalities are reported in RRCReconfigurationComplete for initial reporting; UAI is used for the change of applicability. For Option B, it is FFS.

For the activation of applicable functionalities, for Option A, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 for both periodic and event trigger reporting. For Option B, it is FFS.

Proposal 17: For direct measurement event prediction, for applicability reporting,

- In Step1/Step2, new UE capability is needed, FFS the granularity;

- In Step3/Step 4, both Option A (full inference configuration in Step3) and Option B (inference related parameters in Step3) can be considered:

- For Option A, one or more of ReportConfigNR for inference configuration can be configured in RRM framework, the applicable functionalities are reported in RRCReconfigurationComplete for initial reporting, UAI is used for the change of applicability, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 for both periodic and event trigger reporting;

- For Option B, details are FFS.

Model inference

As mentioned above, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference for indirect measurement event prediction should be same as RRM measurement prediction.

Proposal 18: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

For temporal domain case B, frequency domain, and spatial domain prediction, the goal is measurement reduce, and the measurement reporting is following legacy RRM measurement reporting.  But for the temporal domain case A, which is for enhancing handover performance, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the predicted occurrence time of a certain measurement event.

Proposal 19: For temporal domain case B, frequency domain, and spatial domain indirect measurement event prediction, follow legacy RRM measurement reporting mechanism; for the temporal domain case A, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the predicted occurrence time of a certain measurement event.

In previous RAN2 meeting, we agreed the following agreement for direct measurement event prediction.

Agreements on inputs/outputs and KPIs - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

For direct measurement event prediction, the model input can be measurement results of serving/neighboring cells and the event configuration parameters (e.g., TTT), the model output is the probability of event occurrence within a time window, the model input can also include other information, e.g., UE location etc., therefore, the input/output configuration can be configured from network to UE using RRM framework as mentioned in Proposal 17.

Proposal 20: For direct measurement event prediction, the input/output configuration of model can be configured from network to UE:

- Input configuration: measurement resources of serving/neighbouring cells, event configuration parameters (e.g., TTT), other configuration information is FFS;

- Output configuration: the probability of event occurrence within a time window.

Similar as temporal domain case A indirect measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Proposal 21: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Performance monitoring

In AI/ML based beam management, some agreements were achieved for the performance monitoring. For indirect measurement event prediction, the model performance is up to the model performance of RRM measurement prediction. In RAN2#127bis meeting, the following agreement was achieved for indirect measurement event prediction.

Agreements on inputs/outputs and KPIs - As baseline, we will use RLF event prediction KPI: for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction.

Based on the discussion in RRM measurement prediction section, for indirect measurement event prediction, the UE can report the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): actual RSRP and predicted RSRP, missed event detection, false event detection, true event prediction, true time event reporting triggered and predicted time event reporting triggered;

- Option 2 (UE-assisted performance monitoring): F1 score, RSRP difference, time difference of true time event reporting triggered and predicted time event reporting triggered.

Proposal 22: For indirect measurement event prediction, UE reports the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): actual RSRP and predicted RSRP, missed event detection, false event detection, true event prediction, true time event reporting triggered and predicted time event reporting triggered;

- Option 2 (UE-assisted performance monitoring): F1 score, RSRP difference, time difference of true time event reporting triggered and predicted time event reporting triggered.

In RAN2#127bis meeting, the following agreement was achieved for direct measurement event prediction.

Agreements on inputs/outputs and KPIs - As baseline, we will use RLF event prediction KPI: for direct: F1 score. The following can be reported: missed event detection, false event detection, Continue discussion over email discussion to see if there is a difference.

For direct measurement event prediction, similar as indirect measurement event prediction, the UE can report the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): missed event detection, false event detection, true event prediction;

- Option 2 (UE-assisted performance monitoring): F1 score.

Proposal 23: For direct measurement event prediction, UE reports the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): missed event detection, false event detection, true event prediction;

- Option 2 (UE-assisted performance monitoring): F1 score.

- Conclusion

In this contribution, we provide our views on the specification impact for AIML mobility, and the observations and proposals are summarized as follows:

RRM measurement prediction

UE-sided model: Applicability report

Proposal 1: For AI mobility, the definitions including supported functionalities, applicable functionalities and activated functionalities are reused.

Proposal 2: For AI mobility, UE reports supported functionalities via UE capability.

Proposal 3: RAN2 studies the content of network additional condition for UE-sided model for AI mobility, e.g., whether an associated ID, like that used for BM, is reused for AI mobility.

Proposal 4: RAN2 postpones the discussion on  applicability report procedure for AI mobility until further progresses are achieved in AI-PHY.

UE-sided model: Performance monitoring

Proposal 5: RAN2 starts the study of performance monitoring for AI mobility of UE-sided model including the following options:

-	Performance monitoring is triggered by the network:

-	Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

-	Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

-	Performance monitoring is indicated/requested/reported by the UE.

UE-sided model: Data collection

Proposal 6: RAN2 postpones the discussion on UE side data collection procedure until further progresses made in AI-PHY.

UE-sided model: Inference

Proposal 7: All the 3 sub-use cases, i.e., Sub-use case 1/2/3 are considered for the specification impact study phase.

Proposal 8: The CSI framework is used as the configuration of inference input for Sub-use case 1/3.

Proposal 9: The RRM framework is used as the configuration of inference input for Sub-use case 2.

Proposal 10: The RRM measurement framework is used for the configuration of inference output of Sub-use case 1/2/3.

Proposal 11: The inference output includes at least RSRP, RSRQ and SINR. The other information, e.g. time information can be discussed in WI.

Observation 1: For FR1-FR1 Intra-frequency temporal domain case B, Inter-frequency domain and spatial domain, the UE can report the predicted measurement result periodically or based on event triggers.

Observation 2: For FR2-FR2 Intra-frequency temporal domain case A, when the inference is triggered impacts how to report the predicted measurement results.

Proposal 12: RAN2 to study the inference trigger conditions for Intra-frequency temporal domain case A.

Network-sided model: Data collection

Proposal 13: RAN2 further studies the content logged in the data collection for AI mobility case by case.

Measurement event prediction

Data collection

Proposal 14: For indirect measurement event prediction, the data collection for training, which including e.g., data collection procedure, signaling, and collection contents, is the same as that for RRM measurement prediction.

Proposal 15: For direct measurement event prediction, for UE side data collection,

- Reuse AI/ML based beam management mechanism for UE request and network configuration;

- RRM framework can be used for data collection configuration , details are FFS;

- At least the cell/beam measurement results and event configuration parameters (e.g., TTT) can be logged, other information is FFS;

- Wait for the study progress for the UE-side model data collection reporting in AI-PHY.

Proposal 16: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

Proposal 17 For direct measurement event prediction, for applicability reporting,

- In Step1/Step2, new UE capability is needed, FFS the granularity;

- In Step3/Step 4, both Option A (full inference configuration in Step3) and Option B (inference related parameters in Step3) can be considered:

- For Option A, one or more of ReportConfigNR for inference configuration can be configured in RRM framework, the applicable functionalities are reported in RRCReconfigurationComplete for initial reporting, UAI is used for the change of applicability, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 for both periodic and event trigger reporting;

- For Option B, details are FFS.

Model inference

Proposal 18: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

Proposal 19: For temporal domain case B, frequency domain, and spatial domain indirect measurement event prediction, follow legacy RRM measurement reporting mechanism; for the temporal domain case A, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the predicted occurrence time of a certain measurement event.

Proposal 20: For direct measurement event prediction, the input/output configuration of model can be configured from network to UE:

- Input configuration: measurement resources of serving/neighbouring cells, event configuration parameters (e.g., TTT), other configuration information is FFS;

- Output configuration: the probability of event occurrence within a time window.

Proposal 21: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Performance monitoring

Proposal 22: For indirect measurement event prediction, UE reports the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): actual RSRP and predicted RSRP, missed event detection, false event detection, true event prediction, true time event reporting triggered and predicted time event reporting triggered;

- Option 2 (UE-assisted performance monitoring): F1 score, RSRP difference, time difference of true time event reporting triggered and predicted time event reporting triggered.

Proposal 23: For direct measurement event prediction, UE reports the following information to network for performance monitoring:

- Option 1 (NW-side performance monitoring): missed event detection, false event detection, true event prediction;

- Option 2 (UE-assisted performance monitoring): F1 score.

- References

- R2-2501702, RAN2#129 Meeting Report;

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR;

- R2-2408326, Discussions and evaluations on RRM measurement prediction;

- R2-2408925, Simulation results for RRM measurement predictions;

- R2-2408520, Evaluation on RRM measurement prediction;

- TS 38.300, NR and NG-RAN Overall Description; Stage 2.