3GPP TSG-RAN WG2 Meeting #129bis	R2-2501931

Wuhan, China, 7th– 11th April 2025

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Further discussion on Conditional LTM

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for intra-CU conditional LTM are discussed.

Discussion

TA maintenance and TA timer handling

In the last meeting, the new TA MAC CE and TAT handling related agreements were made:

CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid. If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM The CLTM TAT is per candidate configuration.

Based on the online discussion and offline discussion [AT129][113][MOB] MAC open issues, two leftover issues are still pending:

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

How to handle the CLTM TAT for target cell upon CLTM cell switch execution to the target cell. Offline agreement: CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR - Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; - Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. - Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. Will be revisited next meeting.

CTAT handling for target/new serving cell

The TAT timer is used to determine the TA validation period. In legacy behavior, a TAT associated with pTAG is started for the serving cell when the UE receives the TA from the network. This timer is used to maintain the serving cell TA and is always synchronized between the network and the UE.

When the UE performs a CLTM cell switch to the target cell, the target cell becomes the serving cell, and the pTAG should also be started to avoid spec changes to the legacy UL synchronization maintenance procedure for serving cell.

Observation 1: The pTAG TAT timer should be started to avoid spec changes to the legacy UL synchronization maintenance procedure.

However, the TAT value of pTAG should not be started from a pre-configured TA value (e.g., the IE timeAlignmentTimer) from network, as the TA value is not received when the CLTM cell switch occurs. The TA validation period starts when the UE receives the TA MAC CE for the target cell (while it is still one of the candidate cells), and the associated CLTM TAT timer starts. Therefore, the value of the pTAG timer should inherit from the CLTM TAT timer of the target cell when the UE performs the CLTM cell switch to that cell.

Proposal 1: When UE perform CLTM cell switch to the target cell, the timeAlignmentTimer associated with PTAG is started. The remaining time of the CLTM TAT timer for target cell is used for the pTAG TAT if RACH-less CLTM is performed. Otherwise (RACH-based), the TAT of pTAG start from a pre-configured TAT value as legacy.

Regarding the CLTM TAT timer, it is no longer used for the serving cell, so it doesn't matter if the timer is stopped or maintained during the current cell. There are two possible options:

- Option 1: The CLTM TAT timer of the target cell (new serving cell) is stopped.

- Option 2: The CLTM TAT timer of the target cell (new serving cell) is maintained until it expires. When the timer expires, the UE does not perform any special behavior.

In some scenarios, there may be case that UE performs LTM cell switch fast from other cell->A->B->A, and the TAT timer is long enough. In such cases, the running CLTM TAT timer of cell A may not expire when the first time A is becoming serving cell and could still be useful for the subsequent CLTM cell switch to cell A. If the CLTM TAT timer is not running, a RACH procedure is needed during CLTM execution. If the CLTM TAT timer is still running, a RACH-less CLTM can be performed.

Considering this case, maintaining the CLTM TAT timer may be useful for subsequent LTM. Therefore, we prefer Option 2.

Proposal 2: When the UE performs CLTM RACH-less cell switch to the target cell, the CLTM TAT timer of that cell is maintained until it expires. When the timer expires, the UE does not perform any special behavior.

CTAT handling for other candidate cells

For CTAT handling of other candidate cells, as analysed above, maintaining the CLTM TAT timer may be useful for subsequent LTM if the cell switch is fast and the TAT is long. Therefore, the CTAT timer should be maintained and continue running until it expires. When the timer expires, the UE should not perform any special behavior, as the timer is synchronized between the network and the UE. The network can trigger another PDCCH-ordered early RACH if needed.

Proposal 3: When the UE performs CLTM cell switch to the target cell, the CLTM TAT timer of other candidate cells (if any) are maintained until expiration. When the timer expires, the UE does not perform any special behavior.

Candidate TA maintenance and refresh

As early UL synchronization is critical for latency reduction during LTM, we may further consider enhancement to refresh the candidate TA if the TA timer expires before the conditional cell switch triggered. A typical approach is to send another TA MAC CE to update the TA value. However, the TA value is used for the synchronization of uplink transmission to adjust the timing from UE so that the signalling arrives at the gNB at the correct instance. The expiration of the TA may indicate that the UE's position has changed and the network is unaware. Therefore, another preamble may be needed to inform the network about the change.

Since both the UE and the network are synchronized with the TA timer, they should be aware of the TA expiration and can feasibly trigger the update procedure. For example, the network may send a PDCCH order to trigger the UE to send another preamble.

Proposal 4: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch triggered, the network can trigger TA refresh by sending another PDCCH order to UE.

Target beam selection

In the last meeting, the following agreement are made regarding target beam selection for Conditional LTM:

For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

However, the following procedure after UE selecting target beam is still unclear, depending on different cases. Let’s consider L1 based C-LTM first.

L1 based C-LTM

For L1 based conditional LTM, there may be different cases if a target beam is selected based on the triggering condition.

- Case 1: The early synchronization is not performed yet.

- Case 2: Only Early TCI activation is done.

- Case 3: Early TCI activation + early UL sync is done (assuming TA is still valid when C-LTM is triggered).

In case 2 and 3, there are both two subcases that the selected target beam is/is NOT one of the activated beam in early TCI activation. The detail UE behaviors of each cases can be summarized in the table as follow:

Table1: UE behavior during conditional LTM execution in different scenarios

|  | Target beam is activated beam | Target beam is NOT activated beam |
| --- | --- | --- |
| No early sync | / | DL sync + UL sync (RACH) |
| Only Early TCI activation | FFS DL sync + UL sync (RACH) | DL sync +UL sync (RACH) |
| Early TCI activation + early UL sync | FFS DL sync + RACH-less LTM | DL sync + FFS RACH or RACH-less |

In case1, if early synchronization is not performed when conditional LTM is triggered, the UE will perform both DL and UL synchronization during the cell switch, similar to CHO behavior.

In case 2, if only early TCI activation is performed without early UL synchronization, the UE will perform RACH-based LTM during the cell switch. If the selected target beam is not an activated beam, the UE will perform DL synchronization (TF tracking) to the selected target beam.

However, if the selected target beam is an activated beam, whether the UE can skip DL synchronization (TF tracking) depends on RAN4 requirements.

Observation 2: If the selected target beam is NOT an activated beam, the UE needs to perform DL synchronization (TF tracking) to the selected target beam during conditional LTM execution.

Observation 3: If the selected target beam is an activated beam, whether the UE performs DL synchronization (TF tracking) to the selected target beam during conditional LTM execution depends on RAN4 requirements.

In case 3, both early TCI activation + early UL sync is performed, and the TAT does not expire when C-LTM is triggered. Since there is no RACH during LTM execution, UE select the target beam and does not indicate it to the network.

If the selected target beam is an activated beam, RACH-less LTM can be performed as the UE can use the activated beam prepared by the network in the early TCI state activation. The network will reserve (monitor) all activated beams provided to the UE, meaning that the network sends RSs for each activated beam in the downlink and monitors the uplink resources for any potential first UL transmission. (The network can adjust the TCI states provided in the early TCI state MAC CE according to its resource utilization.) In this case, as long as the TA is still valid, the first UL message can be sent to the network.

Observation 4: If the UE-selected target beam is an activated beam and the TA of the target cell is still valid, the first UL message can be sent to the network with RACH-less LTM.

However, if the selected target beam is not an activated beam, it means the network does not reserve the uplink resources of that beam for this UE. The network may assume the UE will use another activated beam and not monitor this beam for UL transmission. In this case, even if the UE has a valid TA for that target cell, the UE may not successfully send the first UL message to the network without further procedures.

Observation 5: If the UE-selected target beam is not an activated beam and the TA of the target cell is valid (via early UL synchronization), the first UL message may not be successfully sent to the network, as the network may not monitor or reserve the uplink resource for this inactive beam.

Therefore, we would like to verify if Observation 5 is correct, i.e., whether the UE is allowed to use any beam (including inactive beams) as the target beam to send the first UL message without RACH, if the TA of the target cell is valid. This may depend on the network's resource allocation policy. I.e., whether network prepares the resources in LTM candidate configuration but not belong to activated beams.

Proposal 5: RAN2 to discuss and confirm whether UE is allowed to use any beam (including inactive beam) as target beam to send the first UL message without RACH, if the TA of the target cell is valid (via early UL sync).

If the answer to proposal 5 is no, RAN2 may need to further discuss the UE behavior in this case. the UE may perform RACH to the network again and send a preamble using this inactive target beam to inform the network of the UE’s decision.

Another approach is to decouple the event condition and target beam selection. The UE can use another activated beam with the best quality to send the first UL message, and then the network can adjust the target beam after the LTM cell switch is complete (via TCI States Activation/Deactivation MAC CE). It is noted that this solution revises the last meeting agreement.

Proposal 6: If the answer to proposal 5 is no, RAN2 to specify the UE behavior when the selected target beam is not an activated beam (in the scenario where early UL synchronization is performed and the TA is valid):

- Solution 1: The UE performs random access again and uses the selected target beam to send a preamble to inform the network.

- Solution 2: The UE selects the target beam with the best quality among the activated beams. UE does not select an inactive beam as target beam.

L3 based C-LTM

For conditional LTM with L3 measurement configured, besides the potential issue mentioned above, another issue is that it is hard to define the target beam due to the beam consolidation and L3 filtering based on the L3 measurement results. Even without these configurations, selecting a target beam can still be difficult. For example:

- From TTT=0~50ms, beam 1 (of cell 1) above the threshold.

- From TTT=40~130ms, beam 2 (of cell1) above the threshold.

- From TTT=120ms~160ms, beam 1 (of cell 1) above the threshold again.

When TTT=160ms (TTT length), the event condition is fulfilled and the cell 1 is selected as target cell. In this case, it is hard to say if beam1 or beam2 is the target beam.

Observation 6: For L3 based conditional LTM, the target beam cannot be determined with beam consolidation and L3 filtering. Even without these configurations, it is hard to determine the target beam with the best quality.

Therefore, we suggest leaving this decision to UE implementation.

Proposal 7:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

UL grant

In the last meeting, it was agreed that the DG based UL grant acquisition is not supported in R19 conditional LTM.

Do not support DG-based first UL transmission for RACH-less conditional LTM.

For the CG based RACH-less conditional LTM, as legacy way, the UL grant is configurated via RRC, and there should be no issue for UE to send the UL message. However, considering the benefits of resource utilization, the UL grant may also be indicated via MAC-CE with less resource pre-allocation.

From the agreement in last meetings, if PDCCH ordered early TA acquisition is performed, UE receives a TA MAC-CE from source cell after sending RACH preamble to the candidate cell. From a feasibility perspective, the UL grant can also be included in this TA MAC CE, as it indicates the network’s “expectation” for UE to select this candidate cell as target cell. When the network prepares the TA for this candidate cell, the UL grant can also be prepared and included.

However, this requires extra coordination between the source and target cells, as the TA MAC CE is delivered from the source cell to the UE. Considering the possible benefits of resource utilization and the potential impact and complexity, RAN2 can discuss whether to support indicating UL grant in the TA MAC CE.

Proposal 8: For CG based RACH-less conditional LTM, the UL grant can be indicated in LTM candidate configuration via RRC. RAN2 can further discuss if the UL grant can be also included in the TA MAC-CE.

The time to stop evaluating triggering condition

In the last meetings, RAN2 agreed the UE starts execution condition evaluation once it receives CLTM configuration:

The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition

There remain discussions on the time to stop these triggering event evaluations.

To make subsequent conditional LTM work, UE shall continue to evaluate the triggering condition for the next LTM cell switch after one LTM cell switch is performed, except for the serving cell. If network want to stop UE from evaluating for the triggering condition, extra signalling should be sent to UE, like conditionals handover. The signalling may be used to remove part of conditional LTM configuration/condition (to ban part of candidate cells) or remove all of them, which based on network implementation.

Proposal 9: UE continue to evaluate the triggering condition for the next conditional LTM cell switch after one LTM cell switch is performed.

Proposal 10: Introduce conditional LTM removal signalling to remove part of/all CLTM configuration and conditions (similar to CHO). UE stop evaluating the conditions for one candidate cell if it is removed by network.

MR in conditional LTM

The measurement report in conditional LTM was not discussed before. In our understanding, although the LTM execution is determined by UE, the early TCI activation/ early UL sync are still triggered by network, and L1/L3 measurement report is still critical for network to understand UE’s channel condition and determine the candidate cell(s) which are more likely to be the target cell. Therefore, the L1/L3 measurement report in R18 LTM can be reused as baseline in Conditional LTM.

Proposal 11: L3 based measurement report, R18 L1 measurement report and R19 event triggered L1 measurement report are supported for conditional intra-CU LTM.

Conclusion

In this contribution, the following observations and proposals are made:

TA maintenance and TA timer handling

Observation 1: The pTAG TAT timer should be started to avoid spec changes to the legacy UL synchronization maintenance procedure.

Proposal 1: When UE perform CLTM cell switch to the target cell, the timeAlignmentTimer associated with PTAG is started. The remaining time of the CLTM TAT timer for target cell is used for the pTAG TAT if RACH-less CLTM is performed. Otherwise (RACH-based), the TAT of pTAG start from a pre-configured TAT value as legacy.

Proposal 2: When the UE performs CLTM RACH-less cell switch to the target cell, the CLTM TAT timer of that cell is maintained until it expires. When the timer expires, the UE does not perform any special behavior.

Proposal 3: When the UE performs CLTM cell switch to the target cell, the CLTM TAT timer of other candidate cells (if any) are maintained until expiration. When the timer expires, the UE does not perform any special behavior.

Proposal 4: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch triggered, the network can trigger TA refresh by sending another PDCCH order to UE.

Target beam selection

Observation 2: If the selected target beam is NOT an activated beam, the UE needs to perform DL synchronization (TF tracking) to the selected target beam during conditional LTM execution.

Observation 3: If the selected target beam is an activated beam, whether the UE performs DL synchronization (TF tracking) to the selected target beam during conditional LTM execution depends on RAN4 requirements.

Observation 4: If the UE-selected target beam is an activated beam and the TA of the target cell is still valid, the first UL message can be sent to the network with RACH-less LTM.

Observation 5: If the UE-selected target beam is not an activated beam and the TA of the target cell is valid (via early UL synchronization), the first UL message may not be successfully sent to the network, as the network may not monitor or reserve the uplink resource for this inactive beam.

Proposal 5: RAN2 to discuss and confirm whether UE is allowed to use any beam (including inactive beam) as target beam to send the first UL message without RACH, if the TA of the target cell is valid (via early UL sync).

Proposal 6: If the answer to proposal 5 is no, RAN2 to specify the UE behavior when the selected target beam is not an activated beam (in the scenario where early UL synchronization is performed and the TA is valid):

- Solution 1: The UE performs random access again and uses the selected target beam to send a preamble to inform the network.

- Solution 2: The UE selects the target beam with the best quality among the activated beams. UE does not select an inactive beam as target beam.

Observation 6: For L3 based conditional LTM, the target beam cannot be determined with beam consolidation and L3 filtering. Even without these configurations, it is hard to determine the target beam with the best quality.

Proposal 7:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

UL grant

Proposal 8: For CG based RACH-less conditional LTM, the UL grant can be indicated in LTM candidate configuration via RRC. RAN2 can further discuss if the UL grant can be also included in the TA MAC-CE.

The time to stop evaluating triggering condition

Proposal 9: UE continue to evaluate the triggering condition for the next conditional LTM cell switch after one LTM cell switch is performed.

Proposal 10: Introduce conditional LTM removal signalling to remove part of/all CLTM configuration and conditions (similar to CHO). UE stop evaluating the conditions for one candidate cell if it is removed by network.

MR in conditional LTM

Proposal 11: L3 based measurement report, R18 L1 measurement report and R19 event triggered L1 measurement report are supported for conditional intra-CU LTM.

References

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2_128_R16R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM