3GPP TSG-RAN WG2 #129bis	R2-2501932

Wuhan, China, 7th  – 11th  April 2025

Agenda item:	7.2

Source: 	MediaTek Inc.

Title: 	Correction on LTM UE Capability based on the RAN4 LS

Document for:	Discussion

1 	Introduction

In this paper, we discuss the corrections on LTM UE Capability based on the coming RAN4 LS and the updated RAN4 feature list for Rel-18 LTM.

2 	Discussion

2.1 Correction on LTM UE Capability based on the RAN4 LS

In the coming LS from RAN4 (R4-2502619), the UE capability for L1-RSRP measurement in LTM (FG 39-3-1, FG 39-3-3, FG 39-3-5 and FG 39-3-6) are discussed and the UE feature list are updated.

Therefore, changes should be made to TS 38.306 and TS 38.331 accordingly.

2.1.1 Correction to TS 38.306

The updated description in the RAN4 feature list can be reflected in TS 38.306 components description as follows.

TP for TS 38.306

---Start of TP ---

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxFreqLayersL1-Meas-r18 Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. Only frequency layers which are configured with SSB-based L1-RSRP measurement on neighbor cell(s) by LTM-CSI-ResourceConfig-r18 are counted. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxCellsWithoutGapsL1-Meas-r18 Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement are counted, regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| maxSSB-PerFreqLayerL1-Meas-r18 Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. LTM candidate cells and serving cells are counted regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-L1-Meas-r18 Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. Both LTM candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are counted, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |

---End of TP ---

Proposal 1: Adopt the TP of TS 38.306 for the changes in the updated UE feature list.

2.1.2 Correction to TS 38.331

According to the updated RAN1 feature list, the values supportedMaxSSB-PerFreqLayerWithoutGaps-r18 of component maxSSB-PerFreqLayerL1-Meas-r18 in FG 39-3-5 should be extended from {1,2,3,4,5,6,7,8} to {1,2,3,4,5,6,7,8,12,16, 20, 24}.

Since ASN.1 is frozen, the correction should be made in BC ways.

TP for TS 38.331

---Start of TP ---

CA-ParametersNR-v1800 ::= SEQUENCE {

<Skip unchanged IEs>

-- R4 39-3-5: Number of SSB resources for L1-RSRP measurement per frequency layer

dummy                        SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-r18            INTEGER (1..8)                            OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18               INTEGER (1..8)                            OPTIONAL

}                                                                                                   OPTIONAL,

-- R4 39-3-5: Number of SSB resources for L1-RSRP measurement per frequency layer

maxSSB-PerFreqLayerL1-Meas-r18                        SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-r18            ENUMERATED (n1,n2,n3,n4,n5,n6,n7,n8,n12,n16,n20,n24,spare4,spare3,spare2,spare1)                            OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18               INTEGER (1..8)                            OPTIONAL

}                                                                                                   OPTIONAL,

---End of TP ---

Proposal 2: Adopt the TP of TS 38.331 for the changes in the updated UE feature list.

3 	Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1: Adopt the TP of TS 38.306 for the changes in the updated UE feature list.

Proposal 2: Adopt the TP of TS 38.331 for the changes in the updated UE feature list.

4 	References

[1] R4-2502619 LS on UE capability for L1-RSRP measurement in LTM

[2] TS 38.306 V18.5.0

[3] TS 38.331 V18.5.0

Annex A – The updated RAN4 UE feature list for R18 LTM

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 39-3-1 | Number of frequency layers for L1-RSRP measurement | - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. Only frequency layers which are configured with SSB-based L1-RSRP measurement on neighbor cell(s) by LTM-CSI-ResourceConfig-r18 are counted. - The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list and/or 39-2 2. Component 2: 39-2a | 1. Candidate values Component 1: {1,2,3,4,5,6,7,8} 2. Candidate values Component 2: {1,2,3,4,5,6,7,8} | Mandatory with capability signaling if UE supports 45-1, 39-2 and/or 39-2a |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement are counted, regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | 45-1 from RAN1 Rel-18 feature list or 39-2 | candidate values: {1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24} Note: when same PCI is present in serving and candidates, one PCI is counted only once in total cells | Optional with capability signaling |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. LTM candidate cells and serving cells are counted regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list or 39-2 2. Component 2: 39-2a | Candidate value of Component 1: {1,2,3,4,5,6,7,8,12,16,20, 24} Candidate value of Component 2: {1,2,3,4,5,6,7,8} | Optional with capability signaling |

| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. Both LTM candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are counted, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | 45-1 from RAN1 Rel-18 feature list or 39-2 | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- |