3GPP TSG-RAN WG2 Meeting #129b	R2-2501934

Wuhan, China, 7 - 11, Apr, 2025

Agenda Item:	8.1.4

Source:	Xiaomi

Title:	Discussion on UE-side data collection

Document for:	Discussion

Introduction

For UE side model, data collection is essential to collect data to train AI model. In [1], following work is expected for RAN2,

For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

In this contribution, we discuss open issues of UE-side data collection.

Discussion

Reply LS to SA5

During RAN #106 meeting, RAN has sent LS to SA WGs asking SA to start study UE-side data collection methods in [2]. SA5 send reply LS in [3] and ask RAN and RAN2 to confirm SA5’s understanding. SA5’s understanding is showed as following,

SA5 would like to thank TSG RAN for their LS in RP-243316 on AI/ML UE sided data collection. SA5 has discussed the questions about the feasibility for transfer of data over UP for Solution 3 (UE-> gNB->OAM -> Server). SA5 has made the following analysis about the termination points of the UP tunnel for this solution: 1. If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a); 2. If the UP tunnel is between UE and 5GC, SA5 does not see any difference between solution 3 and solution 2. SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. SA5 would like to get confirmation of this assumption from TSG RAN and RAN2, as well as RAN’s perspective on the UP tunnel termination points. Based on the assumption above, SA5 would like to confirm that it is feasible to develop the OAM solution with the necessary controllability and different levels of visibility for transfer of UE side data over UP using solution 3.

It can be seen SA5 assumes there is UP tunnel between UE and gNB. However, there is no UP tunnel between gNB and UE in current spec. We propose RAN2 to send reply LS to correct SA5’s assumption.

Proposal 1: Send reply LS to SA5 to indicate current specification doesn’t support UP tunnel between UE and gNB.

NG-RAN involvement in the control and configuration

During RAN2 #128 meeting, following agreements training data collection configuration for UE-side model:

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. - The network can provide the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. - FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

For UE side model, the data collection is triggered by UE or UE side server. The desired data collection for UE-side model may require NW to provide a specific configuration, e.g. set A or set B. If the desired configuration has been provided by NW, UE can collect the data without additional indication to NW. In this method, there is no impact to NW. However, if the desired data can’t be collected in current network configuration, UE can send request NW to provide the desired configuration and perform UE-side data collection upon receiving network configuration.

Proposal 2: For UE side data collection, if current configuration is not aligned with the desired configuration, UE requests desired configuration for data collection to NW. If NW has provided the desired configuration, UE can start data collection directly without indication to NW.

Some company raised the concern that the requested configuration may be over NW’s capability. Such request may result in unnecessary signalling waste since NW would never provide the requested configuration.

First of all, as mentioned in above agreements, RAN2 agreed that UE can send a request for UE-side data collection and NW can configure UE the allowance to request data configuration. Such allowance indication can avoid UE to request data collection configuration to a NW which doesn’t support UE-side data collection.

Next question is whether there is a case that gNB supports AI/ML beam management, but doesn’t support a specific beam set A/B configuration. We understand this may be possible. gNB may only support limited associated IDs. Therefore, UE may request an associated ID which is not supported by the serving gNB. Whether there are other inference configurations that can’t be supported by a specific gNBs can be up to RAN1.

To avoid UE requesting configurations that are not supported by the network, network can indicate the supported configurations for UE-side data collection, e.g. associated IDs, to UE. UE only requests the configuration supported by NW.

Proposal 3: UE requests configurations and/or associated IDs for data collection, only if the configurations and/or associated IDs are supported by NW. Detail of configuration is up to RAN1.

Proposal 4: Send LS to RAN1 to confirm it’s possible that a specific serving gNB may not support all associated IDs and ask whether there are other inference configurations that a specific serving gNB may not support.

For UE-side training data collection, different from RRM measurement and NW-side training data collection, it is better to allow UE have more controllability of its own model’s training data collection procedure.

First of all, during UE-side training data collection, it is possible that UE may want to terminate its own training data collection based on its implementation, for example:

- When UE considers itself has enough data collected

- When UE is overheated/low power state and wants to temporarily stop training data collection for UE-side model

- When UE want to prioritize other services compared to this latency non-critical training data collection

Moreover, UE-side training data collection is mainly collected to serve for UE-side model, where either UE or UE-side OTT server has a clear understanding and requirement on when and how to collect such data. However, NW is not aware of whether UE has stopped UE-side data collection and may continue to provide the requested configuration. The requested configuration may not be the optimal configuration for NW and results in radio resource waste. To save radio resources over the air interface, UE can inform network to release UE-side data collection configuration if UE does not need it. Upon receiving the indication, NW can release the UE-side training data collection configuration.

Proposal 5: When UE stops UE sided training data collection, UE indicates NW to release the corresponding training data collection configuration.

UE sided data transfer

During RAN #106 meeting, RAN has sent LS to SA WGs asking SA to start study UE-side data collection methods in [2]. It is observed that, for the method of data transfer over UP or NAS, there are some dependency on SA study. To avoid duplicate discussion, RAN2 can wait for SA progress and check whether there is any impact to RAN2 based on SA conclusion.

Proposal 6: For UE sided data transfer over UP or NAS, RAN2 waits for other WG’s progress.

Another data transfer option is data transfer over RRC, where scalability aspects of RRC segment number should be studied by RAN2. In current specification, UL RRC message can support at most 16 segments and each segment has a maximum size of 9000 bytes.

ULDedicatedMessageSegment message -- ASN1START -- TAG-ULDEDICATEDMESSAGESEGMENT-START ULDedicatedMessageSegment-r16 ::= SEQUENCE { criticalExtensions CHOICE { ulDedicatedMessageSegment-r16 ULDedicatedMessageSegment-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } ULDedicatedMessageSegment-r16-IEs ::= SEQUENCE { segmentNumber-r16 INTEGER (0..15), rrc-MessageSegmentContainer-r16 OCTET STRING, rrc-MessageSegmentType-r16 ENUMERATED {notLastSegment, lastSegment}, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL }

## 4.3 Services ### 4.3.1 Services provided to upper layers The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers: - transfer of user plane data; - transfer of control plane data; - header compression; - uplink data compression; - ciphering; - integrity protection. The maximum supported size of a PDCP SDU is 9000 bytes. The maximum supported size of a PDCP Control PDU is 9000 bytes.

Based on above observation, the maximum size of one RRC message supported in UL is 144 KB. However, the size of training data could be very large. According to [4], RAN1 indicated the size of training data for CSI can be 225 MB and even larger if the dataset consisting of supporting multiple configurations. Similar size of training dataset is expected for beam management and positioning use case. Assuming 225MB is considered as an average size for UE-side training data collection, it requires 1500 segments or even more if data size is larger. It’s impossible for single RRC message to carry the UE sided data in this massive number of segments.

Observation 1: Thousands of segments are required to transfer UE sided data over RRC message.

Even if the number of RRC segments can be extended, RRC message transmission would require a much longer time. Handover may occur during the RRC message transmission. In current specification, SRB doesn’t support service continuity during handover, where UE has to retransmit the data from the first RRC segment again in target cell, which results in significant signalling overhead and unnecessary waste of air interface resources.

Observation 2: Significant signalling overhead and waste of air interface resource may be expected after handover, as service continuity in SRB is not supported and UE needs to retransmit all the collected data.

Furthermore, current SRB0/1/2 may have higher priority than DRB, and SRB1 has even higher priority than SRB2. Considering the size of training data collection, the data transfer over RRC message may not be finished in a short time and further block the transmission of other RRC message transmission . Transmission of other RRC message or user plane data may be delayed. To avoid the blocking issue, new SRB with lower priority may be required for training data transfer.

Observation 3: New SRB with lower priority is needed to support UE sided data transfer over RRC.

Based on the above observations, it’s challenging to resolve all above listed issues in R19. Considering the time and work load, we propose RAN2 do not support UE sided data transfer over RRC in R19.

Proposal 7: RAN2 do not support UE sided data transfer over RRC in R19.

UE side data collection for Pos

During RAN1 #116bis meeting, RAN1 made following agreements on PRS configuration for UE-side data collection [2]:

Agreements: For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: - Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. - Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

According to the RAN1 agreements, UE may initiate on-demand PRS procedure to request PRS configuration for training data collection.

It is further observed that RAN2 agreed UE can send a request to NW for BM UE-side data collection during RAN2 #128 meeting:

- For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Therefore, similar to beam management UE-sided model, UE can send a request to LMF for UE-side data collection by sending an on-demand PRS request.

Proposal 8: For DL PRS configuration for training data collection, UE sends on-demand PRS request to request LMF for training data collection.

Furthermore, due to completion of data collection or other UE conditions (e.g. low power, low buffer, etc), UE may want to stop UE-side data collection. Such behavior should also be reported to LMF, where LMF can stop delivering assistance data for UE-side data collection when it receives this indication.

Proposal 9: UE sends an indication to the LMF when UE wants to stop UE-side data collection.

Conclusion

Based on the discussion in section 2, we have following proposals:

Relay LS to SA5

Proposal 1: Send reply LS to SA5 to indicate current specification doesn’t support UP tunnel between UE and gNB.

NG-RAN involvement in the control and configuration

Proposal 2: For UE side data collection, if current configuration is not aligned with the desired configuration, UE requests desired configuration for data collection to NW. If NW has provided the desired configuration, UE can start data collection directly without indication to NW.

Proposal 3: UE requests configurations and/or associated IDs for data collection, only if the configurations and/or associated IDs are supported by NW. Detail of configuration is up to RAN1.

Proposal 4: Send LS to RAN1 to confirm it’s possible that a specific serving gNB may not support all associated IDs and ask whether there are other inference configurations that a specific serving gNB may not support.

Proposal 5: When UE stops UE sided training data collection, UE indicates NW to release the corresponding training data collection configuration.

UE sided data transfer

Proposal 6: For UE sided data transfer over UP or NAS, RAN2 waits for other WG’s progress.

Observation 1: Thousands of segments are required to transfer UE sided data over RRC message.

Observation 2: Significant signalling overhead and waste of air interface resource may be expected after handover, as service continuity in SRB is not supported and UE needs to retransmit all the collected data.

Observation 3: New SRB with lower priority is needed to support UE sided data transfer over RRC.

Proposal 7: RAN2 do not support UE sided data transfer over RRC in R19.

UE side data collection for Pos

Proposal 8: For DL PRS configuration for training data collection, UE sends on-demand PRS request to request LMF for training data collection.

Proposal 9: UE sends an indication to the LMF when UE wants to stop UE-side data collection.

Reference

[1] RP-243307 Way forward on AIML UE sided data collection

[2] RP-243316 LS on AIML UE sided data collection

[3] RP-250030 Reply LS to RP-243316 on AI/ML UE sided data collection

[4] R1-2410922 LS on signalling feasibility of dataset and parameter sharing