3GPP TSG-RAN WG2 Meeting #129b	                                          R2-2501935

Wuhan, China, 7 – 11, Apr, 2025

Source: 	Xiaomi

Title:  	Discussion on generalization performance over cell configuration

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

Introduction

In last meeting, it’s agreed to study the generalization over cell configuration. Following generalization evaluation methodology is agreed

Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

In this contribution, we provide simulation results of generalization performance over cell configuration for FR1 and FR2. Some observations are given.

Discussion

FR1 generalization performance over cell configuration

Following two sets of cell configurations were agreed for generalization evaluation on FR1,

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 1: generalization parameters for FR1

Based on the generalization evaluation methodology, we carried out simulation of following scenarios accordingly,

| Scenario | Training @Dataset: Configuration #1 | Training @Dataset: Configuration #2 | Inference @Dataset: Configuration #1 | Inference @ Dataset: Configuration #2 |
| --- | --- | --- | --- | --- |
| Baseline |  | Yes |  | Yes |
| CG1 | Yes |  |  | Yes |
| GC2 | Yes | Yes |  | Yes |

It’s up to company to select the prediction pattern, i.e. PW an OW. We select multiple patterns to check the generalization performance. The UE speed is set to 60km/h. Following table shows the generalization performance degradation.

| OW/PW (ms) | 80/80 | 80/160 | 120/120 | 200/400 |
| --- | --- | --- | --- | --- |
| Baseline RSRP diff(dB) | 0.75 | 1.11 | 0.95 | 2.09 |
| GC#1 RSRP diff (dB) | 0.83 (0.08 degradation) | 1.35 (0.24) | 1.098 (0.13) | 2.63 (0.54) |
| GC#2 RSRP diff (dB) | 0.78 (0.03 degradation) | 1.24 (0.13) | 1.02 (0.07) | 2.39 (0.3) |

In general, we can observe the performance degradation of GC#2 is less than GC#1, regardless MRRT and PW/OW length. This is also aligned with the generalization evaluation over UE speed. We understand the reason is training data set of GC#2 is partly overlapped with training data set of baseline. Such overlap gives some advantage to GC#2 over GC#1.

Observation 1: GC#1 outperforms GC#2 over cell configuration on FR1 case B.

If MRRT is fixed to 50% and PW/OW length increases from 80 to 120, GC#1 and GC#2 degradation compared with baseline doesn’t change much, i.e. from 0.08db to 0.13db for GC#1 and from 0.03db to 0.07db for GC#2. The generalization performance is good. If MRRT is fixed to 66%, GC#1 and GC#2 degradation compared with baseline increases, i.e. increases from 0.24db to 0.54db for GC#1 and increases from 0.13db to 0.3db for GC#2.

Observation 2: If MRRT is small, there is small performance degradation compared with baseline for both GC#1 and GC#2 on FR1 case B.

Observation 3: Larger MRRT results in larger performance degradation compared with baseline for both GC#1 and GC#2 on FR1 case B.

Proposal 1: Capture above observation 1-3 in TR for generalization over cell configuration on FR1 case B.

FR2 generalization performance

Following two sets of cell configurations were agreed for generalization evaluation on FR1,

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 2: generalization parameters for FR2

Based on the generalization evaluation methodology, we carried out simulation of following scenarios accordingly,

| Scenario | Training @Dataset: Configuration #1 | Training @Dataset: Configuration #2 | Inference @Dataset: Configuration #1 | Inference @ Dataset: Configuration #2 |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| CG1 |  | Yes | Yes |  |
| GC2 | Yes | Yes | Yes |  |

It’s up to company to select the prediction pattern, i.e. PW an OW. We select multiple patterns to check the generalization performance. The UE speed is set to 60km/h. Following table shows the generalization performance degradation.

| OW/PW (ms) | 160/80 | 240/80 | 160/160 |
| --- | --- | --- | --- |
| Baseline RSRP diff(dB) | 0.76 | 0.7 | 1.28 |
| GC#1 RSRP diff (dB) | 0.82 (0.06 degradation) | 0.94 (0.24 degradation) | 1.32 (0.04 degradation) |
| GC#2 RSRP diff (dB) | 0.77 (0.01 degradation) | 0.81 (0.11 degradation) | 1.30 (0.02 degradation) |

In general, we can observe the performance degradation of GC#2 is less than GC#1, regardless PW length. This is also aligned with the generalization evaluation on FR1.

Observation 4: GC#1 outperforms GC#2 over cell configuration on FR2 case A.

If OW is fixed to 160ms and PW is increased from 80ms to 160ms, the performance degradation compared with baseline doesn’t change much for GC#1 and GC#2.

Observation 5: Longer PW doesn’t affect performance degradation compared with baseline for GC#1 and GC#2 on FR2 case A.

If PW is fixed to 80ms and OW is increased from 160ms to 240ms, GC#1 degradation compared with baseline increases from 0.06db to 0.24db and GC#2 degradation compared with baseline increases from 0.01db to 0.11db. Longer OW would result in larger degradation from baseline for GC#1 and GC#2. We understand the reason is longer OW would mislead the AI model, since the radio channel condition changes more rapidly in cell configuration 1. Because the GC1 model is trained by cell configuration 2 data and tends to believe the channel changes in relatively slow way. During the test, if more measurement results from cell configuration 2 are considered as model input, the AI model trained with GC#1 may be confused and give incorrect output.

Observation 6: Longer OW results in larger performance degradation compared with baseline for GC#1 and GC#2 on FR2 case A.

Proposal 2: Capture above observation 4-6 in TR for generalization over cell configuration on FR2 case A.

Conclusions

According to the analysis given above, we have the following observations and proposals:

FR1

Observation 1: GC#1 outperforms GC#2 over cell configuration on FR1 case B.

Observation 2: If MRRT is small, there is small performance degradation compared with baseline for both GC#1 and GC#2 on FR1 case B.

Observation 3: Larger MRRT results in larger performance degradation compared with baseline for both GC#1 and GC#2 on FR1 case B.

Proposal 1: Capture above observation 1-3 in TR for generalization over cell configuration on FR1 case B.

FR2

Observation 4: GC#1 outperforms GC#2 over cell configuration on FR2 case A.

Observation 5: Longer PW doesn’t affect performance degradation compared with baseline for GC#1 and GC#2 on FR2 case A.

Observation 6: Longer OW results in larger performance degradation compared with baseline for GC#1 and GC#2 on FR2 case A.

Proposal 2: Capture above observation 4-6 in TR for generalization over cell configuration on FR2 case A.

Reference

[1] TR 36.839