3GPP TSG-RAN WG2 Meeting #129b	                                          R2-2501936

Wuhan, China, 7 – 11, Apr, 2025

Source: 	Xiaomi

Title:  	Discussion on simulation results of event prediction

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

RAN2 agreed following metrics to evaluate the prediction accuracy of measurement prediction,

- Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa - Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] - Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] - F1 score = 2*Precision*Recall/(Precision + Recall) - Precision = n3/(n1+n3) - Recall =n3/(n2+n3)

Event prediction can be done by direct or indirect way. In this contribution, we provide simulation results of indirect measurement event prediction. Some observations are given.

Discussion

Case A on FR2

Following simulation assumptions were agreed by RAN2 for event prediction evaluation in case A,

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 90 | Open for 60 and 120km/h |
| OW length (ms, note1) | N/A | Up to implementation |
| PW length (ms, note1) | 320 | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |

Although only 320ms is agreed for PW for simplicity, we believe it’s beneficial to evaluate the prediction accuracy using other PW values.

Following is the simulation results of prediction accuracy using different PW length,

| PW (ms) | 400 | 320 | 240 | 160 |
| --- | --- | --- | --- | --- |
| n1 | 470 | 443 | 227 | 116 |
| n2 | 3 | 4 | 5 | 11 |
| n3 | 1161 | 1160 | 1159 | 1153 |
| Precision | 0.7118 | 0.7236 | 0.8362 | 0.9086 |
| Recall | 0.9974 | 0.9966 | 0.9957 | 0.9905 |
| F1_score | 0.8308 | 0.8385 | 0.9090 | 0.9478 |

As we can observe the prediction accuracy increases if shorter PW is used. This is reasonable since the RSRP prediction would increase if shorter PW is used. If PW is already short, e.g. 240ms, even shorter PW can increase the prediction accuracy by 5%. The gain is relatively small.

Observation 1: Shorter PW results in higher prediction accuracy.

Observation 2: Further shorten the PW doesn’t bring large gain on prediction accuracy, if PW is already very short.

During the discussion, it’s also proposed to add one more metric for measurement event prediction for temporal domain case A i.e. the timing distance between 1st time instance of positive true prediction and corresponding ground truth label. We agree this is important to show the first true prediction distance. Following is the timing distance between 1st time instance of positive true prediction and corresponding ground truth label.

| Timing distance (ms) | 80 | 160 | 240 | 320 | 400ms | 480ms |
| --- | --- | --- | --- | --- | --- | --- |
| PW: 400ms | 19(~1%) | 30(~2%) | 61(~5%) | 627(~82%) | 106 (~10%) | 22 (~1%) |
| PW: 320ms | 15(~1%) | 32(~2%) | 67(~5%) | 774(~92%) | N/A | N/A |
| PW: 240ms | 28(~3%) | 57(~6%) | 764(~92%) | N/A | N/A | N/A |
| PW: 160ms | 53(~5%) | 804(~95%) | N/A | N/A | N/A | N/A |

It can be seen it’s difficult to predict the true event when timing distance is longer than TTT. Even if PW is longer than TTT, only 11% of true event prediction occurs at timing distance longer than TTT. 82% of true event prediction occurs at timing distance equal TTT and 8% occurs at timing distance shorter than TTT. We understand this is due to true measurement result within potential TTT is available for AI if PW is equal to or shorter than TTT. If event entering condition has not been met within OW, real event would definitely not happen. This information can help to improve prediction accuracy.

Observation 3: Taking real measurement results within TTT into OW, i.e. PW is equal to or less than TTT, can increase the prediction accuracy a lot.

Observation 4: If PW is longer than TTT, most true prediction occurs at timing distance equal to TTT.

If PW is equal to or shorter than TTT, more than 90% of true event prediction occurs at timing distance equal to PW. 10% of true event prediction occurs at shorter timing distance.

Observation 5: If PW is equal to or shorter than TTT, most true prediction occurs at timing distance equal to PW.

Proposal 1: Capture above observations in TR for event prediction on FR2.

Case B on FR1

Following simulation assumptions were agreed by RAN2 for event prediction evaluation in case B,

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30 | Open for 60 and 90km/h |
| OW length (ms,note1) | N/A | Up to implementation |
| PW length (ms,note1) | 200 (non-sliding) 40ms (sliding) | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |
| MRRT | 50% | Open for more values |

Sliding filtering is used in the simulation. Following is the simulation results of event prediction on FR1 in case B,

| OW/PW (ms) | 40/40 |
| --- | --- |
| n1 | 3 |
| n2 | 22 |
| n3 | 350 |
| Precision | 0.9915 |
| Recall | 0.9409 |
| F1_score | 0.9655 |

As can be seen, the prediction accuracy is very high. It’s aligned with the observation agreed by RAN2 in last meeting.

Observation 6: The prediction accuracy of event prediction is very high for FR1 case B.

Conclusions

According to the analysis given above, we have the following observations and proposals:

FR2 Case A

Observation 1: Shorter PW results in higher prediction accuracy.

Observation 2: Further shorten the PW doesn’t bring large gain on prediction accuracy, if PW is already very short.

Observation 3: Taking real measurement results within TTT into OW, i.e. PW is equal to or less than TTT, can increase the prediction accuracy a lot.

Observation 4: If PW is longer than TTT, most true prediction occurs at timing distance equal to TTT.

Observation 5: If PW is equal to or shorter than TTT, most true prediction occurs at timing distance equal to PW.

Proposal 1: Capture above observations in TR for event prediction on FR2 case A.

FR1 Case B

Observation 6: The prediction accuracy of event prediction is very high for FR1 case B.

Reference

[1] TR 36.839