3GPP TSG-RAN WG2 Meeting #129b	                                          R2-2501937

Wuhan, China, 7 – 11, Apr, 2025

Source: 	Xiaomi

Title:  	Discussion on LCM and spec impact for AI/ML mobility

Agenda Item:	8.3.5

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss required signalling and protocol to enable reporting of inference outcome for RRM measurement prediction and measurement event prediction for a UE sided model and the required signaling and protocol aspects to enable reporting of UE assistance for inference operation of a network sided model for RRM measurement prediction. Furthermore, following aspects are studied in LCM for AI air physical use cases in [1]. We also study the LCM of AI mobility and propose the potential solutions.

The following aspects, including the definition of components (if needed) and necessity, are studied in LCM: - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - Functionality/model identification - Model delivery/transfer - Model inference operation - Functionality/model selection, activation, deactivation, switching, and fallback operation. - Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it) - Functionality/model monitoring - Model update - UE capability

Discussion

Inference solution

First, we would like to discuss the general assumption of the AI mobility solution, i.e. how to use the inference results in mobility procedure. During the SI phase, RRM prediction and event prediction were discussed in RAN2. In the following section, we discuss the spec impact for each use case respectively.

RRM prediction

Based on target study goal and methodology, the RRM prediction can be categorized into 6 scenarios. Three scenarios were prioritized by RAN2 as following,

Table 5.2.1-1: Prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Since there's no conclusion on simulation evaluation of scenario 1/5/6, in this contribution, we only focus on LCM analysis of high priority use cases, i.e. scenario 2/3/4.

RRM prediction scenario 2

For scenario 2, the measurement reduction is achieved by skipped measurement on temporal domain. Although SID [2] indicated both UE and NW sided model are in the scope of RRM prediction, we understand only UE sided model could be used in RRM prediction scenario 2. This is because UE can use predicted measurement in skipped instances for measurement event evaluation, where significant measurement effort and power could be saved at UE-side. If the inference is done at NW, lots of signalling is required to report model input and NW has to send the inference output back to UE, which is highly inefficient.

Proposal 1: Only UE sided model is supported in RRM prediction scenario 2.

UE obtains the L3 cell level measurement results on the skipped instances by performing AI prediction. In legacy RRM procedure (e.g. measurement event evaluation, S criteria evaluation, measurement results reporting), UE uses measured L3 cell level measurement results in each configured time instance. With the help from RRM prediction, instead of using the actual measured results, UE can use predicted L3 cell level measurement results in those skipped instances directly in RRM procedure, e.g. use predicted L3 cell level measurement results to decide whether a measurement event is fulfilled or not, report the predicted measurement results to network as part of measurement reports, etc.

Proposal 2: For RRM prediction scenario 2, UE can use the predicted L3 cell level measurement results in the skipped time instance for RRM procedures, e.g. measurement event evaluation, S criteria evaluation, measurement report, etc.

RRM prediction scenario 3

Both UE sided model and NW sided model can be supported in RRM prediction scenario 3.

In UE sided model, UE would perform inference to predict L3 cell level measurement result on frequency domain. On one hand, UE can report the predicted results on the predicted frequency to NW. It’s up to NW to decide how to use the predicted results. The report shall indicate the predicted measurement results, cell ID and frequency.

Proposal 3: For UE sided model in RRM prediction scenario 3, UE can report the predicted inter-frequency L3 cell level measurement results. It’s up to NW to decide how to use the predicted results in RRM prediction.

On the other hand, same as the discussion in RRM prediction scenario 2, UE may use the predicted inter-frequency measurement results to evaluate inter-frequency measurement event, e.g. A5. In following figure, UE is able to evaluate the target cell on F2 fulfilling A5 event without actually measuring on F2. UE can trigger the measurement report based on predicted event. However, considering the performance of this scenario was not evaluated during SI phase, it is proposed RAN2 to discuss whether predicted results can be used in inter-frequency measurement event evaluation.

The detailed signaling to report inference results can be further discussed during normative phase.

Proposal 4: For UE sided model in RRM prediction scenario 3, RAN2 to discuss whether UE can use predicted measurement results on F2 (the frequency with RRM measurement prediction) without performing actual measurement for inter-frequency measurement event (i.e. A5) evaluation.

In NW sided model, the inference is done at NW. It’s up to NW how to perform inference and use the inference results, which has no impact on the spec. The model input is real measurement results, which has to be reported by UE. Current spec has already supported periodical and event triggered measurement report. Legacy measurement report can be reused to report model input of NW sided model. NW-side training data collection is discussed in Section 2.2.1.

Proposal 5: For NW sided model in RRM prediction scenario 3, legacy measurement report procedure can be reused for UE to report measurement result as model input of NW sided model inference.

RRM prediction scenario 4

Both UE sided model and NW sided model can be supported in RRM prediction scenario 4.

In UE sided model, UE would perform inference to predict future L3 cell level measurement result on temporal domain. We understand the inference result can be used in following ways,

- UE reports the predicted results to NW. It’s up to NW to decide how to use the predicted results. The report shall indicate the predicted measurement results, cell ID and timing.

- UE can evaluate the measurement event based on the predicted results. This is the indirect event prediction. We will discuss this in next clause.

Proposal 6: For UE sided model in RRM prediction scenario 4, UE can report the predicted temporal domain L3 cell level measurement results. It’s up to NW to decide how to use the predicted results in RRM prediction.

In NW sided model, the inference is done at NW. It’s up to NW how to perform inference and use the inference results, which has no impact on the spec. The model input is real historical measurement results, which has to be reported by UE. However, current spec has already supported periodical measurement report. If periodic can be configured to align with the model input pattern, legacy periodical measurement report can be reused to report model input of NW sided model.

Proposal 7: For NW sided model in RRM prediction scenario 4, legacy measurement report procedure can be reused for UE to report measurement result as model input of NW sided model inference.

Measurement event prediction

Only UE sided model is supported in measurement event prediction as indicated in SID [2]. In the study, two options were studied regarding how to decide the time point to transmit handover command,

If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event

Both solutions require UE to report the predicted measurement event when the event is predicted to be fulfilled. Upon receiving the report, it is up to NW to decide which option is used to transmit handover command. Source cell may prepare the handover in advance. The legacy handover preparation procedure can be reused as baseline. Since RAN2 only evaluated measurement event A3, we could event A3 as baseline and other measurement events can be further studied.

Proposal 8: UE reports the predicted measurement event (e.g. A3 as baseline) when it’s predicted to be fulfilled. It’s up to NW to decide how to transmit the handover command.

Event prediction can be achieved in indirect or direct approach. In indirect approach, UE first predicts future L3 cell level measurement results, then evaluates whether measurement report event at one future time instance is fulfilled without further involvement of an AI/ML model. During this procedure, UE can obtain the time instance of predicted measurement event. In direct approach, UE directly predict the measurement event occurrence based on actual measurement results. In this approach, UE can obtain the timing (window) of the predicted measurement event and probability.

In legacy measurement report, UE will report the measurement results, meas ID and triggered cell ID. If the event is predicted, UE shall at least report the predicted timing (window), meas ID and triggered cell ID of the predicted event in both indirect and direct prediction. Furthermore, UE can report the predicted measurement results in indirect prediction and the probability of direct prediction approach if any.

Proposal 9: Besides the predicted event, UE also reports predicted timing (window), the predicted measurement results in indirect prediction and/or the probability of direct prediction if any.

LCM

AI air has discussed the basic LCM procedure and reached some agreements on beam management. The general requirement of LCM is same to AI air and AI mobility. Therefore, the AI mobility shall align with the agreements from AI air as much as possible to avoid repeated discussion.

Proposal 10: The LCM agreements for beam management in AI/ML air interface are also applicable to AI mobility unless explicitly specified.

There are other aspects still under discussion in AI air, such as UE-side data collection, service continuity, fallback, performance monitoring and RRC state handling, etc. Since RAN2 targets to design a general AI/ML framework in AI/ML air,

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

We believe the potential solution can also be applicable to AI mobility as well. Therefore, we propose to wait for conclusion in AI air and further discuss whether anything new is needed for AI mobility on these aspects.

Proposal 11: Wait for conclusion in AI air on UE-side data collection, service continuity, fallback, performance monitoring and RRC state handling before discussing specification impact specific to AI mobility.

However, there are still AI mobility specific issues. We further discuss the gap and identify the potential solutions.

NW side data collection

In legacy measurement reporting, UE doesn’t log data and only reports the latest measurement results. NW sided temporal prediction model training may require consecutive measurement results, which requires UE report measurement results more frequently compared to legacy RRM procedure. Therefore, logging is introduced to avoid frequent UE report of the measurement results. In AI mobility, similar issue exists. Logging is also needed for NW sided data collection in AI mobility.

In AI air, the data logging can be triggered in following ways,

- Radio Event, RSRP of serving cell becomes higher or lower (A1/A2)

- Periodic

As discussed earlier, these triggers can be reused. In addition, new radio event can also be considered for RRM prediction. In RRM prediction 3 and 4, the target goal is to improve mobility performance. Handover is expected to be occur at cell edge rather than cell centre. Therefore, NW would like to collect more data for AI based mobility while UE is at cell edge. For RRM prediction scenario 3 and 4, the data logged in cell centre may not be that useful To support data logging at cell edge and reduce unnecessary data logging at cell centre, new logging radio event, e.g. A3, A5 alike event, can be considered.

Proposal 12: Logging radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

RAN2 needs to discuss what is the data content for NW side data collection. As discussed in Section 2.1, we only consider NW-side model for RRM prediction scenario 3 and 4.

Following 3 sub-use cases are considered for L3 cell level prediction,

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The model input/output can be,

- L1 beam level measurement result/ L1 beam level measurement result for sub-use case 1

- L3 cell level measurement results/ L3 cell level measurement results for sub-use case 2

- L1 beam level measurement result/ L3 cell level measurement results for sub-use case 3

The collected data should be the aligned with model input/output. NW can configure UE which data should be logged according to the model input/output.

In RRM prediction scenario 4, the timing of the measurement results is also essential for temporal prediction model training.

Proposal 13: RAN2 to discuss the data content of NW sided data collection.

UE side data collection

In AI air WI, it’s agreed UE can request the data collection configuration for UE sided data collection, which can also applicable in AI mobility. Unlike AI air, measurement result prediction on frequency domain is also supported in AI mobility. Therefore, UE may need to collect inter-frequency measurement results for model training. Measurement gap may be needed. However, NW is not aware of the required measurement gap. So, UE shall be allowed to request the measurement gap for UE sided data collection.

Proposal 14: UE is allowed to request the measurement gap configuration for UE sided data collection.

Functionality/config management for UE sided model

The AI air framework of functionality report can be reused. However, we need to discuss how to identify a ‘functionality/config’ in AI mobility. In AI air, the functionality of beam management can be identified by following options,

- A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

In short, option A comprise the detailed inference configuration, while option B comprise the general characteristic of inference configuration. Similar options can be supported to identify the functionality in AI mobility. RAN2 can further discuss the detailed configuration.

Based on the simulation, the detailed inference configuration may comprise the measurement configuration, e.g. measurement object and measurement report config. The general characteristic of inference configuration may comprise model input/output measurement results, MRRT, PW.

Proposal 15: RAN2 to discuss the inference configuration in AI mobility for UE sided model.

In AI air, AI inference for beam management activation and deactivation is controlled by explicit NW indication.

Handover is expected to occur at cell edge. If the target goal is mobility performance improvement, the inference shall be aligned with handover. UE may not need to perform inference in cell centre, since handover would not occur at cell centre. If only explicit control is supported, UE may need to report additional measurement result to assist NW activate/deactivate functionality, which results in additional signaling. Alternatively, the UE can activate the functionality based on event (A2, A3, A5 alike event), which is configured by NW.

Proposal 16: RAN2 to discuss the radio event-based inference activation/deactivation for UE sided model.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Inference solution

RRM scenario 2

Proposal 1: For RRM prediction scenario 2, only UE sided model is supported.

Proposal 2: For RRM prediction scenario 2, UE can use the predicted L3 cell level measurement results in the skipped time instance for RRM procedures, e.g. measurement event evaluation, S criteria evaluation, measurement report, etc.

RRM scenario 3

Proposal 3: For UE sided model in RRM prediction scenario 3, UE can report the predicted inter-frequency L3 cell level measurement results. It’s up to NW to decide how to use the predicted results in RRM prediction.

Proposal 4: For UE sided model in RRM prediction scenario 3, RAN2 to discuss whether UE can use predicted measurement results on F2 (the frequency with RRM measurement prediction) without performing actual measurement for inter-frequency measurement event (i.e. A5) evaluation.

Proposal 5: For NW sided model in RRM prediction scenario 3, legacy measurement report procedure can be reused for UE to report measurement result as model input of NW sided model inference.

RRM scenario 4

Proposal 6: For UE sided model in RRM prediction scenario 4, UE can report the predicted temporal domain L3 cell level measurement results. It’s up to NW to decide how to use the predicted results in RRM prediction.

Proposal 7: For NW sided model in RRM prediction scenario 4, legacy measurement report procedure can be reused for UE to report measurement result as model input of NW sided model inference.

Event

Proposal 8: For event prediction, UE reports the predicted measurement event (e.g. A3 as baseline) when it’s predicted to be fulfilled. It’s up to NW to decide how to transmit the handover command.

Proposal 9: For event prediction, Besides the predicted event, UE also reports predicted timing (window), the predicted measurement results in indirect prediction and/or the probability of direct prediction if any.

LCM

Proposal 10: The LCM agreements for beam management in AI/ML air interface are also applicable to AI mobility unless explicitly specified.

Proposal 11: Wait for conclusion in AI air on UE-side data collection, service continuity, fallback, performance monitoring and RRC state handling before discussing specification impact specific to AI mobility.

Proposal 12: Logging radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

Proposal 13: RAN2 to discuss the data content of NW sided data collection.

Proposal 14: UE is allowed to request the measurement gap configuration for UE sided data collection.

Proposal 15: RAN2 to discuss the inference configuration in AI mobility for UE sided model.

Proposal 16: RAN2 to discuss the radio event-based inference activation/deactivation for UE sided model.

Reference

[1] TR 38.843

[2] RP-242393 SID of AI/ML mobility