3GPP TSG RAN WG2 #129bis	R2-2501941

Wuhan, China, Apr 7th–11th, 2025

Agenda Item:	8.1.2.3

Source: 	Xiaomi

Title:  	Discussion on the LCM for AI positioning case 1

Document for:	Discussion and Decision

Agreements:

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

Agreements:

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

Introduction

In the RAN2#129 meetings, RAN2 made some agreements for the AI positioning use case 1 as below:

In this contribution, we first address the FFS on whether UE can autonomously switch between AIML and non-AIML methods, then further discuss activation/deactivation and AI/ML specific abort cause value for AI positioning use case 1.

# Discussion

## Autonomous Switching and Fallback configuration

During RAN2 #129 meeting, UE autonomous switching between AIML and non-AIML method and fallback configuration were discussed, but companies have different understanding on the terminologies and specification impact.

As captured in TS38.305 [1], UE can autonomously switch between one/more methods within the list of positioning methods.

The standard positioning methods supported for NG-RAN access are: - network-assisted GNSS methods; - observed time difference of arrival (OTDOA) positioning based on LTE signals; - enhanced cell ID methods based on LTE signals; - WLAN positioning; - Bluetooth positioning; - terrestrial beacon system (TBS) positioning; - sensor based methods: - barometric Pressure Sensor; - motion sensor. - NR enhanced cell ID methods (NR E-CID) based on NR signals; - Multi-Round Trip Time Positioning (Multi-RTT based on NR signals); - Downlink Angle-of-Departure (DL-AoD) based on NR signals; - Downlink Time Difference of Arrival (DL-TDOA) based on NR signals; - Uplink Time Difference of Arrival (UL-TDOA) based on NR signals; - Uplink Angle-of-Arrival (UL-AoA), including A-AoA and Z-AoA based on NR signals; - SL positioning and Ranging based on sidelink signals, incl.: - Sidelink Round Trip Time Positioning (SL-RTT); - Sidelink Angle-of-Arrival (SL-AoA); - Sidelink Time Difference of Arrival (SL-TDOA); - Sidelink Time of Arrival (SL-TOA). Hybrid positioning using multiple methods from the list of positioning methods above is also supported. Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

Observation 1: In non-AI/ML positioning, UE can autonomously use one or more methods from the list of positioning methods.

Recalling that AI/ML positioning case 1 is introduced as a new positioning method:

1: Introduce AI/ML positioning Case 1 as a new positioning method.

When multiple positioning methods are provided by LMF via RequestLocationInformation, including AIML based positioning Case 1, following legacy behaviour, UE can autonomously switch between AIML based positioning Case 1 and other non-AIML positioning method.

Proposal 1: If multiple positioning methods are supported in RequestLocationInformation:

- UE can autonomously switch between AI/ML and non-AI/ML methods;

- If AI/ML positioning Case 1 becomes non-applicable, UE can autonomously switch to other non-AI/ML positioning method (i.e. fallback).

## AI positioning method activation/deactivation

In legacy UE-based positioning, Location Management Function (LMF) has the full controllability of how positioning is performed at UE side, including positioning methods or instructing UE to stop positioning. For UE-based positioning, LMF can request UE to report location information by indicating the type of location information needed at network side via RequestLocationInformation. The positioning session can further be terminated by Abort procedure, where an on-going procedure (e.g. location reporting from the target device) can be stopped.

Same as legacy UE-based positioning method, LMF should also take the full responsibility of configuring, selecting or stopping positioning methods when using AI/ML.

It was agreed in RAN2 #129 meeting that RequestLocationInformation is used to requesting the results of the UE-side model inference operation.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

It is straightforward that UE may activates AI positioning case 1 right after receiving RequestLocationInformation if AI positioning case 1 is requested.

As for deactivation of AI/ML positioning case 1, depends on whether periodic reporting or triggered reporting is configured, deactivation method could be different. Assuming LMF requests a triggered reporting for AI/ML positioning case 1, UE only reports the inference results upon event is triggered, which is one-time shot. After reporting inference results in ProvideLocationInformation, AI/ML positioning case 1 is deactivated autonomously. On the other hand, if LMF request a periodic reporting for AI/ML positioning case 1, same as legacy UE-based positioning, abort procedure can be used to stop an ongoing procedure (e.g. periodic location reporting from the target device).

Proposal 2: For triggered reporting, AI/ML positioning case 1 is deactivated autonomously after providing inference results in ProvideLocationInformation. For periodical reporting, LMF/UE deactivates AI/ML positioning case 1 via Abort message.

# Conclusions

In this contribution, we discussed LCM for AI positioning case 1 and provide the following observations and proposals:

Proposal 1: If multiple positioning methods are supported in RequestLocationInformation:

- UE can autonomously switch between AI/ML and non-AI/ML methods;

- If AI/ML positioning Case 1 becomes non-applicable, UE can autonomously switch to other non-AI/ML positioning method (i.e. fallback).

Proposal 2: For triggered reporting, AI/ML positioning case 1 is deactivated autonomously after providing inference results in ProvideLocationInformation. For periodical reporting, LMF/UE deactivates AI/ML positioning case 1 via Abort message.

# References

- TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN