3GPP TSG-RAN WG2 Meeting #129bis											R2-2501947

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.7.4.2

Source: 	Sharp

Title:  	Discussion on DSR Enhancements

Document for: 	Discussion

# Introduction

In RAN2#129 meeting, RAN2 made the following agreements on DSR:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

This document discusses remaining open issues on DSR enhancements in XR.

# Discussion

## PDCP/RLC Data Volume Calculation for DSR MAC CE

In RAN2#129 meeting, RAN2 discussed PDCP/RLC data volume calculation for Rel-19 DSR MAC CE, but this issue was not concluded. The main issue was how to handle the Control PDU and PDUs/SDUs to be retransmitted. It is captured by the running CRs [1][2]:

## 5.15 Data volume calculation for delay status reporting … For the purpose of MAC delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - [if i = 1], the PDCP Control PDUs; - [if i = 1], for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - [if i = 1], for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. Editor’s Note: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

## 5.5 Data volume calculation … For the purpose of MAC delay status reporting, the UE shall consider the following as delay-reporting RLC data volume associated with the i:th dsr-ReportingThreshold: - delay-reporting RLC SDUs and delay-reporting RLC SDU segments that associated with the i:th dsr-ReportingThreshold and have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-reporting RLC SDU or a delay-reporting RLC SDU segment associated with the i:th dsr-ReportingThreshold; - [if i=1,] RLC data PDUs that are pending for retransmission (RLC AM). Editor’s Note: Same as PDCP open issue: it is FFS which delay-reporting RLC data volume shall consider RLC data PDUs to be retransmitted. … In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting, as part of delay-critical RLC data volume for MAC delay status reporting, and as part of [the first (i.e. i=1)] delay-reporting RLC data volume for MAC delay status reporting. Editor’s Note: It is FFS which delay-reporting RLC data volume shall consider STATUS PDU to be transmitted

For the issue how Control PDU and PDUs/SDUs to be retransmitted can be associated with a dsr-ReportingThreshold, we may have the following options:

- Option 1: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest configured dsr-ReportingThreshold (i.e., i=1).

- Option 2: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the LCG of the PDCP entity.

- Option 3: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

Considering that Control PDU and PDUs/SDUs to be retransmitted are prioritized over only the initial transmission from the same PDCP entity. The transmission of data from different PDCP entities relies on the LCH priority other than the PDU type. The delay requirement of Control PDU and PDUs/SDUs to be retransmitted should be the same as the delay requirement of the delay-reporting PDCP SDU from the same PDCP entity. It is inaccurate to always consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest confiugred dsr-ReportingThreshold (Option 1) or the smallest dsr-ReportingThreshold for the LCG (Option 2). In addition. Option 1 will result in signalling overhead if the LCG does not have delay-reporting PDCP SDUs associated with smallest configured dsr-ReportingThreshold. Thus, we see that Option 3 is straightforward and reasonable.

Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

When multiple PDCP entities belong to the same LCG, the buffered data in a PDCP entity are purely either Control PDU or PDUs/SDUs to be retransmitted. We can consider two cases:

- Case 1: Only Control PDU

- Case 2: Control PDU and PDUs/SDUs to be retransmitted or only PDUs/SDUs to be retransmitted.

Control PDU is prioritized over the delay-reporting PDCP SDU within the same PDCP entity. If there is no delay-reporting PDCP SDU in the PDCP entity (even if other PDCP entity belonging to the same LCG has delay-reporting PDCP SDU), Control PDU of the PDCP entity should not be considered as PDCP data volume for DSR.

If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

PDUs/SDUs to be retransmitted may need to be treated similarly to Control PDU. Thus, we propose:

If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

We see that the same principle can apply to RLC data volume calculation for DSR.

RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

## Configuration Order of dsr-ReportingThresholds

This subsection discusses the following open issue from the latest PDCP running CR discussion [3], as follows:

4. Whether to keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the PDCP specification.

The current PDCP running CR and RLC running CR assume that the threshold is configured in ascending order whereas MAC DSR reporting does not need the ascending order. Considering the current shape of PDCP and RLC running CRs, it seems clear that the ascending order should be kept. The issue is how and where we capture this. We see that this is more like an RRC configuration issue. The order of configuration parameters is a restriction in RRC. It would be better to capture it in RRC and PDCP does not have such a restriction or reordering.

RRC specifies entries of dsr-ReportingThresholds in the list are configured in ascending order.

## Delay-Reporting Indication from PDCP

This subsection discusses the following open issues from the latest PDCP running CR discussion [3], as follows:

5. Whether to specify delay-reporting indication in the PDCP specification or leave it up to UE implementation. 6. Whether to specify update of delay-reporting indication in the PDCP specification or leave it up to UE implementation.

The PDCP running CR introduced delay-reporting indication for Rel-19 DSR when PDCP SDU as follows:

The transmitting PDCP entity provides a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

Rel-18 text already has a similar indication, as follows:

The transmitting PDCP entity provides a delay-critical indication for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-critical PDCP SDU; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-critical PDCP SDU.

Thus, we see that the newly added texts are aligned with the legacy operation.

Specify delay-reporting indication in the PDCP specification when

- the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

- the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

The running CR introduced the following texts on update of delay-reporting indication when its associated threshold is changed, as follows:

A delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold as its remaining time decreases. The transmitting PDCP entity provides a delay-reporting indication for the PDCP Data PDU to lower layers each time the delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold.

We do not see that the PDCP specification needs to capture these texts. The update does not affect DSR triggering operation. It is possible that the PDCP entity does not provide the indication each time that its associated threshold is changed. It is an internal UE behaviour which can be implemented in different ways. For example, the MAC entity decides the contents of the DSR MAC CE when it is triggered and about to send.

PDCP specification does not specify update of delay-reporting indication in the PDCP specification. It can be left up to UE implementation.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

- If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

- If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

- RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

- RRC specifies entries of dsr-ReportingThresholds in the list are configured in ascending order.

- Specify delay-reporting indication in the PDCP specification when

- the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

- the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

- PDCP specification does not specify update of delay-reporting indication in the PDCP specification. It can be left up to UE implementation.

# Reference

[1] R2-2501950	PDCP running CR for R19 XR, LG Electronics Inc. (Rapporteur)

[2] R2-2502162	RLC running CR for XR, vivo

[3] R2-2501951	Discussion of [POST129][511][XR] PDCP running CR, LG Electronics Inc. (Rapporteur)