3GPP TSG-RAN WG2 Meeting #129bis										R2-2501957

Wuhan, China, 7th - 11th April 2025

Agenda item:		8.5.2

Title: 	Remaining issues of on demand SSB SCell operation

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining open issues of on demand SSB SCell per the latest progress in RAN1.

- Discussion

  - Configuration and activation of on-demand SSB

The following agreements have been achieved at RAN1#120 [1]:

-------------------------------------- RAN1#120 agreement--------------------------------------

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

-------------------------------------- RAN1#120 agreement-------------------------------------

The parameters agreed so far have been summarized below:

| Parameter in od-ssb-config | Content | Case#1 | Case#2 |
| --- | --- | --- | --- |
| od-ssb-Periodicity | Periodicity of the on-demand SSB | X | X |
| od-ssb-sfn-Offset | SFN offset from a reference point | X | X |
| od-ssb- halfFrameIndex | Half frame index | X | X |
| od-ssb-nrofBurst | Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | X | X |
| od-ssb-absoluteFrequency | Frequency of the on-demand SSB | X | FFS |
| od-ssb-PositionsInBurst | SSB positions within an on-demand SSB burst | X | FFS |
| od-ssbSubcarrierSpacing | Sub-carrier spacing of the on-demand SSB | X |  |
| od-ssb-physCellId | Physical Cell ID of the on-demand SSB | X |  |
| od-ss-PBCH-BlockPower | Downlink transmit power of on-demand SSB | X |  |

Also, RAN1 mentioned that higher layer parameter should be introduced for on demand SSB configuration.

Proposal 1: Based on RAN1 agreement, the above parameters should be configured by RRC signaling.

Last meeting, we started discussion on the configuration and activation without reaching agreement:

-------------------------------------- RAN2#129 discussion--------------------------------------

R2-2500256	Discussion on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: The OD-SSB MAC-CE has a fixed size and includes a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC CE.

- Revisit the issues that raised in P1-P3 next meeting.

-------------------------------------- RAN2#129 discussion--------------------------------------

With the on demand SSB parameters configured via RRC, we understand OD-SSB MAC CE with fixed size would be sufficient while one bit refers to one SCell:

Figure 1: An example of on demand SSB activation MAC CE of four octets

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

Proposal 2: The OD-SSB MAC-CE has a fixed size with each bit to indicate whether OD-SSB is activated in each SCell.

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

RAN1 has agreed to introduce the Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated, then we need to discuss the expected UE behavior after N OD-SSB bursts have been transmitted from NW and monitored by UE.

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

Proposal 3: After N (the number of on-demand SSB bursts to be transmitted after on-demand SSB is indicated) OD-SSB bursts have been transmitted from NW and monitored by UE:

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

  - Deactivation of on demand SSB

There has been discussion on how to indicate the transmission of on demand SSB while we have not mentioned whether we can deactivate the on demand SSB transmission.

Per our understanding, the deactivation via MAC CE should be also supported with the corresponding bit for SCell set to 0.

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

Proposal 4: On demand SSB deactivation via MAC CE should be supported:

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

  - L3 Measurement

For L3 measurement, we have agreed last meeting that RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2, then the remaining issues is how to configure and perform measurements after OD-SSB activation if OD-SSB are to be measured at least for case#1.

We had discussion on SSB adaption measurements last meeting with the following agreement reached [2]:

-------------------------------------- RAN2#129 discussion--------------------------------------

R2-2500199	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

-------------------------------------- RAN2#129 discussion--------------------------------------

We do not see the need to differentiate handling for L3 measurement for on demand SSB since it is also SSB adaption which happens in a on demand way.

Thus, we would like to follow the same solution as SSB adaption:

Proposal 5: RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to the OD-SSB for L3 RRM measurement on OD-SSB SCell.

  - On-demand SSB procedure

The on demand SSB procedure can be triggered when UL or DL traffic arrives and more activate SCells are needed. Thus we understand both UE and NW trigger can be considered.

Proposal 6: The on demand SSB procedure can be triggered by NW or UE.

From RAN2’s perspective, UE can send on demand SSB request to network via UE assistance information if we go for pure RAN2 solution without much impact in RAN1. Upon reception of the request, it is still up to network to decide whether to activate the on demand SSB transmission while the activation/deactivation MAC CE can also be applied.

Proposal 7: UE request the on demand SSB activation via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation MAC CE can also be applied.

  - UE capability for on demand SSB

We understand UE capability for on demand SSB should be defined as a per UE capability and provided to network via UECapabilityInformation message. NW would only provide OD-SSB configuration to UE supporting this feature and activate/deactivate OD-SSB.

Proposal 8: UE capability for on demand SSB should be defined as a per UE capability and provided to network via UECapabilityInformation message.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Based on RAN1 agreement, the following parameters should be configured by RRC signaling.

| Parameter in od-ssb-config | Content | Case#1 | Case#2 |
| --- | --- | --- | --- |
| od-ssb-Periodicity | Periodicity of the on-demand SSB | X | X |
| od-ssb-sfn-Offset | SFN offset from a reference point | X | X |
| od-ssb- halfFrameIndex | Half frame index | X | X |
| od-ssb-nrofBurst | Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | X | X |
| od-ssb-absoluteFrequency | Frequency of the on-demand SSB | X | FFS |
| od-ssb-PositionsInBurst | SSB positions within an on-demand SSB burst | X | FFS |
| od-ssbSubcarrierSpacing | Sub-carrier spacing of the on-demand SSB | X |  |
| od-ssb-physCellId | Physical Cell ID of the on-demand SSB | X |  |
| od-ss-PBCH-BlockPower | Downlink transmit power of on-demand SSB | X |  |

Proposal 2: The OD-SSB MAC-CE has a fixed size with each bit to indicate whether OD-SSB is activated in each SCell.

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

Proposal 3: After N (the number of on-demand SSB bursts to be transmitted after on-demand SSB is indicated) OD-SSB bursts have been transmitted from NW and monitored by UE:

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

Proposal 4: On demand SSB deactivation via MAC CE should be supported:

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

Proposal 5: RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to the OD-SSB for L3 RRM measurement on OD-SSB SCell.

Proposal 7: UE request the on demand SSB activation via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation MAC CE can also be applied.

Proposal 8: UE capability for on demand SSB should be defined as a per UE capability and provided to network via UECapabilityInformation message.

- Reference

- Draft_Minutes_report_RAN1#120_v030

- Draft_RAN2_129_Meeting_Report_v3