3GPP TSG-RAN WG2 Meeting #129											R2-2501959

Wuhan, China, 7th - 11th April 2025

Agenda item:		8.5.4

Title: 	Remaining issues of common signal/channel adaption

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining issues of common signal/channel adaption.

- Discussion

  - Paging occasion adaptation

Issue 1: New values for firstPDCCH-MonitoringOccasionOfPO

The following agreements have been made in the recent meeting on paging adaptation [1]:

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

If we introduce N with new value T/32 and Ns with new value 8, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

Proposal 1: The above new values for firstPDCCH-MonitoringOccasionOfPO should be introduced in 38.331.

Issue 2: PEI handling

The following agreements have been made in the recent meeting on PEI handling:

- Introduce a separate PEI configuration.

But we have not reached consensus on the usage of the separate PEI configuration.

Since there can be R19 UE supporting paging adaption with Ns=8 and N=T/32 and other UEs camping on the same cell so it is possible for NW to configure both of the existing pcch-Config we have since R15 and the newly introduced pagingAdaptation-NS-r19 + pagingAdaptationNAndPagingFrameOffset-r19 as we have to ensure that legacy UEs will not be impacted.

For this case, we can consider how to configure the PEI and the expected UE behaviors.

Here are the options:

- Option 1: One single PEI configuration to be followed by the R19 UE supporting paging adaption with Ns=8 and N=T/32 and other UEs but they will have different interpretation on the PEI and association with POs

- Option 2: Separate PEI configuration to be followed by the R19 UE supporting paging adaption with Ns=8 and N=T/32 and other UEs

  - R19 UE supporting paging adaption with Ns=8 and N=T/32 follow pagingAdaptationPEI-Config-r19

  - Other UE follow pei-Config-r17

From NW’s perspective, we understand both options are useful as for option 1 NW can send the same PEI to wake up both legacy UEs and R19 UEs while for option 2 more flexibility is offered.

So we have the following proposal on the handling of PEI:

Proposal 2a: For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

Proposal 2b: For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

  - PRACH adaptation in time domain

The following agreements have been reached in RAN1#120 for PRACH adaption [2]:

--------------------------------------- RAN1#120 Agreements---------------------------------------

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

--------------------------------------- RAN1#120 Agreements---------------------------------------

In summary, they have agreed to support DCI based adaptation for additional PRACH resources:

- For UE in connected, 1-bit field in DCI 1_0 with C-RNTI will be introduced to trigger PRACH adaption

- For UE in idle/inactive and connected, DCI 1_0 with P-RNTI will indicate the following

  - a reference point for the availability information for additional PRACH resource

  - semi-static signaling of a single PRACH mask to identify the subset of the additional PRACH resources

From RAN2’s perspective, we need to consider the configuration of the additional PRACH resources. Since the common adaption is applicable for UE in idle/inactive and connected, we understand it is more appropriate to configure the additional PRACH resources as cell specific parameters and broadcast via system information together with the DCI triggered adaption.

Proposal 3: The additional PRACH resources should be cell specific and broadcast via system information.

RAN1 has agreed that the Msg1-FDM and msg1-FrequencyStart can be configured separately for the additional PRACH resources.

Apart from that, the following parameters for 4-step RACH can also be configured separately per our understanding. If any of these parameters is absent, the corresponding field for legacy PRACH resources applies.

- prach-ConfigurationIndex

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- groupBconfigured

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

- ra-PrioritizationForAccessIdentity

- ra-PrioritizationForSlicing

- featureCombinationPreamblesList ((at least for RedCap, SDT, and Slicing, and any combinations, FFS on CE)

Proposal 4: The following parameters can be configured separately for the additional PRACH resources. If any of these parameters is absent, the corresponding field for legacy PRACH resources applies.

- Msg1-FDM (agreed by RAN1)

- msg1-FrequencyStart (agreed by RAN1)

- prach-ConfigurationIndex

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- groupBconfigured

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

- ra-PrioritizationForAccessIdentity

- ra-PrioritizationForSlicing

- featureCombinationPreamblesList (at least for RedCap, SDT, and Slicing, and any combinations)

  - SSB adaptation in time domain

--------------------------------------- RAN1#120 Agreements---------------------------------------

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signaling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

--------------------------------------- RAN1#120 Agreements---------------------------------------

The SSB adaption in time domain mainly refers to the SSB periodicity adaption, which can be triggered by MAC CE or DCI. And the following options can be considered for the MAC CE design:

- Option 1: Reuse the OD-SSB activation deactivation MAC CE to indicate SSB adaption

- Option 2: Introduce separate MAC CE for SSB adaption.

The MAC CE design is also related to the actual value of X, if X=1, one bit per cell would be sufficient. If X>1, the MAC CE also needs to indicate which one of the periodicity will be applied.

Proposal 5: RAN2 to down select from the following on the MAC CE design for SSB adaption:

- Option 1: Reuse the OD-SSB activation/deactivation MAC CE to indicate SSB adaption

- Option 2: Introduce separate MAC CE for SSB adaption.

  - UE capability for common signal/channel adaption

We have reached the following agreement last meeting on the UE capability for paging adaption:

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

For PRACH adaption, the additional PRACH resources can be broadcast via system information with the availability information and PRACH mask indicated in DCI 1_0 with P-RNTI. DCI 1_0 with C-RNTI is also introduced to trigger PRACH adaption per UE and would only be triggered towards the UE who support PRACH adaption.

With the above understanding, UE capability for PRACH adaption should be defined as a per UE capability and signed to network via UECapabilityInformation message.

Proposal 6: PRACH adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

For SSB adaption, it should also be defined as a per UE capability and provided to network via UECapabilityInformation message as NW would only provide SSB adaption periodicity to UE supporting this feature and activate/deactivate via MAC CE or DCI.

Proposal 7: SSB adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

As proposed in other contributions [3][4], we will have the following capabilities for NES features:

- UE capability for paging adaption in UE-RadioPagingInfo

- On demand SIB1 should be an optional feature without UE radio access capability parameters

- UE capability for on demand SSB should be defined as a per UE capability and provided to network via UECapabilityInformation message.

- PRACH adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

- SSB adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

As listed above, UE capability for paging and on demand SIB1 cannot be merged with others while UE capability for on demand SSB/PRACH adaption/SSB adaption should be provided to NW via UECapabilityInformation message. Then we can consider whether to have a common capability for on demand SSB, PRACH adaption and SSB adaption.

Proposal 8: RAN2 to discuss and decide whether to have a common capability for on demand SSB, PRACH adaption and SSB adaption, which would be a per UE capability provided via UECapabilityInformation message.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: The following new values for firstPDCCH-MonitoringOccasionOfPO should be introduced in 38.331.

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

Proposal 2a: For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

Proposal 2b: For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

Proposal 3: The additional PRACH resources should be cell specific and broadcast via system information.

Proposal 4: The following parameters can be configured separately for the additional PRACH resources. If any of these parameters is absent, the corresponding field for legacy PRACH resources applies.

- Msg1-FDM (agreed by RAN1)

- msg1-FrequencyStart (agreed by RAN1)

- prach-ConfigurationIndex

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- groupBconfigured

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

- ra-PrioritizationForAccessIdentity

- ra-PrioritizationForSlicing

- featureCombinationPreamblesList (at least for RedCap, SDT, and Slicing, and any combinations)

Proposal 5: RAN2 to down select from the following on the MAC CE design for SSB adaption:

- Option 1: Reuse the OD-SSB activation deactivation MAC CE to indicate SSB adaption

- Option 2: Introduce separate MAC CE for SSB adaption.

Proposal 6: PRACH adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

Proposal 7: SSB adaption should be defined as a per UE capability and provided to network via UECapabilityInformation message.

Proposal 8: RAN2 to discuss and decide whether to have a common capability for on demand SSB, PRACH adaption and SSB adaption, which would be a per UE capability provided via UECapabilityInformation message.

- Reference

- Draft_RAN2_129_Meeting_Report_v3

- Draft_Minutes_report_RAN1#120_v030

- R2-2501957_Remaining issues of on demand SSB Scell operation

- R2-2501958_Remaining issues of on demand SIB1