3GPP TSG-RAN WG2 Meeting #129 bis	R2-2501978

Wuhan, China , 7-11 April 2025

Agenda item:	8.1.4

Source:	T-Mobile USA

Title:	Performance metrics / KPI’s for UE sided data collection

WID/SID:	NR_AIML_air-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper proposes a set of performance metrics /KPI’s that form the basis of data collected by the UE for offline AI/ML model training. 3GPP is developing a framework for AI/ML data collection, the three use cases developed in R19 are just the beginning of a framework to support AI for RAN.   The goal is to create a robust data lake with many different elements that enable companies to come up with innovative AI/ML solutions to problems. Layer 1 and Layer 2 data is but a piece of the data lake, there are other attributes available to operators that can further enhance AI/ML models running on the RAN.

# 2	Discussion

One of the challenges that RAN2 has faced is the lack of clarity on the type and scope of the data to be collected. Companies have expressed concerns that the 3GPP process to add/change/ modify the performance metrics / KPI’s is to burdensome and therefor the focus should be on developing standardized containers. We think the middle ground is to develop a robust set of standardized performance metrics from the beginning which relegates information sent in the containers to proprietary information about a single vendors implementation. A Robust standardized performance metrics/KPI’s also reduces the need for future metrics which address the process issue raised by vendors. If we only have to modify the standard occasionally then the burden to add other metrics is small.

Observation 1 – The lack of UE sided data performance metrics/KPI’s has slowed UE sided data collection framework

Observation 2 - Vendors are concerned that 3GPP process is too cumbersome to add new KPI’s

Observation 3 - A Robust standardized set performance metrics/KPI’s also reduces the need for future metrics which addresses the process issue raised by vendors

It is also important to consider the practicality of collecting each of the performance metrics/KPI’s. To that end we surveyed drive test equipment vendors and came up with 192 different performance metrics that are currently being collected using commercially available UE’s and drive test equipment. The categories consist of general information (time, date and location), Cell measurements, Serving cell information, Physical Channel Information (for each serving cell). Link Adaptation (for each serving cell), CSI information, RACH Parameters, Mobility Parameters, MAC (for each serving cell), RLC/PDCD/SDAP, RRC information, Upper Layer information.

Proposal 1 – Adopt the list performance metrics/KPI’s found in appendix A as the minimum set of KPI’s for UE sided data collection.

If drive test equipment vendors can extract this information from commercially available UE’s with commercially available chipsets surely UE sided data collection can collect the same information for offline training of AI/ML models. The presumption is that UE sided data collection doesn’t have the same processing power as drive test equipment and that there will be limitations on the number of simultaneous KPI’s that can be collected during a period.

Operators also have a requirement for controlling data transfer and the data collection, it is expected that operators will have complete control over when, where and how the 192 performance metrics are collected.

From a signalling perspective it’s important that RAN2 develop a management framework that allows for the configuration of data collection and allows for the quick and easy addition of new metrics.

Proposal 2 – Develop signalling that allows operators to manage when, where and how any one of the 192 parameters is collected and transferred to the network.

# 3	Conclusion

We seek approval of proposal 1 and proposal 2.

Observation 1 – The lack of UE sided data performance metrics/KPI’s has slowed UE sided data collection framework

Observation 2 - Vendors are concerned that 3GPP process is too cumbersome to add new KPI’s

Observation 3 - A Robust standardized set performance metrics/KPI’s also reduces the need for future metrics which addresses the process issue raised by vendors

Proposal 1 – Adopt the list performance metrics/KPI’s found in appendix A as the minimum set of KPI’s for UE sided data collection.

Proposal 2 – Develop signalling that allows operators to manage when, where and how any one of the 192 parameters is collected and transferred to the network.

# Appendix A – List of Performance metrics / KPI’s

- Band

- Band type (FDD/TDD)

- Beam index

- Beam type

- Cell type

- CSI PDSCH PMI i11, i12, and i13

- CSI PDSCH PMI i2

- CSI Rank indicator (RI)

- CSI WB CQI

- Date

- DL NR-ARFCN Point A

- Frequency offset (Hz, for serving cell)

- Global raster bandwidth

- Global raster Point A

- L1-RSRP (type, per index)

- L3-RSRP

- L3-RSRQ

- L3-SINR

- Location

- NR cell ID

- NR-ARFCN (BWP)

- NR-ARFCN Freq (BWP)

- Offset to point A

- PCI

- PDSCH MCS

- PDSCH PRBs (count)

- PDSCH rank

- PDSCH throughput

- PUSCH MCS

- PUSCH power headroom

- PUSCH PRBs (count)

- PUSCH rank

- PUSCH throughput

- PUSCH, PUCCH, and SRS max TX power

- PUSCH, PUCCH, and SRS pathloss

- PUSCH, PUCCH, and SRS TPC adjustment

- PUSCH, PUCCH, and SRS TX power

- RACH access delay

- RACH logical root sequence index

- RACH Numerology

- RACH pathloss

- RACH preamble count

- RACH preamble format

- RACH preamble index

- RACH preamble initial power

- RACH preamble power

- RACH preamble responses with backoff time

- RACH preamble responses with PUSCH resource

- RACH PUSCH power

- RACH reason

- RACH response window

- RACH result

- RACH SSB ID

- RACH subchannel

- RACH Timing

- RACH timing advance

- RACH type - RRC state

- RSSI

- RX beam ID

- SSB GSCN

- SSB NR-ARFCN

- SSB NR-ARFCN frequency

- SSB periodicity

- SSB RSSI per antenna port

- SSB SINR per antenna port

- SS-RSRP

- SS-RSRQ

- SS-SINR

- Time

- Timing advance

- Timing offset (ns, for serving BI 0)

- UL NR-ARFCN Point A

Mobility parameters - To be added during Mobility Discussion

MAC (for each serving cell) - To be added during Mobility Discussion

Upper layers - To be added later