3GPP TSG-RAN WG2 Meeting #129bis	R2-2501982

Wuhan, China, 7th April 2025 – 11th April 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues of on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several agreements have been achieved [2]:

Agreements on OD-SIB1

1.	There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

2.	Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

3.	The UE follows the legacy validity principle of stored SIB.

4.	For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

5.	SIB-x can be cell specific or area specific, as legacy.

6.	Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will discuss the remaining issues of on-demand SIB1 operation.

# Discussion

## 2.1 Handling of si-BroadcastStatus

In the current specification, the SI can be broadcasted in an on-demand manner. When a UE wants to acquire the needed OSI, it should first check the si-BroadcastStatus IE in SIB1 to know whether it is broadcasted.

However, in the OD-SIB1 case, since the SIB1 is not always broadcasted, a UE may not know whether the OSI is broadcasted or not. In our understanding a UE in RRC_IDLE/INACTIVE can reuse the OD-SIB1 procedure, if it needs to obtain an up-to-date version of SIB1 to check si-BroadcastStatus.

Observation 1: UEs in RRC_IDLE/INACTIVE can reuse the OD-SIB1 procedure, if it needs to obtain an up-to-date version of SIB1 to check si-BroadcastStatus.

There are several options to solve this issue for RRC_CONNECTED UEs:

1)  A UE may trigger OD-SIB1 request reusing the IDLE/INACTIVE procedure first and after acquiring the up-to-date SIB1 check the si-BroadcastStatus IE. Then it can follow the legacy procedure to acquire the OSI.

2)  A UE may use the valid stored SIB1 to check the broadcast status of the concerned OSI. However, there will be a case that the si-BroadcastStatus is set to broadcast in the stored SIB1, but the status has changed since the SIB1 was acquired. Since the change of si-BroadcastStatus will not cause SI change notification, a UE cannot know whether the status change has happened.

3)  UE may assume the OSI is always provided in an on-demand manner. Hence, when UE wants to acquire the corresponding OSI, it will directly send the request to trigger the NW to broadcast the corresponding SI. However, such option will cause unnecessary signalling overhead if the particular OSI is being broadcasted at the moment of request.

In our view, the legacy procedure should be maintained, i.e. the UE needs to check the si-BroadcastStatus before requesting for OSI. Meaning that the UE needs to obtain the latest SIB1 to know whether OSI is broadcasted or not, which can be done by reusing the IDLE/INACTIVE procedure.

Proposal 1: UEs in RRC_CONNECTED can reuse the IDLE/INACTIVE procedure to obtain the latest SIB1 to know whether the required OSI is broadcasted or not before requesting OSI.

## 2.2 How to provide SIB1 if searchspaceSIB1 is configured and SIB1 is not broadcasting

During the RAN2#129 discussion, we have agreed that a UE can rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured for RRC_CONNECTED UEs. But companies have no consensus on how to provide SIB1 when searchSpaceSIB1 is configured and if the UE needs to obtain e.g. si-BroadcastStatus of OSI.

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline. [Vivo]: NW cannot know when the UE wants to request OSI, then proposal 5 cannot work in the case. [Ericsson]: Want to have more time to think of two options. We will make conclusion on P5 next meeting.

In the current specification, if searchSpaceSIB1 has been configured, a UE could acquire the periodically broadcasted SIB1 via such searchSpace configuration. However, since SIB1 is not periodically broadcasted in the OD-SIB1 case, the UE cannot acquire SIB1 using the searchSpaceSIB1 configuration even if it is configured.

One simplified way is to follow the agreement we have made when searchSpaceSIB1 is not configured, i.e. it is up to NW implementation to deliver SIB1. That solution doesn’t solve all cases in which the NW cannot know when the UE wants to acquire SIB1. Hence, to solve all the cases it should be possible for the UE to request for OD-SIB1 reusing the IDLE/INACTIVE procedure in the case that searchSpaceSIB1 is configured and SIB1 is not broadcasted in addition to relying on the NW implementation to deliver SIB1.

Proposal 2: For Rel-19 NES UEs in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, a UE can reuse the IDLE/INACTIVE procedure to obtain the latest SIB1, if needed.

## 2.3 Details of the UL WUS configuration

RAN1 has discussed UL WUS configuration contents during RAN1#118bis and RAN1#119 and reached the following agreements.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

-	rsrp-ThresholdSSB

-	prach-RootSequenceIndex

-	msg1-SubcarrierSpacing

-	restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

-	(working assumption) ULSubCarrierSpacing

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

-	FFS: FDD system

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | frequencyBandList |  |  |
|  | absoluteFrequencyPointA |  |  |
|  | offsetToCarrier |  |  |
|  | p-Max |  |  |
|  | ULSubCarrierSpacing |  |  |
|  | n-TimingAdvanceOffset |  |  |
|  | ssb-PeriodicityServingCell |  |  |
| Purpose | Parameters |  |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

RAN1 has agreed to include the above parameters in the WUS configuration. The CORESET#0 and the SS#0 can be included in the WUS configuration when SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Furthermore, whether the above parameters are mandatory or optional is also under RAN1’s discussion. RAN2 can consider this as the baseline and wait for more progress.

Due to the fact that the WUS config is provided by Cell A, if the Cell A bars a specific UE type (e.g., NTN or RedCap) from accessing or camping on it, that UE type would not be able to obtain the WUS config for neighbouring NES cells. In the current spec, if a UE bars the Cell, there is no need for it to acquire its OSI. Since these system information blocks are useless for a UE that is barred. But in the OD-SIB1 case, the WUS config in new SIBX provided by Cell A is useful for other NES Cells which might not be barred for this UE type. Therefore, we propose that NES UEs could acquire Cell A’s SIB1 to know whether Cell A is an anchor cell or has the broadcast information (i.e., the SchedulingInfo) of the new SIBX with WUS configs. If yes, the NES UE could acquire Cell A’s SIBX to be able to trigger OD-SIB1 in neighbouring NES Cells.

Proposal 3: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Another issue that has not been discussed before is whether to transmit SIB1 barring bits to the UE before it requests OD-SIB1. In previous releases, several new barring bits were added in SIB1 for access control of new UE types, such as RedCap, NTN, R18 NES, etc. It would be beneficial to include those bits in the WUS config so that relevant UEs can know in advance whether they are allowed on an OD-SIB1 cell or not. If these bits are not provided beforehand, the UE would only find out if it is barred after acquiring OD-SIB1, which would waste UE energy and cause a delay in network access because the UE would need to go back to Cell A or reselect to another cell if it is barred.

Proposal 4: Feature specific barring bits from SIB1 are included in the WUS config.

## 2.4 WUS configuration validity and area specific parameters

In the last meeting, we have achieved a general agreement that the SIBX can be cell specific or area specific, as legacy.

In the current specification, the area specific system information is already supported with the field systemInformationAreaID. The system information area ID is located at the top of all system information, indicating that all system information belongs to a single area. Whether a specific system information block belongs to the area is marked by the araeScope field.

Observation 2: The systemInformationAreaID is common for all SI in a given cell.

Cited from 38.331 v18.4.0:

SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SIB-TypeInfo ::= SEQUENCE { type ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610, sibType14-v1610, spare3, spare2, spare1,... }, valueTag INTEGER (0..31) OPTIONAL, -- Cond SIB-TYPE areaScope ENUMERATED {true} OPTIONAL -- Need S }

In the OD-SIB1 WUS config case, there may be multiple WUS configurations in the new SIBX. The legacy mechanism of areaScope and valueTag fields can only represent the whole content of the new SIB, not the specific WUS config. In order to support per WUS config validity, a WUS config specific valueTag and areaScope would be needed.

Observation 3: The SI valueTag and areaScope are defined per SI type.

RAN2 has decided to leave SIBX area specific as legacy, i.e.:

- Support valueTag and areaScope for SIBX on SI level (as legacy). Since the SIBX will most probably be different in different cells (contain different WUS configs), this option has limited applicability.

The existing RRC spec on UE checking SI validity area is highlighted below:

Cited from TS 38.331 The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the UE is NPN capable and the cell is an NPN-only cell: 4> if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 5> consider the stored SIB as valid for the cell; 3> else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

In the existing specification, the areaScope and systemInformationAreaID is used to determine if the stored SIB is valid or not. We think common WUS configuration and area based SIBX are different issues. Although multiple cells may share the same UL WUS config, the UE may not know whether the stored common SIBX is valid or not. UEs still need to acquire the SIBX in the following cases, e.g., after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

Because of the limited applicability of the legacy solution, the legacy SI structure is not fully applicable to UL WUS case. Since these parameters can be generated by the NES Cell, a Cell A can broadcast the WUS config identification and WUS specific valueTag and areaScope in its SIB1. This would allow the UE to check the specific WUS validity information and determine whether the stored WUS config is valid. If the valueTag and areaID is the same, UE could use the stored WUS configuration to trigger the corresponding NES Cell to broadcast OD-SIB1, when needed. Thus, the unnecessary SIBx acquiring procedure could be avoided to save both the NES Cell’s and UE’s energy.

Proposal 5: RAN2 to discuss whether valueTag and areaScope are defined per WUS configuration.

In order to reduce the overhead of multiple NES cells’ UL WUS configurations several companies had proposals to allow multiple NES cells to have a common UL WUS configuration. Our understanding is that it would mean a PCI list inside the WUS config, which would list cells with the same WUS config in the area, possibly reducing the size of SIBX. It is worth noting that this solution brings more complexity to cell A’s, because of the need of comparing and combing the received neighbour NES cell’s WUS configurations. RAN2 should consider the whether the signalling reduction benefit is worth the added implementation complexity.

Proposal 6: RAN2 to discuss whether a PCI list is added to the WUS config, if it is shared between NES cells in the same area.

## 2.5 Discussion on the parameter inconsistency between OD-SIB1 and UL-WUS

In the RAN1#120 meeting, the parameter inconsistency issue has been discussed with no consensus and the topic was left for RAN2 to decide.

As observed in RAN1’s agreements about the UL WUS configuration, several parameters are shared with SIB1. RAN1 did not decide whether OD-SIB1 IEs can be different from the value of the corresponding IEs in the WUS configuration, and how to define the UE behaviour if the values are different.

### Copied from R1-2501380 ### FL Proposal 9-2-1 Huawei’s version: - If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. ETRI’s version: - For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. Rapporteur suggestion: - It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

From RAN2 perspective, the common parameters in WUS config and SIB1 should be aligned, since the WUS config is also generated by NES Cell itself. However, there could be a case where the obtained WUS config parameters from Cell A and the OD-SIB1 parameters acquired from the NES Cell are different. This may occur since there might be a delay between updating SIB1 in the NES cell and sending an updated WUS config to neighbouring Cell A’s. In our understanding SIB1 will always be updated first, so the UE should use the parameters from OD-SIB1 if there is a mismatch. That means that the value in OD-SIB1 should override the value of the corresponding IE in the WUS configuration.

Proposal 7: If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. It is up to the gNB to decide that the corresponding IE is not configured in OD-SIB1, i.e. the parameters in OD-SIB1 are optional.

# Conclusion

In this contribution, we discussed the remaining issues of on on-demand SIB1 operation, and made the following observations and proposals:

Observation 1: UEs in RRC_IDLE/INACTIVE can reuse the OD-SIB1 procedure, if it needs to obtain an up-to-date version of SIB1 to check si-BroadcastStatus.

Observation 2: The systemInformationAreaID is common for all SI in a given cell.

Observation 3: The SI valueTag and areaScope are defined per SI type.

Proposal 1: UEs in RRC_CONNECTED can reuse the IDLE/INACTIVE procedure to obtain the latest SIB1 to know whether the required OSI is broadcasted or not before requesting OSI.

Proposal 2: For Rel-19 NES UEs in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, a UE can reuse the IDLE/INACTIVE procedure to obtain the latest SIB1, if needed.

Proposal 3: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Proposal 4: Feature specific barring bits from SIB1 are included in the WUS config.

Proposal 5: RAN2 to discuss whether valueTag and areaScope are defined per WUS configuration.

Proposal 6: RAN2 to discuss whether a PCI list is added to the WUS config, if it is shared between NES cells in the same area.

Proposal 7: If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. It is up to the gNB to decide that the corresponding IE is not configured in OD-SIB1, i.e. the parameters in OD-SIB1 are optional.

# References

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM.docx

- Chair notes RAN1#120 eom0.doc