3GPP TSG-RAN2#129bis 	R2-2501993

Wuhan, China, 7th Apr – 11th Apr, 2025

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed issues related to asymmetric DL sTRP/UL mTRP and made some agreements.

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we further discuss RAN2 impacts of asymmetric DL sTRP/UL mTRP.

2.	Discussion

In the current MAC specification, PHR is triggered if the pathloss has changed more than a threshold, as follows.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In the last meeting, RAN2 agreed that PHR is triggered based on PL change (pathloss change) taking into account PL offset and whether/how to capture PHR triggering is remained as FFS. This is because RAN2 thought PL offset have impacts on the pathloss change.

However, we have different understanding on this.

The text above is inherited from LTE, and in LTE, the path loss change is compared with dl-PathlossChange, i.e., the path loss means DL pathloss.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - prohibitPHR-Timer expires or has expired and the path loss has changed more than dl-PathlossChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In addition, in early version of NR MAC specification, it is clearly captured that the path loss stands for DL pathloss.

[38.321 v110] Editor's note: The name of phr-Tx-PowerFactorChange can be updated as it applies not only to DL pathloss but also for P-MPR backoff. All the names of the parameters should be aligned with RRC. Editor's note: phr-ModeOtherCG comes from LTE, but RAN2 (and also other WGs) should discuss whether it would be needed for EN-DC. If not needed, the procedural text below should also be updated. A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

Observation 1. In legacy, PHR is triggered based on DL pathloss change.

Meanwhile, RAN1 made some agreements for uplink power control formulation impacted by PL offset as follows.

[RAN1#116] Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. [RAN1#116bis] Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

According to above agreements, "legacy PL" is replaced with UL PL (for separate) or PL (for joint), and here ""legacy PL" means UL pathloss. Thus, DL pathloss is not changed by PL offset and there is no impact on PHR triggering by PL offset.

Observation 2. DL pathloss is not changed by PL offset

Observation 3. There is no impact on PHR triggering by PL offset.

However, the agreement of the last meeting seems to introduce UL pathloss change based PHR triggering condition, taking into account PL offset.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

UL pathloss change based PHR triggering is a new UE behaviour, and RAN2 has not discussed UL pathloss change based PHR triggering. If UL pathloss change based PHR triggering is introduced, RAN2 needs to discuss further, e.g. new threshold for UL pathloss change.

Observation 4. The agreement of the last meeting seems to introduce UL pathloss change based PHR triggering condition, taking into account PL offset.

If the intention of the agreement of the last meeting is not to introduce UL pathloss change based PHR triggering, the agreement is useless because DL pathloss is not impacted by PL offset.

Observation 5. If the intention of the agreement of the last meeting is not to introduce UL pathloss change based PHR triggering, the agreement of the last meeting is useless

Therefore, based on Observation 1-5, we propose to revert the agreement of the last meeting.

Proposal 1. RAN2 consider to revert the following agreement in the last meeting.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

Meanwhile, in RAN1#116bis and RAN1#118, RAN1 agreed to support applying PL offset on PDCCH-order based PRACH as follows.

[RAN1#116bis] Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification. - Above is subject to a separate UE capability signaling. [RAN1#118] Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

With PL offset on PDCCH-order PRACH, one of joint/UL TCI state is indicated by PDCCH-order and a PL offset associated with the indicated joint/UL TCI state is applied on PDCCH order PRACH transmission, then Tx power is calculated based on the PL offset.

In the last meeting, some companies suggested to consider PL offset for PRACH resource selection because companies assume that PDCCH-order PRACH indicating PL offset can be used for CBRA as well as CFRA.

However, in last RAN1 meeting, RAN1 concluded that in asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA.

In addition, as RAN1 already agreed, PL offset on PDCCH-order PRACH is used for PDCCH-order towards a UL TRP, but the UE does not know RACH preamble is transmitted to UL/DL TRP or UL-only TRP because there is no association between RA configuration and TRP. Thus, technically, PL offset on PDCCH-order PRACH cannot be used for PDCCH-order CBRA.

Therefore, we propose to confirm PDDCH-order PRACH can be used for CFRA only.

Proposal 2. RAN2 confirms that PL offset for PDCCH-order PRACH is used for CFRA only.

The other remaining issue is related to 2-TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

According to [2], one of objectives is to apply 2-TAs for asymmetric DL sTRP/UL mTRP scenario reusing Rel-18 specification. In Rel-18, 2-TAs are introduced only for mDCI case, but for asymmetric DL sTRP/UL mTRP, only sDCI is possible, so 2-TAs method needs to be extended considering sDCI.

In order to apply 2-TAs for asymmetric DL sTRP/UL mTRP, RAN1 made agreements and sent LS [3] as follows.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

According to RAN1 agreement, almost things are inherited from Rel-18 2-TAs principle. However, for sDCI, coresetPoolIndex is not configured and SRS resource set is used to distinguish TAG, so RAN1 agreed to remove the restriction of of coresetPoolIndex.

In the current RRC specification, 2-TAs can be configured only when two coresetPoolIndex is configured, so the field description needs to be fixed for asymmetric DL sTRP/UL mTRP.

tag2 This field to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

Considering the restriction is still needed for mDCI case, RAN2 needs to discuss how to remove the restriction of coresetPoolIndex in the field description of tag2.

Proposal 3. RAN2 discusses how to remove the restriction of coresetPoolIndex in the field description of tag2.

3.	Conclusion

In this document, we discuss RAN2 impacts from asymmetric DL sTRP/UL mTRP, and made observations and proposals as follows.

Observation 1. In legacy, PHR is triggered based on DL pathloss change.

Observation 2. DL pathloss is not changed by PL offset

Observation 3. There is no impact on PHR triggering by PL offset.

Observation 4. The agreement of the last meeting seems to introduce UL pathloss change based PHR triggering condition, taking into account PL offset.

Observation 5. If the intention of the agreement of the last meeting is not to introduce UL pathloss change based PHR triggering, the agreement of the last meeting is useless

Proposal 1. RAN2 consider to revert the following agreement in the last meeting.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

Proposal 2. RAN2 confirms that PL offset for PDCCH-order PRACH is used for CFRA only.

Proposal 3. RAN2 discusses how to remove the restriction of coresetPoolIndex in the field description of tag2.

4.	Reference

- 3GPP TS 38.213	Physical layer procedures for control	V18.6.0