3GPP TSG RAN2 Meeting #129bis                                                              R2-2501999

7th Apr. – 11th Apr. 2025, Wuhan, China

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

During the previous meeting #127bis ~ #129, with respect to Procedures for LP-WUS in RRC Connected Mode agenda, RAN2 agreed as follows:

#127bis For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. #128 The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE. #129 For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

- Option 1-1 and 1-2

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'.

For a quick summary, we capture the each options examples from RAN1’s discussion [6]:

Option 1-1: Replacing DCP following legacy configurations

Figure 1: LP-WUS monitor procedure in Option 1-1

Option 1-2: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered

Figure 2: LP-WUS monitor procedure in Option 1-2 with ‘duty-cycled’ LP-WUS

Figure 3: LP-WUS monitor procedure in Option 1-2 with ‘continuous’ LP-WUS

Based on the discussions from RAN2#127bis, RAN2 progressed and established the precise PDCCH monitoring timing as follows:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

The remaining question is whether this timer is newly configured or utilizes an existing traditional timer such as drx-onDurationTimer. It is noteworthy that if the abovementioned timer for Option 1-2 corresponds to drx-onDurationTimer and the LP-WUS periodicity is comparable to the DRX cycle, then Option 1-2 might be equivalent to Option 1-1.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

In the meantime, when the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

- Activation/ Deactivation

Previously, RAN1 had discussed and agreed that the PDCCH monitoring triggered by LP-WUS can be enabled/disabled by RRC signaling when the UE is in RRC Connected mode.

- RAN1#116 Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

With the above RAN1 agreements, RAN2 can discuss whether the LP-WUS monitoring based PDCCH monitoring when the UE is in RRC Connected mode can be enabled/disabled using RRC signalling.

For the sake of UE power saving and operational efficiency, we

Proposal 2. RAN2 confirms that the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using RRC signalling.

And for further study, RAN2 may discuss whether the MAC signalling could be introduced or not.

Proposal 3. RAN2 needs to discuss whether the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using MAC signalling.

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 2. RAN2 confirms that the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using RRC signalling.

Proposal 3. RAN2 needs to discuss whether the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using MAC signalling.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC