3GPP TSG-RAN WG2 #129bis                                             	    R2-2502017

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.6.2

Source: 		Qualcomm Incorporated

Title: 	Remaining Issues for Inter-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129 has agreed on the following for inter-CU LTM [1]:

Agreements on RRC issues: - For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID. - Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM. - The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field. - UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG. - FFS whether only one indication is needed to indicate LTM configuration as complete configuration. - The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue. - The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles. - Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18. Agreements on inter-CU LTM: - Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s). - Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES. - Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM. - For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2. - For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2. - How to support shared RACH resources for inter-CU LTM is left to RAN3.

This paper discusses further aspects related to inter-CU LTM.

# 2	Security update for non-DC

RAN2 received a Reply LS [2] from SA3 on the response RAN2 provided on the security handling for subsequent inter-CU LTM.

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

SA3 provided a compromised solution of delivering NCC to UE via MAC CE, but the potential risk of using the unprotected MAC-layer signaling still remains, and it’s also confirmed that any protection mechanism of MAC CE messages won’t be considered by SA3 in Rel-19.

Observation 1: SA3 provided a compromised solution of delivering the NCC via MAC CE, but there are known security threats and concerns such as increased attacks and possible handover failure, due to the unprotected MAC CE message nature in Rel-19.

Even though RRC reconfiguration during every inter-CU LTM switch could incur signaling overhead and latency for subsequent inter-CU LTM switches and it goes against the WID objective of aiming to avoid RRC configuration between cell switches (i.e., inter-CU LTM switches), it seems more sensible to prioritize the network security aspect from RRC signaling over the reduced signaling & latency overhead from MAC CE.

Observation 2: Between RRC and MAC CE signaling, it seems more sensible to prioritize the security aspect from RRC signaling over MAC CE, even though RRC could incur higher signaling overhead and latency.

Proposal 1: RAN2 agree to use only RRC signaling for the delivery of NCC for inter-CU LTM (i.e., no MAC CE-based NCC delivery)

# 3	LTM recovery

RAN2#127b agreed that Rel-18 LTM failure recovery, i.e., UE selects an intra-CU LTM candidate cell after intra-CU LTM cell switch failure, is reused in Rel-19, and the other cases were left as FFS.

The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Rel-19 LTM failure scenarios can be categorized into the following four cases:

- Case 1: Upon intra-CU LTM failure, UE selects an intra-CU LTM candidate cell

  - Rel-18 LTM failure recovery scenario.

- Case 2: Upon intra-CU LTM failure, UE selects an inter-CU LTM candidate cell

- Case 3: Upon inter-CU LTM failure, UE selects an intra-CU LTM candidate cell

  - The selected intra-CU LTM candidate cell is under the same CU as the serving cell who triggered the inter-CU LTM cell switch.

  - This is effectively same as Rel-18 LTM failure recovery scenario because the serving cell and the selected candidate cell are under the same CU.

- Case 4: Upon inter-CU LTM failure, UE selects an inter-CU LTM candidate cell

  - The selected inter-CU LTM candidate cell is under the different CU as the serving cell who triggered the inter-CU LTM cell switch, and it can be under the same or different CU as the target inter-CU LTM candidate that the LTM cell switch command indicated.

When the serving cell’s CU and the selected candidate cell’s CU are compared, Case 3 can be considered as LTM recovery to intra-CU LTM candidate cell like Case 1 because the selected candidate cell is under the same CU as the serving cell. Therefore, for Case 3, the Rel-18 LTM recovery procedure can be mostly reused without any security handling issue.

Observation 3: When the selected cell is under the same CU as the serving cell, it can be considered as LTM recovery to intra-CU LTM, irrespective of the target candidate cell in LTM CSC command being intra-CU or inter-CU candidate.

Similarly, Case 2 and Case 4 can be considered as LTM recovery to inter-CU LTM candidate cell because the selected candidate cell is under the different CU as the serving cell, which requires the security handling due to the CU switch. In order for UE to perform the LTM recovery towards an inter-CU candidate cell, the UE shall have the valid security configuration. The valid security configuration can be maintained at UE in two cases:

- Case 1: When the LTM configuration is provided to UE for the first time (i.e., initial LTM configuration) and before a successful inter-CU LTM execution

  - Serving CU can get the security configuration from candidate CUs, and this security configuration is valid till the next successful inter-CU LTM switch (i.e., any intra-CU LTM switches is not counted).

- Case 2: When the new security configuration is provided to UE after a successful inter-CU LTM execution (i.e., subsequent inter-CU LTM execution followed by the security configuration update)

  - Upon a successful inter-CU LTM execution, new serving CU is responsible for exchanging the new security configuration with candidate CUs, and providing it to UE.

Observation 4: LTM recovery to inter-CU LTM candidate can be performed when the UE has received the valid (or up-to-date) security configuration for upcoming inter-CU LTM cell switch.

Proposal 2: RAN2 agree to support the LTM failure recovery (refer to Figure 1) to inter-CU LTM candidate cell for the following cases:

- UE has received the initial LTM configuration and before successful inter-CU LTM switch OR

- UE has received the RRC reconfig with security parameter update after successful inter-CU LTM switch

Figure 1. Flow chart for LTM failure recovery procedure towards both intra-CU and inter-CU LTM candidate

# 4	Coexistence with MIMO 2TA/mTRP

RAN2#129 agreed to support the coexistence for Rel-19 inter-CU LTM to be same as Rel-18 intra-CU LTM, i.e., coexisting with CovE, (e)RedCap, but not with NRU, and MIMO 2TA was left as FFS.

Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

From RAN2 point of view, the coexistence support of MIMO 2TA in Rel-18 intra-CU LTM (i.e., TAG ID info in CandidateTCI-UL-State and CandidateTCI-State IEs) can be applied to Rel-19 inter-CU LTM without further specification impact. Therefore, it would be sensible to support the coexistence of MIMO 2TA/mTRP in Rel-19 inter-CU LTM, same as Rel-18 intra-CU LTM.

Observation 5: From RAN2 point of view, there’s no difference in supporting MIMO 2TA/mTRP between Rel-18 intra-CU LTM and Rel-19 inter-CU LTM.

Proposal 3: RAN2 agree to reuse the Rel-18 intra-CU LTM coexistence of MIMO 2TA/mTRP for Rel-19 inter-CU LTM.

# 3	Conclusion

This contribution discussed aspects related to inter-CU LTM. The following observations and proposals have been made:

Observation 1: SA3 provided a compromised solution of delivering the NCC via MAC CE, but there are known security threats and concerns such as increased attacks and possible handover failure, due to the unprotected MAC CE message nature in Rel-19.

Observation 2: Between RRC and MAC CE signaling, it seems more sensible to prioritize the security aspect from RRC signaling over MAC CE, even though RRC could incur higher signaling overhead and latency.

Proposal 1: RAN2 agree to use only RRC signaling for the delivery of NCC for inter-CU LTM (i.e., no MAC CE-based NCC delivery)

Observation 3: When the selected cell is under the same CU as the serving cell, it can be considered as LTM recovery to intra-CU LTM, irrespective of the target candidate cell in LTM CSC command being intra-CU or inter-CU candidate.

Observation 4: LTM recovery to inter-CU LTM candidate can be performed when the UE has received the valid (or up-to-date) security configuration for upcoming inter-CU LTM cell switch.

Proposal 2: RAN2 agree to support the LTM failure recovery (refer to Figure 1) to inter-CU LTM candidate cell for the following cases:

- UE has received the initial LTM configuration and before successful inter-CU LTM switch OR

- UE has received the RRC reconfig with security parameter update after successful inter-CU LTM switch

Observation 5: From RAN2 point of view, there’s no difference in supporting MIMO 2TA/mTRP between Rel-18 intra-CU LTM and Rel-19 inter-CU LTM.

Proposal 3: RAN2 agree to reuse the Rel-18 intra-CU LTM coexistence of MIMO 2TA/mTRP for Rel-19 inter-CU LTM.

# References

- Chairman Notes; TSG RAN WG2 Meeting #129

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, CATT