3GPP TSG-RAN WG2 #129bis                                             	    R2-2502019

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	L1 event-triggered measurement reporting for LTM

Document for:	Discussion

# 1	Introduction

RAN2#129 has agreed on the following for L1 event-triggered measurements for LTM [1]:

Agreements on MAC issues: 8. RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation. 9. The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'. 10. The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR. 11. The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam. 12. There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell. 13. Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader. Agreements on L1 event-driven MR: - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE.

This paper discusses further aspects related to L1 event-triggered measurement report enhancements for LTM.

# 2	LTM event-triggered L1 measurement reporting configuration

## 2.1	SP-CSI-RS resource support

In RAN1#120, support of semi-persistent CSI-RS for gNB scheduled LTM CSI reports (e.g., periodic, semi-persistent, aperiodic report) was confirmed; the corresponding LS [2] sent to RAN2 has requested signalling for activation/deactivation of the semi-persistent CSI-RS resource. However, for LTM event-triggered L1 measurement reporting, there was no consensus during the RAN1 discussion. Therefore, it would be desirable to postpone the RAN2 discussion on supporting semi-persistent CSI-RS for LTM event-triggered L1 measurement report until further progress or conclusion is reached in RAN1.

Observation 1: There was no consensus in RAN1#120 about supporting semi-persistent CSI-RS for LTM event-triggered L1 measurement reporting.

Proposal 1: RAN2 discussion on supporting semi-persistent CSI-RS for LTM event-triggered L1 measurement reporting is postponed until further RAN1 progress.

## 2.2	Current beam RS type mapping for Event LTM2

In RRC running CR for LTM event-triggered L1 measurement report, the following parameter (eventEvaluationRS-Type-r19) has been added for Event LTM2 to capture the RAN2#128 agreement “Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.”

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

<Text omitted>

eventTriggered-r19                        SEQUENCE {

eventId-r19                                     CHOICE {

eventLTM2-r19                                     SEQUENCE {

ltm2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

eventEvaluationRS-Type-r19                        ENUMERATED {csi-rs, ssb}                    OPTIONAL, -- Need R

...

},

<Text omitted>

}

However, this explicit RRC parameter of RS type for Event LTM2 conflicts with previous agreements from both RAN1 and RAN2.

- Agreement from RAN2#127

  - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Agreement from RAN1#120

  - On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement:

    - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

      - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

        - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

        - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Since the current beam RS has to correspond to the one for the indicated TCI state as per RAN2#127 agreement, the RS mapping rules from a RAN1 agreement (i.e., Scheme-1 and Scheme-2) have to be satisfied. Therefore, the implicit current beam RS mapping following the RAN1 agreement is the better approach that is straightforward and can avoid any potential misconfiguration in the explicit RS type signalling method (e.g., RS type is set to CSI-RS, but there is no appropriate CSI-RS configuration provided to UE).

Plus, as per previous RAN2 agreements that Rel-18 LTM CSI resource & LTM CSI report configurations are reused for LTM event-triggered measurement report, the LTM CSI resource would be present for all event types including Event LTM2. Therefore, we can consider utilizing the LTM CSI resource configuration for two purposes in Event LTM2:

- Measurement RS resource of which L1-RSRP values are reported when the Event LTM2 condition is met.

- Reference for the current beam RS determination

  - Similar to Event LTM3 case, UE implicitly determines the current beam RS based on the RS type of the configured LTM CSI resource.

    - If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

    - If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

Note) LTM CSI resource would serve as the candidate beam list for Event LTM3/4/5.

Observation 2a: Explicit RS type (eventEvaluationRS-Type-r19) in RRC running CR conflicts with not only RAN2#127 agreement, but also RAN1#120 agreement, and it can cause a potential misconfiguration (e.g., RS type is set to CSI-RS, but there’s no proper CSI-RS configuration for BM).

Observation 2b: LTM CSI resource is present for all events, and for Event LTM2 hence Event LTM2 can leverage the concept of implicitly determining the current beam RS based on RS type in LTM CSI resource, which is effectively the same as current beam RS determination for Event LTM3.

Proposal 2a: For Event LTM2, UE measures the list of RSs in LTM CSI resource and include the measured L1-RSRP results in the measurement report MAC CE.

Proposal 2b: For Event LTM2, the current beam RS is implicitly determined by the RS type of the configured LTM CSI resource (following the RAN1 agreement):

- If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

- If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

## 2.3	mTRP support

In RAN1#120, the coexistence of LTM event-triggered reporting and mTRP operation at serving cells was discussed, and it was concluded that it is up to RAN2 to decide the support of coexistence between LTM event-triggered reporting and mTRP [3]. While the WID for NR mobility enhancements Phase 4 didn’t specify the simultaneous operation of LTM event-triggered measurement reporting and mTRP at serving cells, the WID for NR MIMO Phase 5 explicitly stated that the UE-initiated/event-driven beam management is targeting FR2 and sTRP. Considering the fact that LTM event-triggered measurement reporting in RAN2 is also dependent on RAN1 decisions, it would not be desirable to support the mTRP operation just for LTM purpose without necessary RAN1 discussions.

Observation 3: Given the dependency between LTM event-triggered measurement reporting in RAN2 and UE-initiated/event-driven beam reporting in RAN1, it is not desirable to support the coexistence between LTM event-triggered measurement reporting and mTRP operation without necessary RAN1 discussions.

Proposal 3: Coexistence of LTM event-triggered measurement reporting and mTRP operation is not supported in Rel-19.

# 3	LTM event evaluation

## 3.1	Prohibit timer

When the event is evaluated per-beam level, it is possible that another beam satisfies the event condition, shortly after the measurement report for that event has been transmitted. Allowing the UE to trigger a new measurement report whenever the event condition is satisfied may lead to too many event-triggered L1 measurement reports for each event configuration. As more event configurations provided to UE, the overall number of event-triggered reports could be unnecessarily large.

In order to achieve timely event-triggered but less frequent report transmissions, a new event-triggered measurement report can only be triggered after a “sufficient amount of time” has passed since the last measurement report for the same event. This sufficient amount of time can be controlled by a prohibit timer which starts after the measurement report for the triggered event is sent, and while this timer is running, no new measurement report can be triggered. Triggering event-triggered measurement reports for every triggering beam can be also supported by setting this prohibit timer value to zero.

Observation 4: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 4a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 4b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

## 3.3	Different HYS/TTT values for leaving condition

In RAN2#127b, it was agreed that a measurement report can be sent when the leaving condition is met based on NW configuration, and in RAN2#128, it was agreed that TTT is applied to the leaving condition.

In L3 event-based measurement reports, one set of HYS and TTT values are configured per event configuration, and they are applied to both entering and leaving conditions as illustrated in Figure 1. Applying the same HYS and TTT values to both entering and leaving conditions wouldn’t make much difference in triggering the L3 measurement reports and subsequent L3 HOs because (i) the cell metric is used for the evaluation hence the metric is less sensitive to the beam level metric differences, (ii) the measurement report through RRC signaling has a larger time scale than lower layer signaling (UCI or event-triggered L1 measurement MAC CE), and (iii) L3 HO has a longer overall switching latency than LTM switch (i.e., from the measurement report trigger to the cell switch complete).

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Figure 1. Illustration of entering and leaving condition in L3 measurement Event A3

In event-triggered L1 measurement reports for LTM, however, the situation becomes different. For a seamless LTM cell switch, it is crucial to keep track of the best candidate beam among those who fulfilled the entering condition for the potential cell switch so that serving cell can perform the candidate TCI state activation and/or early UL synchronization with that chosen beam. While a candidate beam satisfying the entering condition can be chosen conservatively via HYS/TTT, any candidate beam that is no longer good for cell switch (i.e., leaving condition) should be identified by serving cell through a measurement report as quickly as possible. In this aspect, applying the same HYS and TTT could delay the leaving condition to be triggered significantly.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

In order to expedite the event-triggered L1 measurement report for leaving condition, there can be two options to be considered:

- Option 1: Keep one set of HYS and TTT for both entering and leaving conditions, and set them to smaller values

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Option 1 can be the simplest solution without any specification impact, but the smaller HYS and TTT values would impact the entering condition to be triggered quicker as well, which could lead to unexpectedly frequent measurement reports triggered due to potential noisy L1 measurements (without any spec-defined filtering applied). Both Option 2 and 3 provide the configurability of different HYS and/or TTT values between entering and leaving conditions, but Option 3 could be preferrable due to its flexibility in not providing additional HYS and/or TTT for leaving condition if NW chooses not to.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# 4	Measurement report MAC CE

## 4.1	L1-RSRP change rate information

In RAN2#127b, L1-RSRP was agreed to be included as the beam quantity in event-triggered L1 measurement report for LTM. While reporting the ‘snapshot’ of measurement results (filtered or unfiltered) has been traditionally adopted as the beam quantity included in L1 or L3 measurement reports, the lack of additional information such as the change rate of measurement quantity change has brought several challenges (e.g., handover failures and ping-pong handovers) in real deployment scenarios.

Observation 6a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

In Figure 2 below, the left and right figures show two different expectations after LTM execution which is assumed to be done based on the event-triggered measurement report from UE. If UE can include additional information of the L1-RSRP change rate (or “slope”) along with the L1-RSRP value, serving cell could make the better choice for the candidate beam & candidate cell for the LTM cell switch.

Figure 2. (left) negative L1-RSRP slope and (right) positive L1-RSRP slope in event-triggered L1 measurement report MAC CE transmission

Observation 6b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 6: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

## 4.2	Prioritization among triggered events

Since gNB can configure multiple event configurations with different sets of candidate beams and/or different event types, it is possible that more than one event can be triggered at UE and those reports need to be transmitted simultaneously through the available PUSCH grant instance. When the grant size is not sufficient enough to accommodate all the triggered events (even with the truncated MAC CE), then UE would have to drop one or more triggered events’ report to fit into the grant size. It would be desirable to discuss how UE can handle this scenario.

Observation 7: It is possible that UE detects multiple events triggered but the available grant size is not sufficient to accommodate all triggered events’ report even with the truncated MAC CE hence the prioritization among triggered events needs to be discussed.

Proposal 7: The following event prioritization rule is discussed and down-selected for UE behaviour when the available grant size is not enough to accommodate all triggered events’ report:

- Option 1: Define a priority order among Event LTM 2/3/4/5 in the spec.

- Option 2: Introduce an RRC parameter for the priority value to be associated with the event configuration

- Option 3: Leave it up to UE implementation

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Observation 1: There was no consensus in RAN1#120 about supporting semi-persistent CSI-RS for LTM event-triggered L1 measurement reporting.

Proposal 1: RAN2 discussion on supporting semi-persistent CSI-RS for LTM event-triggered L1 measurement reporting is postponed until further RAN1 progress.

Observation 2a: Explicit RS type (eventEvaluationRS-Type-r19) in RRC running CR conflicts with not only RAN2#127 agreement, but also RAN1#120 agreement, and it can cause a potential misconfiguration (e.g., RS type is set to CSI-RS, but there’s no proper CSI-RS configuration for BM).

Observation 2b: LTM CSI resource is present for all events, and for Event LTM2 hence Event LTM2 can leverage the concept of implicitly determining the current beam RS based on RS type in LTM CSI resource, which is effectively the same as current beam RS determination for Event LTM3.

Proposal 2a: For Event LTM2, UE measures the list of RSs in LTM CSI resource and include the measured L1-RSRP results in the measurement report MAC CE.

Proposal 2b: For Event LTM2, the current beam RS is implicitly determined by the RS type of the configured LTM CSI resource (following the RAN1 agreement):

- If the configured LTM CSI resource includes SSB resource, then UE can follow Scheme-2.

- If the configured LTM CSI resource includes CSI-RS resource, then UE can follow Scheme-1 or Scheme-2 depending on gNB configuration (i.e., CSI-RS resource set configured with repetition).

Observation 3: Given the dependency between LTM event-triggered measurement reporting in RAN2 and UE-initiated/event-driven beam reporting in RAN1, it is not desirable to support the coexistence between LTM event-triggered measurement reporting and mTRP operation without necessary RAN1 discussions.

Proposal 3: Coexistence of LTM event-triggered measurement reporting and mTRP operation is not supported in Rel-19.

Observation 4: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 4a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 4b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Observation 6a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

Observation 6b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 6: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

Observation 7: It is possible that UE detects multiple events triggered but the available grant size is not sufficient to accommodate all triggered events’ report even with the truncated MAC CE hence the prioritization among triggered events needs to be discussed.

Proposal 7: The following event prioritization rule is discussed and down-selected for UE behaviour when the available grant size is not enough to accommodate all triggered events’ report:

- Option 1: Define a priority order among Event LTM 2/3/4/5 in the spec.

- Option 2: Introduce an RRC parameter for the priority value to be associated with the event configuration

- Option 3: Leave it up to UE implementation

# References

- Chairman Notes; TSG RAN WG2 Meeting #129

- R1-2501500, LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells, Fujitsu

- R1-2500423, Final FL summary of Measurements related enhancements for LTM, Fujitsu