3GPP TSG-RAN WG2 Meeting #129bis                                 R2-2502025

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.2.3

Source: 	Fujitsu

Title: 	Discussion on LCM for Positioning Use Case

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to give our views on the LCM for positioning use cases. As agreed in the RANP meeting, both prioritized NW-side and UE-side use cases will be discussed, including the following:

- UE-side model:

  - UE-side model (Case 1)

- NW-side models:

  - gNB-side model (Case 3a)

  - LMF-side model (Case 3b)

- Discussion

  - UE-side model (Case 1)

2.1.1 Functionality-based LCM

In the RAN2 128 meeting [1], the following agreements were given:

Agreements: 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability changes, UE can send an unsolicited LPP ProvideCapabilities message to LMF. - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered.

And in RAN2 129 meeting [3], the following agreement was given for functionality switching and/or fallback.

Agreement

3. FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

6. UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

We will give our views on functionality-based LCM by responding to the remaining FFSs above.

Fallback Configuration

We think it is beneficial to have one fallback scheme to guarantee the positioning accuracy in case of AI/ML failure, the most two crucial KPIs for positioning are accuracy and latency, UE may switch to another better positioning method with pre-configuration from LMF to avoid the service discontinuity and extra signaling exchange overhead.

We think the fallback strategy is also aligned with the discussion of current capability/applicability reporting procedure. LMF is the entity to decide which positioning method(s) can be used, and it may request multiple capabilities/applicability via LPP signaling, after the applicability reporting, LMF may send indications and/or configurations of more than one positioning method including AI/ML and legacy for UE.

Proposal 1 New fallback configuration (e.g., pre-configuration) for AI/ML positioning case 1 may be introduced to guarantee the accuracy and latency of positioning service.

As for the “autonomous switching” between AI/ML and non-AI/ML methods, a consensus must be made among companies on the definition of “autonomous switching”. From our perspective, LMF is the entity which decides the positioning method used for LCS request of location services, therefore, LMF is responsible for configuring the list of positioning methods to UE, this behavior applies to AI/ML positioning methods as well, and multiple positioning methos can be configured at the same time to UE as legacy defined.

In this case, case 1 is a UE-side model and UE will have the full picture on how the model performs, if the positioning accuracy suffers from degradation, UE may make its own choice to switch the positioning method from one to another, at least from AI/ML method to non-AI/ML method, considering the possible AI/ML functionality identification procedure.

Therefore, the procedure will be: 1) LMF configures one or more positioning methods to UE after necessary procedures (e.g., LPP capability exchange, AI/ML functionality identification). 2) UE activates the AI/ML functionality and starts the inference. 3) UE detects AI/ML performance degradation. 4) UE decides to “autonomously” fallback/switching/deactivates to another positioning methods for service continuity.

LMF is fully aware of UE’s action, since it configures the full sets of the positioning methods and UE will feedback the positioning results by corresponding IEs (e.g., ProvideLocationInformation per positioning methods) to LMF, so LMF will notice which positioning method is being used at UE by decoding the received IEs. In this case, it is possible to maintain both the NW control and the positioning accuracy/service continuity.

Proposal 2 UE may “autonomously” perform functionality switching/deactivation or fallback to another positioning method, if multiple positioning methods have been pre-configured by LMF.

The configuration for AI/ML and legacy can be largely shared, the similarity on configuration between AI/ML and legacy method can be one of the considerations for LMF to select the fallback.

Proposal 3 Fallback and functionality switching may be integrated into one procedure, details can be further discussed.

Inference Configuration

Similar to our parallel discussion for beam management [4], we believe the inference configuration can be signaled via either step 3 or step 5 so that more flexibility can be achieved. UE may start the AI/ML inference once the applicability is determined, without waiting for further signaling.

However, similar to the discussion for beam management in RAN2, there are still no final clarifications or conclusions on inference configuration and other configurations. The down selection between step 5 and step 3 may only be available after finalizing the detailed contents of the inference configuration.

Proposal 4 NW may signal the inference configuration in step 5 or step 3.

Applicability Reporting

It has been agreed that the UE will report the functionality change of to the LMF by the LPP provide capabilities message. There are two contents for UE to report:

- The fact that applicability has changed.

- The details on how/what/why the change is and the potential follow-up actions.

UE may report the simple fact by the ProvideCapability message, maybe 1 bit is enough to indicate. There is no extra NW control needed for this message, because LMF does not have the ability to predict when and how the applicability of AI/ML functionality will be changed in the future.

Proposal 5 Additional NW control is not necessary for the UE to indicate the change of functionality applicability via LPP ProvideCapability.

After receiving the fact that the current AI/ML functionality has been changed, multiple choices are available for both sides to exchange the message of the second bullet above. E.g., UE can send RequestAssistanceData to request further LMF command for further actions, while LMF can also send Further RequestCapability or RequestLocationInformation messages to let UE report more information on how/what/why the change is, so that it can configure further actions for the UE. Anyway, we consider these signaling as subsequent procedure after the change reporting, so we do not consider them as “LMF control” on applicability reporting.

Proposal 6 Further information on the applicability change and the potential follow-up actions may be exchanged between UE and LMF, but it is not considered as “additional NW control” on applicability reporting.

2.1.2 Performance Monitoring

In RAN2 #125bis meeting, the following agreements were given:

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

For positing case 1, both UE and LMF can be used to calculate the monitoring metrics according to RAN1’s current discussion. The feasibility of the two options is still under investigation in RAN1, so RAN2 may wait for further RAN1 progress.

According to the current LPP signaling and framework design, even if UE can make decision of the preliminary monitoring results, a final confirmation from LMF is preferred.

Proposal 7 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 8 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

  - NW-side model (Case 3a/3b)

In RAN2 #125bis meeting [2], the following agreements were given for positioning NW-side model LCM:

Agreements: For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and waits for any further inputs from other WGs. For POS, RAN2 assumes that NRPPa is used for the signaling between gNB and LMF for case 3a and 3b and the detailed signaling design is up to RAN3.

According to the above agreements, for the performance monitoring for case 3a, either gNB or LMF can derive monitoring metrics or make preliminary decisions. gNB and LMF are both NW entities, but LMF is superior to gNB during LCM procedure, so the following options are available for Case 3a monitoring decision making.

- gNB makes both the preliminary and final monitoring decisions and reports to LMF.

- gNB makes the preliminary monitoring decision while LMF makes the final monitoring decision/confirmation.

- LMF makes the preliminary and final monitoring decisions.

Further study of the details of the above three options depends on further RAN1 progress.

For case 3b, LMF can derive the performance monitoring metrics and make the monitoring decisions. Some assistance information from gNB/PRU/UE may be necessary for case 3b metrics calculation. The details depend on RAN1 progress.

Potential NRPPa signaling design for decision making, monitoring assistance information between gNB and LMF need to involve RAN3.

Proposal 9 Both gNB and LMF can make monitoring decision for case 3a. The details depend on further RAN1 progress.

Proposal 10 LMF can make monitoring decision for case 3b. RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

- Conclusion

Proposal 1 New fallback configuration (e.g., pre-configuration) for AI/ML positioning case 1 may be introduced to guarantee the accuracy and latency of positioning service.

Proposal 2 UE may “autonomously” perform functionality switching/deactivation or fallback to another positioning method, if multiple positioning methods have been pre-configured by LMF.

Proposal 3 Fallback and functionality switching may be integrated into one procedure, details can be further discussed.

Proposal 4 NW may signal the inference configuration in step 5 or step 3.

Proposal 5 Additional NW control is not necessary for the UE to indicate the change of functionality applicability via LPP ProvideCapability.

Proposal 6 Further information on the applicability change and the potential follow-up actions may be exchanged between UE and LMF, but it is not considered as “additional NW control” on applicability reporting.

Proposal 7 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 8 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

Proposal 9 Both gNB and LMF can make monitoring decision for case 3a. The details depend on further RAN1 progress.

Proposal 10 LMF can make monitoring decision for case 3b. RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

- References

[1] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #128, 2024.

[2] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #125bis, 2024.

[3] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #129, 2025.

[4] R2-2502024, Discussion on LCM for UE-sided Model for Beam Management Use Case, Fujitsu, 3GPP TSG RAN WG2 Meeting #129bis, Wuhan, China, April 7th-11th, 2025.