3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502031

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.5.4

Source: 	Fujitsu

Title: 	Adaptation of common signal or channel

Document for:	Discussion and decision

Introduction

Rel-19 NES WID was updated and approved in RAN#105 [1]. The WID describes the following objective on the adaptation of common signal/channel transmissions.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we would like to discuss and provide our views on the adaptation of common signal/channel transmissions.

Discussion

Adaptation of SSB in time domain

Regarding SSB adaptation, the following agreements were made in the RAN1#120 [2].

Conclusion@RAN1#120 There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Agreement@RAN1#120 For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement@RAN1#120 For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) - Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

It has been agreed in RAN1 that the UE can be configured with multiple SSB burst periodicities, and group common DCI signalling (DCI format 2_9) can be utilized for switching the SSB burst periodicity. Furthermore, SSB occasions with larger burst periodicity are a subset of the SSB occasions with shorter periodicity.

Based on the note above, RAN1 would leave the decision on whether to use a new MAC CE for SSB adaptation up to RAN2. If MAC CE is used for switching e.g., SSB periodicity, the NW must send it to all RRC connected UEs in the cell. Otherwise, it will cause uncertain measurement results because the UE may not measure the SCell SSB correctly. Thus, MAC CE based SSB adaptation will increase signalling overhead compared with DCI-based one. Also, in general, it is not preferred to adopt a duplicate solution for the same feature unless it has clear benefits. With the above, a new MAC CE is not supported in RAN2.

Proposal 1: MAC CE based SSB adaptation is not supported.

The remaining issue is whether SSB adaptation has impacts on RLM and BFD/BFR. According to the current specification, RLM is performed in PCell rather than SCell, and BFD reference resource is restricted to 1-port CSI-RS. Thus, SSB adaptation in SCell has no impact on RLM and BFD. For BFR, an SSB resource identified by ssb-Index can be configured as an BFR reference resource. However, it has been agreed the time domain positions of SSBs within a burst remain unchanged during SSB adaptation, ensuring that ssb-Index remains consistent before and after SSB adaptation. Therefore, SSB adaptation has no impacts on BFR.

Proposal 2: RAN2 to confirm that SSB adaptation has no impact on RLM and BFD/BFR

The following agreements were made in the RAN2 #129 [3].

Agreements on RACH adaptation@RAN2#129 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

For L3 measurements, RAN2 has agreed to introduce additional SMTC configurations according to SSB adaptation for L3 RRM measurements on SCell. Specifically, when a UE is configured with multiple SSB burst periodicities, multiple SMTC configurations may be provided, each aligned with a respective SSB burst periodicity. From RAN2 perspective, the DCI format 2_9, already standardized for SSB periodicity switching, can be repurposed for switching SMTC configurations without extra signaling overhead.

Proposal 3: DCI signaling for switching SSB periodicity can be repurposed for switching SMTC configuration.

Adaptation of PRACH in time domain

The following agreements about PRACH adaptation were made in RAN2 #129 [3].

Agreements on RACH adaptation@RAN2#129 1. Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. 4. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. 6. Will follow legacy mechanism regarding how to select RACH resource.

In RAN1 #120, it was agreed to support a 1-bit field in DCI 1_0 with C-RNTI to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. This implies PRACH adaptation can be applied to both CFRA and CBRA initiated by PDCCH order.

Proposal 4: RAN2 to confirm CFRA is supported for PRACH adaptation.

For 2-step RA, it should be noted that 4-step and 2-step are configured by separate parameters in current specification. Therefore, additional PRACH resource needs separate configurations for 4-step and 2-step RA, respectively. Furthermore, PUSCH configuration for 2-step RA, as well as the fallback procedure from 2-step RA to 4-step RA require further discussion. Given the time constraints and specification impacts, it is proposed that PRACH adaptation initially supports 4-step RA in Rel-19, while the support for 2-step RA may be deferred to future release.

Proposal 5: Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19.

Next, we discuss PRACH resources selection. In RAN2 #129, it was agreed to follow legacy mechanism for PRACH resource selection. This implies that additional PRACH resources are not prioritized, and UE implementation determines whether to select legacy PRACH resources or additional PRACH resources. The key unresolved issues include:

- Whether legacy and additional PRACH resources belong to the same PRACH resource set.

- Whether it is necessary to check the availability of the additional PRACH resource in RA initialization and during the RA procedure.

Proposal 6: For PRACH resource selection, RAN2 to discuss:

      - Whether legacy and additional PRACH resources belong to the same PRACH resource set.

      - Whether it is necessary to check the availability of the additional PRACH resource in RA initialization and during the RA procedure.

Another aspect requiring discussion is RA-RNTI calculation. After transmitting the PRACH preamble on the selected PRACH occasion, the UE will compute the RA-RNTI based on the following equation.

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

Where s_id, t_id, f_id are determined based on the selected PRACH occasion, and ul_carrier_id is determined based on the UL carrier used for preamble transmission.

According to the agreements reached in RAN1 shown below, it is possible to configure the legacy PRACH occasions to be separate with the additional PRACH occasions in frequency domain while overlapping in time domain (i.e., configuring the same prach-ConfigruationIndex value but different msg1-FrequencyStart values). An example of this configuration is illustrated in Figure 1. Since the Rel.19 UEs are allowed to select both legacy and additional PRACH occasions, there is a chance that two UEs select different PRACH occasions (legacy RO#1 and additional RO#1 for instance) but having the same values for s_id, t_id and f_id, resulting in identical RA-RNTI values. In this case, if these two UEs happen to choose the same PRACH preamble, they will be unable to distinguish which RAR is intended for them. It leads to simultaneous Msg.3 transmissions on the same UL resource from both UEs.

Agreement@RAN1#118b For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement@RAN1#119 At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement@RAN1#120 - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Figure 1. An example of PRACH configuration where additional PRACH resources and legacy PRACH resources are configured by the same prach-ConfigurationIndex value but different msg1-FrequencyStart values.

Observation 1: It is possible that two UEs select different PRACH occasions having identical RA-RNTI values, which results in simultaneous Msg.3 transmissions on same UL resource from both UEs.

The above issue can be addressed by configurating different prach-ConfigurationIndex values for legacy PRACH resources and additional PRACH resources, respectively. However, this is inconsistent with RAN1’s agreement. Therefore, we think RAN2 can discuss how to address this issue with alternative approaches.

Proposal 7: RAN2 to discuss how to address the issue of additional PRACH occasions and legacy PRACH occasions having the identical RA-RNTI values.

Conclusion

In this contribution, we have some discussions about the adaptation of common signals or channels, and the following proposals are made:

Proposal 1: MAC CE based SSB adaptation is not supported.

Proposal 2: RAN2 to confirm that SSB adaptation has no impact on RLM and BFD/BFR.

Proposal 3: DCI signaling for switching SSB periodicity can be repurposed for switching SMTC configuration.

Proposal 4: RAN2 to confirm CFRA is supported for PRACH adaptation.

Proposal 5: Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19.

Proposal 6: For PRACH resource selection, RAN2 to discuss:

      - Whether legacy and additional PRACH resources belong to the same PRACH resource set.

      - Whether it is necessary to check the availability of the additional PRACH resource in RA initialization and during the RA procedure.

Observation 1: It is possible that two UEs select different PRACH occasions having identical RA-RNTI values, which results in simultaneous Msg.3 transmissions on same UL resource from both UEs.

Proposal 7: RAN2 to discuss how to address the issue of additional PRACH occasions and legacy PRACH occasions having the identical RA-RNTI values.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, RAN#105 meeting.

- RAN1#120 chair’s note

- RAN2#129 NES session chair's note